QBserial version 2.0

Serial I/O Routines for use with QuickBASIC & BC6/7

This library will provide you with serial I/O communications

routines for use in QuickBASIC 4.x (with or without PDQ), and the

Microsoft Basic Compiler 7.x Professional Development System. No

longer are you forced to use the poor communications support

provided by QB.  This program will allow you to control 8250 type

communications ports at speeds of up to 115200 baud. Communication

ports 1 - 4, and non-standard addresses are supported. You will no

longer have problems with the DTR signal. DTR is left in the same

state it was in when before you called this driver, or it can be

controlled by your program. The serial driver includes XON/XOFF and

CTS handshaking. Serial input is interrupt driven, using any IRQ

you specify (1 - 7), with incoming XOFF flow control (if enabled)

to prevent overrunning the input buffer. These are the same serial

I/O routines used in Sparkware's Qmail door, and the User Database

System Doors "Query Door" and "Upload Door".

The driver is written in Microsoft's QuickC version 2.51. This

driver is useable with QuickBASIC version 4.x, and Basic Compiler

6.x & 7.x PDS. (With or without PDQ). BC 1.x, QuickBASIC 2.x, and

3.x are not supported by this driver. The reason is that versions

of QuickBASIC prior to 4.0 do not support the extensive multi-

language interface that QB4.x has (via the DECLARE statement and

Microsoft language extensions). Throughout this manual QB will be

used to refer to both QuickBASIC and the Basic Compilers (6.x & 7.x

PDS).

Before the driver can be used, the following DECLARE statements

must be added to the beginning of your QB program:

DECLARE SUB OpenComm CDECL ALIAS "_open_comm" (BYVAL Port%, BYVAL_

IRQ%, BYVAL Wlen%, BYVAL Parity%, BYVAL Baud&,

BYVAL HS%)

DECLARE SUB CloseComm CDECL ALIAS "_close_comm" ()

DECLARE FUNCTION WriteChar% CDECL (BYVAL Ascii%)

DECLARE FUNCTION ReadChar% CDECL ()

DECLARE SUB Transmit CDECL ALIAS "_transmit_string" (addr$)

DECLARE FUNCTION DataWaiting% CDECL ALIAS "_data_waiting" ()

DECLARE SUB ClearInputBuffer CDECL ALIAS "_clear_input_buffer" ()

DECLARE SUB CarrierDetect CDECL ALIAS "_carrier_detect_flag"_

(BYVAL OnOff%)

DECLARE FUNCTION CarrierLost% CDECL ALIAS "_carrier_state" ()

DECLARE SUB DTRcontrol CDECL ALIAS "_dtr" (BYVAL OnOff%)

DECLARE FUNCTION DriverCopyright% CDECL ()

These declarations specify all the entry points into the serial

driver, They are in the include file QBSERIAL.DEC. DO NOT change

them or the driver may not function.

QBSERIAL User Manual - V 2.0                               Page 1

Refer to the included sample programs SIMPLE.BAS & PCBDOOR.BAS,

they use most of the calls described below.

Port Initialization

OpenComm Port%, IRQ%, Wordlen%, Parity%, Baudrate&, HS%

Port%     This is an integer value. If the specified port is 1 to

4, the driver opens that port. If you specify the port as

ZERO (0) the driver enters "LOCAL" mode. This allows you

to call the driver with data, but the driver won't send

anything. This is useful when working with "doors". if

you specify the port as any other value, QBserial will

use that as the base address for the port. This allows

you to work with non-standard I/O addresses. Note however

that if you open a non-standard port, you must specify an

IRQ value of 1 to 7

IRQ%      Specifies which interrupt to use with this port. If the

value of IRQ% is ZERO (0), then the default IRQ values

are used (COM1 & COM3 use IRQ4, COM2 & COM4 use IRQ3).

This is what most applications will use. Specify an IRQ

value of 1 through 7 when you want to use an IRQ value

other than the default. Such as when you want to use IRQ7

with COM3. NOTE: Be careful when choosing an IRQ value

other than the default. Most machines use the other IRQ

inputs for other machine functions such as the Hard drive

and system clock. QBserial DOES NOT chain the interrupt,

it takes it over entirely. If you choose an IRQ that is

used for something already, your machine will most

certainly operate improperly.

Wordlen%  an integer specifying the word length of the serial data.

It has a value of 7 or 8.

Parity%   an integer, 0 = NONE, 1 = ODD, 2 = EVEN.

BaudRate& is a LONG INTEGER representing the desired baud rate.

Valid numbers are 0, 300, 1200, 2400, 4800, 9600, 19200,

38400, 57600, and 115200. The value you send is NOT

checked and I assume you could generate crazy baud rates

if desired. 115200 is the absolute maximum rate that can

be generated. If you specify a baud rate of ZERO then the

serial port is used AS-IS. The rate, word length, &

parity are NOT changed. This is useful for door operation

since the port is already initialized at the proper

settings when you get control.

HS%       an integer specifying the type of handshake you wish to

use between the CPU and Modem (or destination device).

QBSERIAL User Manual - V 2.0                               Page 2

Valid numbers are: 0 = NO handshake, 1 = XON/XOFF, 2 =

CTS, 3 = XON/XOFF and CTS.

Serial Output

To send a string of data out the serial port, you use the

"Transmit" call as follows:

Temp$ = "Transmit this..."

Transmit Temp$

or

Transmit "Send this also...."

If you want to transmit single characters, you can use the

"WriteChar" function. Transmit calls WriteChar to send the actual

characters to the port. If you wish to use WriteChar, it has the

following format:

Status% = WriteChar(Char%)

Char% is the integer value of the character you want to send, it is

not a string. Status% is an integer returned by WriteChar, it is

NON-ZERO if the character was actually sent to the port, and ZERO

if the character was not sent. WriteChar will loop indefinitely on

CTS hold, XOFF hold, or Transmit Buffer Busy. These loops will exit

if carrier is lost in FULL or PARTIAL modes and return a ZERO

status%. WriteChar gives you more low level control of the sending

of characters, but you must send each character separately and

monitor its status.

The Transmit function monitors the status of its calls to WriteChar

and triggers QuickBASIC's User Event trap (UEVENT) if a carrier

loss occurs in FULL mode. The UEVENT trap (FULL mode) is used by

default. You can disable the use of this event trap in the event

you already use the UEVENT trap for something else (PARTIAL mode).

Therefore, by default, carrier loss detection in your program is

done with the ON UEVENT GOSUB/UEVENT ON statements. The example

programs show the use of this method. By default UEVENT is also

used for catching carrier loss on keyboard input. this is described

below.

Serial Input

Serial input is managed with three functions: DataWaiting,

ReadChar, & ClearInputBuffer. DataWaiting is used to test the input

buffer to see if any characters need to be read from it. ReadChar

QBSERIAL User Manual - V 2.0                               Page 3

is then called to get the characters. ClearInputBuffer flushes the

input buffer of any existing characters. Both example programs

contain a subroutine called "KeyboardInput" that demonstrates the

use of these calls. It also properly handles input from the local

and remote keyboards, and can be the routine you use in your

programs if you wish. Basically the procedure for reading the

remote keyboard (serial input) is as follows:

IF DataWaiting THEN

C$ = CHR$(ReadChar)

END IF

The DataWaiting function also monitors the state of the carrier

detect signal and triggers, in FULL mode, the user event trap

(UEVENT) if carrier is lost. As you can see a singular trap routine

catches carrier loss on output as well as input. You will notice

that the "KeyboardInput" subroutine loops using DataWaiting, and as

such, carrier is constantly checked while waiting for input. If the

ReadChar function is called when no data is available (as indicated

by DataWaiting) a NULL character is returned.

If XOFF/XON or BOTH is enabled, then the serial buffer will be

protected from overfills. When the buffer reaches 75% of capacity,

an XOFF will be sent on the serial output. When the buffer empties

out to 25% of capacity, an XON will be sent. The capacity of the

input buffer is 1024 bytes.

Carrier Detect Control

There are three modes of carrier detection: Full, Partial, & None.

Each has it own distinct use for a particular purpose. The driver

defaults to FULL carrier detection unless you change it. The

following call is used to change control modes:

CarrierDetect 0 [or] 1 [or] 2

Specifying a TWO (2) sets carrier detection to FULL. This is the

default mode. When a loss of carrier occurs while transmitting,

polling for input data, or sitting in a wait loop (XOFF/XON or

CTS), the driver will return to the calling program and trip the

UEVENT flag. Tripping the UEVENT flag causes Basic to go to the

User Event trap routine (providing ON UEVENT GOSUB/UEVENT ON has

been set up in the user program). You should use this mode only if

you want to use the UEVENT trap for carrier loss detection. You

cannot transmit data (with the Transmit function) in this mode if

there is no carrier. You can send characters in this mode with the

WriteChar function.

Specifying a ONE (1) sets carrier detection to PARTIAL. This mode

is similar to FULL with the exception that the UEVENT flag is not

QBSERIAL User Manual - V 2.0                               Page 4

tripped when carrier is lost. This mode should be used if UEVENT is

used for something else in your program, or you don't wish to

utilize UEVENT. This mode can also be used for talking to a modem

when there is no carrier signal present (such as dialing and

initialization commands). The Transmit and WriteChar routines will

always transmit data in this mode with or without carrier present.

Specifying a ZERO (0) sets carrier detection to NONE. An example of

this mode would be where "plain Jane" serial I/O is required (such

as communicating with a "dumb" terminal). In this application there

are no modem handshake signals (3 wire EIA/RS-232). Carrier

detection must be disabled in this mode to prevent calls to Uevent

and disable the no carrier escape mechanisms built into the

XON/XOFF loops. Important: if you are using the XOFF/XON protocol

in this mode and receive an XOFF while transmitting, you will sit

forever waiting for an XON. This is a normal condition, with the

exception that there is no escape from this loop other than XON.

Carrier Loss Detection by Polling

As mentioned above, by default, the UEVENT trap is used to detect

when a loss of carrier occurs. There is also a function that allows

you to detect loss of carrier without using the UEVENT trap. While

I feel that using UEVENT is the simplest way to catch a carrier

loss in a program, it is possible that UEVENT might need to be used

by something else. This function is:

X% = CarrierLost

This function call allows to you to 'poll' for the state of

carrier. CarrierLost returns a NON-ZERO value when there is NO

carrier, and a ZERO when there IS carrier. CarrierLost is a real

time function, it returns the current carrier condition at the time

of the call. This allows you to code simple IF or DO loops to

detect loss:

IF CarrierLost THEN

..process carrier loss

END IF

Program Termination

Since "OpenComm" seizes interrupt vectors, these vectors need to be

restored BEFORE your program ends. If you neglect to restore these

vectors, you are guaranteed a crash. The routine used to reset the

vectors is: CloseComm. Only use CloseComm if you specified a port

value of 1, 2, 3, 4, or a non-standard port address in the OpenComm

call. If you specified a port value of ZERO (Local mode) the

interrupt vectors are NOT grabbed, and do not need to be reset.

QBSERIAL User Manual - V 2.0                               Page 5

CloseComm also resets any UART registers to their original value,

it also resets the programmable interrupt controller (8259 PIC) to

its original settings.

Data Terminal Ready (DTR)

Everybody that has used QuickBASIC's OPEN COMn statement is very

familiar with the problems of the DTR signal. Apparently Microsoft

felt that DTR would never be needed after a program terminates.

Anybody who has ever tried to write a door, or chain from one

program to another has cursed this decision at some time during

their programs development. This driver handles the DTR with a

different view. When you close the comm channel the DTR signal is

left in the same state it was when you opened the comm channel. For

door applications, this is a must.

A function call is provided however that allows your program to

control the DTR signal:

DTRcontrol 1 [or] 0           ' 0 = DTR OFF, 1 = DTR ON

Additionally when you use DTRcontrol to change the state of the DTR

signal it also instructs the driver to leave it this way when the

CloseComm function is called. Remember that the driver leaves DTR

the way it found it when OpenComm was called. If you used some

other method to change DTR after OpenComm was called, it would

automatically return to that original state when you called

CloseComm. Therefor only DTRcontrol should be used if you wish to

change the state of DTR.

Driver Version and Copyright

A function is available to access the drivers version and copyright

string. This would be used if you wish to display in your final

product the version of QBserial that your are using, and/or my

copyright along with yours. In order to access the copyright

string, the following lines should be added to your Basic program:

X& = DriverCopyright

WHILE (PEEK(X&))

CP$ = CP$ + CHR$(PEEK(X&))

X& = X& + 1

WEND

This places the version/copyright string into string variable CP$.

This string may now be handled in whatever method you choose.

Cresent's PDQ Library

QBSERIAL User Manual - V 2.0                               Page 6

An object module for use with PDQ is also included. PDQ does not

support UEVENT, so all references to UEVENT in the text should be

ignored. You will need to do carrier checking by polling with the

CarrierLost function. If you use FULL mode and carrier is lost it

will operate the same way as PARTIAL mode. The only exception to

this is the Transmit function, which when in FULL mode, will not

transmit data.

NOTE:     An additional note for PDQ users. Since PDQ replaces the

Microsoft run time libraries, the public variable

ACRTUSED is not satisfied during the link stage. A dummy

stub file, ACRTUSED.OBJ is included with QBserial to

prevent this error from occurring. The next paragraph

explains the proper linking sequence.

Linking

Since object modules are provided for QB4.x, BC7, and PDQ, you will

need to select which one to use for your program. The additional

library (SERIAL.LIB) only contains routines from the C library that

are required by the driver. The file QBSER.OBJ should be used for

programs compiled with QuickBASIC, BC7SER.OBJ is to be used for

programs compiled with BC7 PDS, and QBSERPDQ.OBJ is for PDQ users.

Typical link command lines would look like this:

Link yourprog qbser,,,serial;                (For QuickBASIC)

Link yourprog bc7ser,,,serial;               (For BC7 PDS)

Link yourprog qbserpdq acrtused,,,serial;    (For PDQ)

Quick Libraries

If you wish to develop programs from within the environment, you

will first need to create a Quick library using one of the object

modules (QBSER.OBJ, BC7SER.OBJ, QBSERPDQ.OBJ) and the SERIAL.LIB

file. To create a Quick lib for QuickBASIC 4.x, use the following

Link command:

Link QBSER,,,SERIAL BQLB4x/q

Where "BQLB4x" is one of the following: BQLB40.LIB, BQLB41.LIB, or

BQLB45.LIB. Use the one that came with your compiler. If you use

the wrong BQLB file you might get an "Invalid Format" error when

attempting to start QuickBASIC. QBSER.OBJ & SERIAL.LIB may also be

added to Quick Libraries containing other routines and OBJ's as

well.

QBSERIAL User Manual - V 2.0                               Page 7

To create a Quick Library for use within QBX (BC7's environment)

use the following link command (be sure to use Link v5.xx that came

with BC7):

Link BC7SER,,,SERIAL QBXQLB/q;

For PDQ, use the method for QB4.x, since you will be using the

built in run time routines built into the environment. You ONLY use

QBSERPDQ.OBJ when you link the EXEcutable file.

Remember too, that you can also add BC7SER.OBJ & SERIAL.LIB to

Quicklibs containing other routines.

Specifications

The following port addresses and default interrupts are used:

Comm      Base Address        IRQ (Default)

1          3F8 hex            4

2          2F8 hex            3

3          3E8 hex            4

4          2E8 hex            3

Please read the section explaining the OpenComm function for

information on how to use addresses or IRQ's other than the

defaults.

Support

I'm reachable in the ILINK Basic conference, or the number below.

I'll help there if you have questions. No other support is

necessary. Bugs will be tended to if required, and good suggestions

tend to be implemented.

This driver is copyrighted by me, but is being distributed as

shareware. You may copy and distribute it freely. In the spirit of

programmer to programmer I am not requesting any registration fee

for the use of these routines. All I ask is that you acknowledge in

your documentation that you are using them (Non-commercial use

only). If you want display my version/copyright notice in your

final product to give me a bit of free advertising, see the section

pertaining to this. If you feel that this driver is worth something

and you would like to send some sort of registration (say $20), I

won't refuse it! If you are using them in a PCBoard door, a copy of

the door for our system would be a nice gesture!

QBSERIAL User Manual - V 2.0                               Page 8

If you plan to use QBserial in a commercial product for resale, you

will need to license the driver first. please contact me for

details.

Source Code

The source code for QBserial is available. The source is ONLY

available by signing a license agreement. The source may be used

for you own internal uses only. The source may not be distributed

by you to any other person, even if you make changes to it. Nor can

the resulting object code be sold or distributed. Modifications can

only be used in your end product, and without any type of royalty.

The license fee for the QBserial source code is $75.00. Please

contact me if you are interested.

Jeff Sumberg

Sysop(2) - SailBoard BBS

Wayne, NJ, 201-831-8152

[ or ]

Box 212

Ringwood, NJ, 07456

Acknowledgments

Thanks go out to Mark "Sparky" Herring for using these routines in

his highly successful Qmail door (versions 3 & 4). Since his door

runs on so many systems of varied configuration, it has made

testing and proving out much easier.

Thanks to Mark Wilson for some very good ideas.

Changes and Revisions

06/09/89  1.0       Initial Release

06/21/89  1.1       Added carrier detection control. Calling

program can turn detection off and on as

desired. Necessary if data sending/receiving

required if carrier isn't present.

09/01/89  1.2       Fixed a bug in Readchar where 'extended'

characters (ASCII 128 to 255) were causing an

Illegal Function Call in the CHR$() conversion

(because they were being returned as negative

numbers). Extended character may now be

QBSERIAL User Manual - V 2.0                               Page 9

received properly. Added three new functions:

CDtrap, CarrierLost, and DTRcontrol.

1.3

1.4       Un-released interim versions.

03/24/90  1.5       Added support for Basic Compiler 7.0 PDS and

PDQ. Added sections to manual about Linking,

Source code availability, and updated section

on Quicklibs. Separate object files now

supplied for QB, BC7, and PDQ.

05/01/90  1.6       Oops! PDQ users found out fast that I forgot to

compile the PDQ module with stack checking

removed. I also forgot to include ACRTUSED.OBJ

to satisfy linking requirements. Sorry 'bout

that! Released only as beta to those that

needed it.

11/01/90  2.0       Added capability to specify non-standard port

addresses and which IRQ to use when the port is

opened. Removed CDtrap function. Created new

carrier detection modes using the CarrierDetect

function. (0 = NONE, 1 = PARTIAL, 2 = FULL).

Also added DriverCopyright function to allow

program access to the driver version.

QBSERIAL User Manual - V 2.0                              Page 10