When  every  we  get a new highspeed modem we  always  run  a

series of file transfer tests using a group of  protocols  to

see how they fair with the new modem.

Protocols Tested:

Tmodem v 6.10 (Micro TECH Systems, Inc.)

Zmax v 1.00b (Micro TECH Systems, Inc.)

Zmodem

Xmodem-1K

Ymodem Batch

Ymodem with OverThrust

Ymodem-G

MPT v1.10 (Matthew Thomas)

Jmodem v38 (Unknown)

BiModem v1.24 (Erik Lab)

Protocol Characteristics:

Tmodem  v6.10  :  Batch  Protocol,  Error  Recovery,  Restart

Aborted  Transfers  (Registered version), 32 Bit CRC  at  low

speeds and proprietary Error Correction similar to 64 bit CRC

at highspeeds.

Zmax  v1.00b    :  Batch Protocol,  Error  Recovery,  Restart

Aborted Transfers, and 32 Bit CRCs.

Zmodem         :  Batch  Protocol,  Error  Recovery,  Restart

Aborted Transfers, 16 bit CRCs and 32 bit CRCs.

Xmodem1K : Non-Batch, Error Recovery, and 16 Bit CRC

Ymodem Batch : Batch Protocol, Error Recovery, 16 Bit CRCs

Ymodem  Batch with Overthrust/Overdrive : Batch Protocol,  No

Error Recovery, 16 Bit CRCs.

Ymodem-G   : Batch Protocol, No Error Recovery, 16 Bit CRCs.

MPT  1.10 : Batch Protocol, Error Recovery,  Assumed  16  BIT

CRCs, Restart Aborted Transfers.

Jmodem   : Non-Batch Protocol, Error Recovery, 16 BIT CRC.

Bimodem   : Batch Protocol, Error Recovery,  Restart  Aborted

Transfers,  16 BIT CRCs (assumed). Bimodem can  transfer  and

receive  at  the same time, but because this is  a  highspeed

modem  and like many, not all, but many highspeed modems,  it

is  half  duplix and bimodem can only function  as  a  single

direction protocol.

Test File : 124,991 byte compressed LHARC (LZH) file.

Test Computers: Two 6 Mhz AT's with Mono Graphic screens  and

40 Mls Harddrives. One computer running Osiris SE  Multi-Line

BBS and the other computer running QT 2000.

NOTE:  Slower computers were used to bring out the  protocols

CPU  overhead.  Any protocol can drive a highspeed  modem  if

it's running on a 33 Mzh 386 or 486....but put it on a slower

computer....

Test  Modems  :  Two Speedmodems with latest  ROM  locked  at

19,200.

Transfers  were  all done on the same call so the  same  line

quality would be present for all transfers.

Results in order from highest to lowest:

Rank     Protocol        Efficiency        CPS

1         Tmodem             96%            921.6

2         Zmax               94%            902.4

3         Ymodem-G           94%            902.4

4         Zmodem             90%            864

5         Ymodem OD/OT       87%            835.2

6         Bimodem            79%            758.4

7         MPT 1.10           78%            748.8

8         Jmodem             54%            518.4

9         Ymodem Batch       40%            384

10        Xmodem1K           40%            384

Not  bad  CPS ratings for a 169 dollar  Modem/Fax/Voice  Mail

Card.

With  *H2,  errors aren't possible between  the  two  modems.

Strictly  speaking, they are but the modems take care  of  it

and you never see it on your end (protocol driver).

The place errors occur is between the modem and the  protocol

driver.

This  tests  the protocols timing sensitivity,  data  testing

reliability,  and the protocols ability to keep up  with  the

incoming data stream.

Two protocols reported errors:

MPT  1.10 always (during 5 test downloads) reported an  error

on the first block. Error recovery speed was good.

Bimodem  1.24  reported  errors  during  2  of  the  5   test

downloads. The error recovery was very slow.

In  all  fairness to Bimodem, it did test out to be  fast  at

2400  baud  during bi-directional  transfers,  providing  the

lines were very clean (not likely under real life conditions)

and no errors were likely.

When  line  noise  WAS encountered, it aborted  2  of  the  6

transfers  and the ones that did not abort dropped  to  below

130 cps, not good at all.

MPT 1.10 seems to work good at 2400 baud, about Zmodem speed.

The results of the tests were not surprising and pretty  much

as expected.

Tmodem  was clearly the winner followed closely by our  other

protocol, Zmax.

One  thing I would like to point out, Bimodem's doc's show  a

comparison  against Zmodem and they claim it is faster.  What

they  fail  to  tell you; the tests are rigged  in  favor  of

Bimodem.  I'm no fan of Zmodem, but I like rigged tests  even

LESS, we get enough of that from magazines.

As  you can plainly see, even old reliable Ymodem Batch  with

Overthrust  walks  all over Bimodem in real  life.....do  you

really  want  to spend 25.00 for a protocol that  YMODEM  CAN

BEAT?

Bare in mind that one of these days you will want to move  to

a  highspeed modem (if you don't already have one)  and  v.32

(full duplex) may be a "STANDARD" on paper, but it sure isn't

in the REAL world, and never will be. That is if US  Robotics

(keeping  in mind that when two USR's connect, they do so  in

half duplex mode), CompuCom, and several other major  players

have anything to say about it and they will.