BinkleyTerm-OS/2 Ver. 2.40                                 11/18/90

Fixed the problems with all the Spawned stuff when SnoopPipe was not

defined, especially AfterMail, CleanUp, Packer and SpawnBBS.  Now

compiled with MS C 6.00 and maximum optimization.

BinkleyTerm-OS/2 Ver. 2.40                                  7/18/90

Fixed the problem with Snoop/SnoopPipe.  Used to have to have a string

SNOOPPIPE set with the \Pipe\name in order to avoid Traps (either at Bink

start or returning from Max).  Updated B_INITVA.C and BTCONFIG.C to

fix it.

BinkleyTerm-OS/2 Ver. 2.40                                  7/14/90

Fixed the set of MaxBytes problems so they now work as advertised.

Fixed the ZMODEM Receive Debris message so the correct message shows.

Apparently still a problem with ExtrnMail.

Differences from the DOS version:                          7/12/90

1.  Since it doesn't have a FOSSIL, BTP relies on the BAUD and LOCKBAUD

to set up buffered MODEMs.  Set the BAUD to the highest CONNECT

value your MODEM will return, usually 9600.  Set the value after

LOCKBAUD to the highest rate between your serial port and your

MODEM.  For HST, that's usually BAUD 9600 and LOCKBAUD 19200 or 38400.

2.  Several Environment strings are used to provide information or to

override defaults values unique to the OS/2 version:

Set SnoopPipe=\pipe\pipename      where pipename matches your PMSnoop.

Set TBuf=buffersize               to override the default 8K xmit buffer.

Set RBuf=buffersize               to override the default 8K recv buffer.

Set JANUSPriority=jprior          to override the default priority 3.

Set RegularPriority=rprior        to override the default priority 2.

Set MODEMPriority=mprior          to override the default priority 4.

3.  To specify a PMSnoop's pipe named MAXSnoop:  Set SnoopPipe=\pipe\MAXSnoop

If its on a Server, use Set SnoopPipe=\\servername\MAXSnoop

4.  BTP leaves most of the serial port flags alone now.  If you run a buffered

MODEM, be sure it supports CTS/RTS handshaking, and then use the MODE

command to set them on:  MODE COM1:9600,N,8,1,TO=ON,RTS=ON,OCTS=ON

5.  If you have a Western Digital 16550-PL UART, I recommend that you only

run with BUFFER=ON and insure that the serial port to MODEM is locked

above 2400 BPS.  If you have a National Semiconductor 16550A or AF, then

you can run with BUFFER=AUTO (both are MODE command flags) and not worry

about the serial port to MODEM rate.

6.  Add the BSRCP240 on top of the BSRC_240 archive's contents.  Tailor the

LINK path to your own.  Use the FSTAT.OBJ (Large Model) unless you have

the INTERNAL.H and DOSTYPES.H from the MicroSoft C Compiler's Library

source.

7.  There will be a multithread version of BTP 2.40 coming out soon, watch

for it.

Bill Andrus 1:109/301.10@fidonet