HIGH MEMORY UTILITIES

This disk contains several utilities, some of which are

general purpose utilities, and others are designed to work with

high memory.  High memory is defined as that memory between the

DOS 640K and the ROM BIOS at the top of the PC's address space.

In the normal PC, this memory is simply non-existent.

Memory may be added, or accessed, in a variety of ways:

1.      Some clones make this memory available.  I guess

the reason is that it is easier to manufacture a

motherboard with three rows of 256K chips (768K)

than it is to make four rows, two with 256K's and

two with 64K's (to give 640K).

If they have done this, it is quite easy to jigger

the addressing so that the extra 128K (768 - 640)

shows up as available memory.  This is usually

addressed at the D000 and E000 portions of memory.

2.      There were some commercial boards manufactured by

IBM and others which could have their DIP switches

set to make the memory available pretty much any

where you wanted.  Though these boards are obsolete,

several mail order houses sell them and they often

show up in flea markets, swap meets, etc.

3.      With current memory technology, it is fairly easy

to make a board at home which will provide the

required memory, and provide other advantages as

well, such as the ability to back it up with a

battery.

4.      Even if none of these avenues is open to you, you

may still have some 'free' memory available if you

are using a Hercules or clone video card, or a CGA

card.  See the file HERCUSER.DOC on this disk.

The above options were discussed in more detail in the

author's article "Filling in the holes on the XT" in Micro

Cornucopia magazine for January 1990, Issue # 52.

THE UTILITIES

The utilities included on this disk are as follows:

EDISK.BIN               A Ramdisk which uses the 128K in the

D and E blocks.  It will retain its

contents after a warm-boot. Depending

on your computer's BIOS, it may even

survive a reset.  Will not survive a

power off/on sequence, however.

EDISK-S.BIN             Similar to the above program, but

must be used with the home-made memory

board.  This board uses static RAM and

it may be backed up with a battery.

This RAMdisk will survive a power off.

HILOAD.COM              A utility to load memory-resident programs,

(TSR's) such as SideKick, in upper memory,

thus freeing up your DOS memory for larger

application programs.

ZERORAM.COM             Small utility to zero out a block of

memory.  Used to initialize non-DOS

memory to prevent parity check errors.

DIAGS.EXE               A public-domain utility by Joan Riff.

Very nice for 'hackers' to nose around

in the PC to see what's going on.  I

used this extensively in developing

HILOAD to keep track of the interrupt

vectors.

MAPMEM.COM              Another public domain utility.  Gives

a map of what's loaded in your computer

and other information.

RMAP.COM                Similar to MAPMEM, but is memory resident

(and can be HILOADed). Very useful.

VIDEOTSR.COM            A small utility that looks at the

interrupt vector table to see if you

have loaded any TSR's there.  Meant to

be used by batch files to see if it's OK

to go into graphics mode.

MEMPEEK.COM             A utility to examine the entire 1 MB of

the PC memory.  Handy for poking around

in memory.

MEMPEEKC.COM            Same as MEMPEEK, but with de-glitching

code for use on some CGA monitors.  It

is slower, so don't use it unless you

have to.