Documentation for

PageMate

Shareware Version 1.0

======================================================================

PageMate (TM)

Copyright (C) 1991 by R.J. Matter & Associates

All Rights Reserved

The  author makes no warranty of any kind, expressed or implied,  with

regard  to  this  program or documentation. The author  shall  not  be

liable in any event for incidental or consequential damages in connec-

tion  with, or arising out of, the furnishing, performance, or use  of

this program.

----------------------------------------------------------------------

DESCRIPTION

PageMate is a program that monitors your telephone line for an  incom-

ing  ring and then dials a number you specify after a specific  delay.

PageMate is most commonly used in conjunction with display type pagers

(or  "beepers")  and answering machines to notify owners  of  incoming

messages,  but it can call any telephone number and an  answering  ma-

chine  is not necessary. PageMate will work with rotary pulse or  tone

telephone line service.

----------------------------------------------------------------------

SETUP

What  you need: An IBM compatible computer with a 300 baud or  greater

Hayes  compatible modem, a text editor or word processor and  a  tele-

phone line.

With  a  text  editor  or word processor,  edit  the  comma  delimited

PAGE.DAT  file to include the com port number your modem is  installed

on (1,2,3, or 4), a "P" or "T" to indicate PULSE or TONE line service,

the number of seconds PageMate should delay before calling the dialing

string, and the dialing string surrounded by quotation (" ") marks  in

the format:

COM PORT NUMBER,SERVICE TYPE,SECONDS TO DELAY,"DIALING STRING"

Note:  If you use a word processor to do this, make sure you save  the

file in ASCII or non-document form! Also make sure the parameters  are

in  the  exact order shown and that each parameter is separated  by  a

comma (,)!

You  can  place commas in your dialing string to force  the  modem  to

pause for 2 seconds per comma. Special characters like * # are allowed

also.

Example 1:     1,T,180,"555-5555,,,,999-9999#"

Example  #1 translation: Modem is installed on COM PORT  1.  Telephone

line service is TONE. After a delay of 180 seconds, PageMate will call

555-5555,  pause for 8 seconds, then send the number 999-9999 to  dis-

play  on the pager followed by the common # character  indicating  the

end  of number entry.

Note: You must have TONE service to send a number to a display pager.

Example 2:     2,P,120,"9,555-5555"

Example  #2 translation: Modem is configured as COM PORT 2.  Telephone

line service is PULSE. After a delay of 120 seconds PageMate will dial

9 (to get an outside line for example), pause for 2 seconds, then dial

pager number 555-5555.

If  you  are using PageMate with an answering  machine,  allow  enough

delay  time  for the caller to leave a message and for  the  answering

machine  to  cycle for the next call. Usually 3 or 4 minutes  will  be

enough time (180 to 240 seconds).

----------------------------------------------------------------------

DIRECTIONS

The  modem and telephone/answering machine must be on the  same  line.

The  computer and modem must be turned on. Internal modems are  always

on when the  computer is on.

PAGE.EXE  and PAGE.DAT must reside in the same directory. Type  "PAGE"

and the PageMate status window will appear. The date and time will  be

displayed and the seconds should be ticking.

If you specified the wrong communications port in the PAGE.DAT file or

your modem is not on, you will get a "Bad file name" or "Device  time-

out"  error.  You will need to edit the PAGE.DAT file  and  enter  the

correct COM port number and make sure your modem is on. Valid COM port

numbers are 1,2,3, and 4.

You can test PageMate by typing "T". You can exit PageMate at any time

by  pressing the Escape key.

If  everything  is installed and working correctly, you can  turn  off

your  monitor  and leave PageMate and your computer  unattended.  When

PageMate  detects the first incoming ring on your telephone  line,  it

will  begin counting down the delay seconds you specified in  PAGE.DAT

and then call the dialing string you specified in PAGE.DAT.

NOTE: The shareware version of PageMate allows only one incoming  call

per  session.  You must register to receive version 2.0  which  allows

unlimited calls. To register, print and fill out PAGE.FRM and mail  it

with your check or money order for $25 to:

Robert J. Matter - PageMate

P.O. BOX 9042

Highland, IN  46322

To print PAGE.FRM from the DOS prompt type

C> PRINT PAGE.FRM

or

C> COPY PAGE.FRM LPT1

----------------------------------------------------------------------

This software may be distributed freely provided that it is distribut-

ed as a complete set (PAGE.EXE, PAGE.DOC, PAGE.DAT, PAGE.FRM)  without

modification  and no fee is charged other than a nominal  handling  or

duplication fee of $5.00 or less.

by Robert J. Matter

CompuServe ID: 71021,2654

GEnie: R.Matter

----------------------------------------------------------------------

###########################################################################################################