Instructions for Patrick Swayne's

SummaSketch ADI Driver for AutoSketch

** Introduction

This  collection contains a device driver that uses the  Autodesk

Device Interface (ADI) specification to let you use a Summagraph-

ics  MM961,  MM1201, or SummaSketch graphics  tablet  with  Auto-

Sketch.   The  driver makes AutoSketch a more  powerful  drafting

tool  because a graphics tablet allows more  precise  positioning

than a mouse, joystick, or Koala pad.  Support utilities for  the

driver  are included that let you test your graphics  tablet  and

alter the X,Y resolution.

** Distribution

These copyrighted programs are provided free of charge.  They may

be  distributed freely as long as no charge is made for the  pro-

grams  themselves,  or any software package with which  they  are

included.  The programs and all related documentation files  must

be  distributed  together, and may not be  modified.   A  nominal

charge may be made for the disk containing these programs.

** Warranty

These programs are provided AS IS without any warranty, expressed

or implied, including but not limited to fitness for a particular

purpose.

** Disclaimer

In no event will Patrick Swayne be liable to you for any damages,

including any lost profits, lost savings, or other incidental  or

consequential  damages arising out of the use of or inability  to

use  these programs, even if Patrick Swayne has been  advised  of

the  possibility of such damages, or for any claim by  any  other

party.

** Preparation and Testing

If you have a hard disk, copy the files ADISUM.COM,  TESTSUM.COM,

and SETSUM.COM from this collection to your AutoSketch directory.

If you have a floppy only system, you can copy the files to  your

AutoSketch startup disk if there is room, or you can copy them to

your system disk.

The  file ADISUM.COM is the SummaSketch tablet driver.  It  is  a

TSR (Terminate and Stay Resident) program that installs itself in

memory  when you run it and remains there after it exits back  to

DOS.   TESTSUM.COM  is the program that  tests  your  SummaSketch

tablet,  and SETSUM.COM is the program that allows you  to  alter

its resolution.  SETSUM is also used to set up the driver for use

with the particular model of SummaSketch tablet you have.

To  test  the operation of your SummaSketch tablet  with  ADISUM,

1

first  connect  the tablet to your computer as described  in  the

SummaSketch  User's Guide.  Make sure that the stylus or puck  is

plugged  into the tablet, and that the power supply is  connected

and plugged into an AC outlet.  Make the drive/directory contain-

ing ADISUM.COM and TESTSUM.COM the default.  If your  SummaSketch

tablet is connected to serial port COM1, type

ADISUM

at the DOS prompt, and press Enter or Return.  If your tablet  is

connected to COM2, type

ADISUM 2

and press Enter or Return.  After ADISUM signs on, type

TESTSUM

and  press  Enter or Return.  If the driver and your  tablet  are

working correctly, TESTSUM will print

SummaSketch tablet tests ok.

and exit to DOS.  If you see the message

Unable to communicate with tablet.

it may means that you have specified the wrong COM port, or  that

your SummaSketch tablet is not getting power, or that your serial

port  is not functioning correctly.  If the problem is  that  you

specified the wrong COM port, unload ADISUM from memory by enter-

ing

ADISUM U

and reinstall it using the correct command.

If  your  tablet  is connected properly, but it  is  not  working

properly, you will see the message

SummaSketch tablet did not initialize properly.

Analog circuitry test:

Digital circuitry test:

Cursor/stylus operation:

Beside  each of the last three lines above you will see the  word

"Passed" or "Failed", indicating the result of each test.  If the

cursor/stylus operation fails, it probably means that your stylus

or puck (cursor) is not plugged in.  If you plug in the stylus or

puck while the tablet is powered up, and then run TESTSUM, it may

report  that all three tests have failed.  If that  happens,  run

TESTSUM  again.  If either the analog or digital test fails  con-

sistently when you have run TESTSUM a number of times, it  proba-

bly  means  that you have a serious problem in  your  SummaSketch

2

tablet, and you should contact Summagraphics Corporation.

** Configuring AutoSketch

Before you can use ADISUM and your SummaSketch tablet with  Auto-

Sketch,  you will have to configure AutoSketch to work  with  the

tablet.   If you have already configured AutoSketch for use  with

another  pointing device, there will be a file called  SKETCH.CFG

in  your AutoSketch disk/directory.  It is a good idea to  rename

this file so that it will be available if you should decide to go

back  to  the other pointing device.  For example,  if  you  have

configured  AutoSketch  for  use with a  mouse,  you  can  rename

SKETCH.CFG to SKETCH.MOU.  Note:  either renaming or deleting the

file SKETCH.CFG will cause AutoSketch to enter the  configuration

mode the next time it is run.

After  you have renamed SKETCH.CFG, or if you  have not yet  con-

figured  AutoSketch,  make  sure that ADISUM  is  installed,  and

configure  AutoSketch  as directed in its manual.  When  you  are

prompted  to "Select pointing device", enter 1  (Autodesk  Device

Interface Pointer).  When you see the prompt "Interrupt number in

hex  <79h>", just press Return or Enter.  Continue with the  rest

of the configuration as required for your equipment.

** Operation

To use AutoSketch with the SummaSketch tablet and ADISUM,  follow

these steps.

1.   Load ADISUM into memory.  To do this, type ADISUM or  ADISUM

2  at the DOS prompt, and press Enter or Return.   You  only

need  to do this once each type you boot up  your  computer,

but no harm is done if you do it more than once.  If  ADISUM

detects  that  it has been already installed in  memory,  it

will display "ADISUM is already installed" and exit.

2.   Configure ADISUM for your tablet.  ADISUM is  pre-configured

for a 12 x 12 inch tablet, so you can skip this step if  you

have  that type.  If you have a 6 x 9 inch tablet, you  must

use  SETSUM.COM to configure ADISUM to work with it.  To  do

that, enter

SETSUM M9

at the DOS prompt, and press Enter or Return.  You can  also

use  SETSUM  to alter the resolution on both  tablet  types.

This will be explained later.

3.   Run  AutoSketch.  Using the graphics tablet with  AutoSketch

is a lot like using a mouse, except that the position of the

stylus  or  puck on the tablet corresponds  to  an  absolute

position  of the pointer on the screen.  In other words,  if

the stylus or puck is moved from anywhere to the lower  left

corner  of your tablet, the pointer will move to  the  lower

left  corner of your screen.  The pointer will move even  if

3

the  stylus or puck is picked up and placed at the new loca-

tion.

4.   When you are through using AutoSketch, you can remove ADISUM

from memory by entering

ADISUM U

at  the  DOS prompt, and pressing Enter or  Return.   ADISUM

uses  less  than 1 k of memory, but if things are  tight  on

your system, every little bit of memory freed can help.

** Altering the X,Y Resolution

A little explanation of how the ADISUM driver works is  necessary

in order to explain what you will be up against if you decide  to

alter  the X,Y resolution of the driver.  The  ADI  specification

divides the screen up into 20480 units in both the horizontal (X)

and vertical (Y) directions.  This is true even if the screen  is

wider  than  it is high, as is usually the case.   Therefore  the

vertical and horizontal ADI units are usually not the same  size,

and the driver or tablet must compensate for this.

The SummaSketch and MM series graphics pads are capable of having

their horizontal and vertical resolutions set independently,  and

so they can, in themselves, compensate for a screen divided  into

different  horizontal and vertical units.  However,  the  maximum

resolution in this mode is 508 units per inch.  If you have a  12

x 12 tablet, you can see that means 6096 units across the tablet,

which is far less than the 20480 units that the screen is divided

into.   To compensate for this, the ADISUM driver multiplies  the

reports  from the graphics tablet by a factor of 6 if it  is  set

for a 12 x 12 tablet, or by 8 if it is set for a 6 x 9 tablet.

My  goal in choosing the default resolutions used by  the  ADISUM

driver was to have a given movement of the puck or stylus in  the

X  direction  produce a certain movement of the  pointer  on  the

screen, and the same movement in the Y direction produce the same

movement on the screen.  Screen movements were determined by  how

many  drawing units the pointer moved (with the grid  turned  on)

rather  than  by actual movement on the screen as measured  by  a

ruler.   Just about every screen will have a  slightly  different

height to width ratio.

When  a  resolution is specified for the graphics tablet,  it  is

specified as the number of units across the whole tablet,  rather

than the number of units per inch (while keeping in mind that the

resolution cannot exceed 508 units per inch).  When ADISUM is set

up  for  the 12 x 12 tablet, it sets it up with a  resolution  of

3420 units in the horizontal (X) direction, and 4956 units in the

vertical  (Y) direction.  These settings ensure equal  horizontal

and vertical movement, and they also ensure that the entire width

of the drawing tablet is used.  You must move the puck or  stylus

from one side of the tablet drawing surface to the other in order

to move the pointer from one side of the screen to the other.  In

4

the vertical direction, a portion of the tablet at the top is not

used,  because  the screen is not as tall (in  terms  of  drawing

units) as it is wide.

When ADISUM is set up for a 6 x 9 tablet, it sets the  resolution

to 2655 units in the horizontal (X) direction, and 2562 units  in

the  vertical  (Y) direction.  With this tablet,  all  of  tablet

surface is used in the vertical direction, put a small portion at

the right edge is not used in the horizontal direction.  This  is

because the width to height ratio of the tablet (in terms of even

units) is greater than that of the screen.  Note:  I do not  have

a 6 x 9 tablet, and the default values I used are based on calcu-

lations, and have not been verified.

If  you  would  like to experiment with setting  your  tablet  to

different  X and Y resolutions than the default values,  you  can

use the SETSUM program with this syntax

SETSUM x,y

Where  x and y are decimal numbers in the range 100 to 6000.   If

you  have a 6 x 9 tablet, do not supply an X value  greater  than

4572,  or  a  Y value greater than 3048.  When  you  are  through

experimenting,  you  can  return the settings  to  their  default

values by entering

SETSUM M12

if you have a 12 x 12 tablet, or

SETSUM M9

if you have a 9 x 9 tablet.

** Drawing One to One

If you would like to adjust things so that one inch, for example,

equals one drawing unit on the screen, I recommend that you  make

AutoSketch  Zoom an appropriate amount rather than  messing  with

the  graphics tablet resolution.  If you have a 12 x 12  graphics

tablet,  you can turn the grid on and then use the Zoom Box  com-

mand to zoom to a box 11.7 units wide (the tablet drawing surface

is  actually 11.7 x 11.7 inches) by 8 or less units high.   After

the zoom, a one inch movement on the tablet will equal a one unit

movement on the screen.  If you have a 6 x 9 inch tablet, zoom to

a  window 6 units high by 8 or fewer inches wide to get the  same

result.

In  metric  measure, the 12 x 12 tablet is 29.7 x  29.7  cm.   By

setting the grid to a fractional unit size, it should be possible

to zoom to a setting where one centimeter of movement on the  pad

equals one grid unit on the screen.

5

#####################################################################