���������������

QUICK INSTALL

���������������

There are two versions of TED v3.2a, one for AutoCAD running normal

AutoLISP; another for AutoCAD running Extended AutoLISP or ACAD386.

����������

ANSI.SYS

����������

You must have the ANSI.SYS console driver loaded.  It is loaded by

DOS during bootup.  The following line must appear in your CONFIG.SYS

file (the "d:\pathname\" denotes where ANSI.SYS is located).

DEVICE=d:\pathame\ANSI.SYS

�����������������

Normal AutoLISP

�����������������

Assuming the TED v3.2 files are on a disk in drive A: and you want TED

to be installed in the C:\ACAD directory, do the following.

COPY A:\TED.LSP C:\ACAD\*.*

COPY A:\TED.00? C:\ACAD\*.*

The AutoLISP heap and stack sizes must be set in either your AutoCAD

startup batch file or AUTOEXEC.BAT.  Include the following lines.

SET LISPHEAP=35000

SET LISPSTACK=10000

From within the AutoCAD drawing editor, load TED.

(load "TED")

This creates a command called "TED" which is callable at any time from

the "Command:" prompt.

�����������������������������

Extended AutoLISP / ACAD386

�����������������������������

Assuming the TED v3.2 files are on a disk in drive A: and you want TED

to be installed in the C:\ACAD directory, do the following.

COPY A:\TED-BIG.LSP C:\ACAD\*.*

The AutoLISP stack size must be set in either your AutoCAD startup

batch file or AUTOEXEC.BAT.  Include the following lines.

SET LISPSTACK=20000

From within the AutoCAD drawing editor, load TED.

(load "TED-BIG")

This creates a command called "TED" which is callable at any time from

the "Command:" prompt.

[End Of File: QWIKINST.DOC 03/09/91]