CPCHND.SYS Documentation

CPCHND.SYS is a DOS device driver that allows the Complete

PC Scanner to be used with PC PaintBrush with INTERRUPTS.

Dither and Brightness are set on the scanner itself.

Resolution is dependent on the scanner type:

Scanner			    Resolution(s) in DPI

Complete Hand Scanner/400	    	400,300,200

Complete Half Page Scanner		200

Complete Half Page Scanner/400 		400,300,200

It is unnecessary to run the bundled software that comes with

the Complete PC Scanner. Insert CPCHND.SYS into the CONFIG.SYS file

using the appropriate options. At boot the driver will respond with

a message based on the options. When PaintBrush is run, the scanner

will light up briefly before the tools are drawn on the screen. If this

happens you are in buisiness. If not, you must power down and re-address

the dip	switches and try again.

Options are:

Default	Complete Hand Scanner/400

/1	Complete Hand Scanner/400

/2	Complete Half Page Scanner

/3	Complete Half Page Scanner/400

/C	channel for DMA, 1 or 3 (default is channel 3)

/H	hex port value, default is 3E0H (no 'H' needed)

/I	irq number 2-5 (INTERRUPT number is required)

Examples:

device=cpchand.sys /2 /H2E0 /I2 /C3

Complete Half Page Scanner,  port = 220H ,IRQ = 2, DMA=3

Available Port Settings:

220H	260H	2A0H	2E0H	320H	360H	3A0H 	3E0H

Scanner interface card DIP Switches

Port	#1	#2	#3	#4	#5	#6	#7	#8

|N.A.	|IRQ2	|IRQ3	|IRQ4	|IRQ5

220H	on 	on 	on 	off	off	off	off	off

260H	off	off	on 	off	off	off	off	off

2A0H	off	on 	on 	off	off	off	off	off

2E0H	on 	off	off     off	off	off	off	off

320H	off	on	on	off	off	off	off	off

360H	off	on 	off	off	off	off	off	off

3A0H	off	off	on 	off	off	off	off	off

3E0H	off	off	off	off	off	off	off	off

Note: DIP switch  UP = ON

Make sure that only one interrupt is set on.