DFI.SYS

The DFI.SYS driver supports the DFI 2000, DFI 3000 and the DFI 3000 PLUS

hand scanners.

DFI.SYS allows the DFI 3000 and the DFI 3000 PLUS to use four resolutions,

400, 300, 200, and 100 dpi. The DFI 2000 only supports 200 dpi.

Here are some examples of the switches available to configure DFI.SYS to

different computer environments.

DFI 3000 (default)  'device=dfi.sys'

DFI 2000  'device=dfi.sys /A'

Micro Channel Adapter  'device=dfi.sys /M'

DFI 3000 Plus  'device=dfi.sys /P'

While this driver was primarily designed for use under Windows, it works well

in either Publishers or Plus, as long as there is EMS available. The scanner

driver automatically recognizes the type of EMS available and displays what

it finds at boot time, so the /W option has been removed.

DFI.SYS will work without EMS, but in a restricted manner. The scanned image

must be reduced in size. For instance, on most 640 K machines, typical scanned

images sizes are:

Resolution   Image Size   DOS Memory Used

200 DPI      4" X 6"          120K

300 DPI      4" X 3"          120K

400 DPI      4" X 1.5"        120K

Always use a 2 color mode when scanning in images, since the scanner only

provides black and white information. This will leave more DOS ram free to

scan into.

The '/M' switch is for the DFI Micro-Channel interface.

If the driver doesn't work or a bad image is scanned in, check for

interference from a printer port setup for LPT2.

Be sure that Paintbrush and the switches on the Handy Scanner are set for

the same resolution, or else there will be garbage included on the side of

your scanned image.

Image quality is very dependent on scanning speed. Scanning too fast will

squash the image. The DFI 3000 PLUS has a scan over-run light, that goes out

if you are scanning too fast. With PaintBrush, you can stop scanning, release

the scan switch momentarily and the over-run light will come back on.

LIMITATIONS:

THIS DRIVER IS GAMMA RELEASE LEVEL.  THIS MEANS THAT IT WORKS WELL ENOUGH TO

PASS OUR TESTING DEPARTMENT; HOWEVER, WITH THE MYRIAD OF PC HARDWARE

AVAILABLE ON THE MARKET TODAY, THERE MAY BE SOME PROBLEMS REMAINING.

WE DO KNOW THAT THIS DRIVER (VERSION 4.01) CAN PRODUCE SEAMLESS IMAGES WITH

EMS (LIM 4.0 SPEC.) AND 'AT' CLASS COMPUTERS.