Genius Documentation

ZSoft Corporation's GENIUS.SYS is designed to work with the

Genius hand scanner. This scanner device driver presently has

only one option: DMA channel.

DMA channel:	/C1		DMA channel 1

/C3		DMA channel 3

Example CONFIG.SYS entry:

DEVICE=GENIUS.SYS /C3

The driver is setup for the Genius 4500 and DMA channel 3.

This device driver does not support interrupts, therefore

to obtain the best performance, GENIUS.SYS requires EMS.

The scanner supports 100,200,300 and 400 Dots per inch.

Paint Brush may save buffers to the hard disk while scanning.

When the disk is accessed, scanlines are lost, resulting in gaps

in the image. To prevent this, the entire scan may be scanned into

Expanded Memory and then read back as required, greatly reducing the

chance of losing scan data.