DO NOT READ THIS OR WORRY ABOUT IT IF YOU DO * NOT * RUN A

NON STANDARD PORT ADDRESS OR A NON-STANDARD IRQ!

-----------------------------------------------------------

This game will allow those with Multi-nodes and ports that

utilize Non-Standard addresses and non-standard IRQ values.

It is very simple to alter the normal Port 3 or Port 4 to

utilize any address or interrupt that you wish.

To Alter port 3

---------------

The normal port 3 address is &H3E8 in Hex with an IRQ of 4.

To change this to another value you need only to create a two

line ASCII text file using Copy Con, Edlin, or other word

processor.

The two line file you create MUST be called PORT3.TXT.

The first line of Port3.txt contains either the port # (3 or

4) OR the Hex value of the address you wish to use (in the

format of &H???). You may enter any address your hardware

supports in this HEX format.

The second line is the interrupt that you wish to use for the

address.  Proper IRQ's range from 1 to 7.

An example of Port3.txt would be as follows:

&H3E8             <- Hex value of the address

4                 <- IRQ to use

Thats all there is to it!

To alter Port #4

----------------

If it is the fourth port that you need a non-standard

address and interrupt on you need only to create a file

called PORT4.TXT.  Port4.txt will contain the same info as

described above in Port3.txt. It is just a two line file with

the new address in Hex on the first line and the new IRQ on

the second line.

I hope this helps�����������������������������������������������������������������������������������������������������������������������������������������������������������������������������������