XA Command Interpreter

Version 01.01

March  1991

Copyright 1991 by Bruce Christensen

All rights reserved.

Table of Contents

1  Introduction....................................................  3

2  Installation....................................................  4

3  XA - the X10 Command Interpreter................................  5

4  XA Tokens.......................................................  7

4.1  XA Token Reference............................................  8

4.2  Tokens in detail.............................................. 10

4.2.1  Action Tokens (ON, OFF, DIM)................................ 10

4.2.2  Module Address Tokens (HOUSE, UNIT, A1...P16)............... 11

4.2.3  Coordinate Tokens (LATITUDE, LONGITUDE, TIMEZONE)........... 12

4.2.4  Clock Tokens (SYNCHRONIZE X10, PC).......................... 13

4.2.5  Day Tokens (SUNDAY...SATURDAY, WEEKDAYS, WEEKENDS, etc)..... 14

4.2.6  Time Tokens (TIME, SUNRISE/SET, OFFSET, SECURITY)........... 15

4.2.7  Misc Tokens (BASECODE, PORT, DEFINE, EVENT, CLEAR, DIAGNOSE,

MONITOR, IRQ, IO)........................................ 16

5  Command Files................................................... 20

6  Monitor Events.................................................. 21

7  POWERUP Utility................................................. 23

8  License Agreement and Registration.............................. 24

XA Command Interpreter                                                   PAGE 3

1) Introduction

This floppy contains a set of extremely useful DOS utilties for the

X10 Computer Interface (CP290). These utilties are being marketed under

the try-before-you-buy concept to allow you a complete test of the

product before you are required to register. See Section 8.0 - License

Agreement and Registration for more details.

Utilities provided in this release include:

A) XA - X10 Command Interpreter.

This utility reads and downloads commands to the CP290

in two modes. The first mode is from command line arguments,

ie, from the DOS prompt. The second mode can read an

entire file of commands. These commands can be either

timed events or direct commands.

In addition to these functions, the interpreter has several

features not found with the original X10 software. Some of

these features are:

1) Sunrise/Sunset times based on the latitude,

longitude, and timezone of your city.

2) Synchronize your PC's clock with the time maintained

by the CP-290, or synchronize the CP-290 clock with the

time maintained by your PC.

3) Monitor events as the CP-290 executes them.

4) The ability to change the base housecode without

losing all the data already stored in the interface.

5) String substitution in the command file allows you

to equate a device (FAMILY_ROOM_LAMP) with its module

address (HOUSE A UNIT 3).

6) Comments are allowed in the file to properly document

its content.

XA Command Interpreter                                                   PAGE 4

B) POWERUP - AUTOEXEC Time Check Utility

This utility is provided for those of you that control your

computer with an X10 Appliance module. When placed in your

AUTOEXEC.BAT file, this utility compares the current PC time

with a pre-set time you define. During your computers powerup,

if the PC's time falls within a certain "window", it will

return an errorlevel of 1. This errorlevel can then be checked

and used to perform any number of functions you desire.

For instance, X10 turns my computer on at 3:00 AM every

Sunday morning. POWERUP checks the computers time and then

invokes a complete optimization and backup of my hard drives as

well as downloads updated times to the X10 Computer Interface.

There will be more on this subject in Section 7 - Powerup

Utility.

2) Installation

You will probably want to copy the contents of this floppy to the same

drive and directory in which your X10 software resides. It's also a

good idea to include this drive and directory within your PATH statement

of the AUTOEXEC.BAT file.

Ex: SET PATH=C:\DOS;D:\X10;...

You should set the XA environment variable to the drive and directory

of your command files. This can also be placed in your AUTOEXEC.BAT file.

Ex: SET XA=D:\X10

(See the AUTOEXEC.BAT supplied with this package for an example).

XA Command Interpreter                                                   PAGE 5

3) XA - The X10 Command Interpreter

As stated earlier, XA can operate on commands from the DOS command

line, or from a file. In either mode, XA uses several options to

alter its operation. The XA command has the following form:

XA [f=filename] [c=comm port] [i=irq] [o=io_addr] ["commands"]

[-e] [-s] [+m] [+h]

The options may appear in any order, but the presence of some of

these options negates other options.

[f=filename]   - this is the name of the file containing

the commands to be interpreted by XA. To use XA

in its file mode, you would type:

XA f=filename

The default file name is XA.CMD. If you want to use

the default file, you do not have to specify the

filename - just type:

XA

[c=comm port]  - this allows you to select the serial port

for communications with the interface. You may

select ports 1...4. For example, if your CP290

is connected to COM3:

XA c=3

The default port is 1. To use the default, you do

not have to specify the port number - just type:

XA

See Section 4.2.7 - Miscellaneous tokens for a

different way to set the comm port within a command

file. Also, see the next two options if your comm

port uses a different IRQ level, or a different IO

port address.

XA Command Interpreter                                                   PAGE 6

[i=irq]        - use this option if your communications port uses

a different IRQ level than what is "standard" (see

Section 4.2.7 - PORT COMx for the default

assignments). The default IRQ level is 4. If your

serial port uses IRQ 5, then include:

XA i=5

[o=io_addr]    - this option allows you to select a different IO

address from the default 3F8 hex. If your serial

port is setup for 2E8 hex, then use:

XA o=2E8

["commands"]   - "commands" consist of several tokens (huh? - see

Section 4 - Tokens) that describe an event or

direct command. For example:

XA "A1 ON"

In the above example, A1 and ON are tokens.

Together they form a single command. Multiple

commands may appear in the command line, but they

must be enclosed by quotation marks (" "). The

next example shows the use of multiple commands

to create a flashing effect:

XA "A1 ON" "A1 OFF" "A1 ON" "A1 OFF"

One other note, the presence of any commands on

the command line will nullify the use of a file.

Therefore in the next example, the XA utility will

ignore the file request and process just the

commands:

XA f=null.cmd "A1 A2 A3 A4 ON"

[-e]           - this option is used to prevent any communications

to the interface. Use this option only when

you want to validate commands in a file.

[-s]           - this option is used to prevent sorting the commands

based on their EVENT numbers (see EVENTS described

in Section 4.2.7 - Miscellaneous tokens).

[+m]           - this option will monitor the comm port for any

events reported by the CP-290. Press the <ESC>

key to terminate. Note: You can run this as a

background task in Windows 3.0 (386 enhanced

mode) in order to avoid tying up your computer.

[+h]           - displays a brief summary of the options described

above.

XA Command Interpreter                                                   PAGE 7

4) XA Tokens

What are tokens?

A token is to a command as a word is to a sentence. Individual words are

constructed to form a complete sentence - tokens are constructed to form

complete commands. This section will describe each token that is

recognized by the interpreter. In most cases, tokens may appear anywhere

in the command. For example, the command

XA "A1 ON"    may also be constructed as

XA "ON A1"

In other instances, tokens are must sometimes be arranged in groups,

and their order is important. For example, the command to set your PC's

internal clock to the time that is maintained by the CP290 is:

XA "SYNCHRONIZE PC"      <---This is legal

If the tokens SYNCHRONIZE and PC were reordered as in the following

command:

XA "PC SYNCHRONIZE"      <---This is illegal

The interpreter would respond with the following message:

WARNING: Unrecognized token "PC" in line 1.

Therefore, throughout this documentation any tokens that must be

arranged in a certain order will be contained in curly braces {..}.

XA Command Interpreter                                                   PAGE 8

4.1) Token Reference

The following is list of available tokens. They are discussed in more

detail in the next section:

4.1.1 Action tokens -

ON             - turns ON the specified modules.

OFF            - turns OFF the specified modules.

{DIM %%}       - DIM to a % (percentage) level of brightness.

4.1.2 Module address tokens

{HOUSE x}      - module HOUSEcode A...P

{UNIT z}       - module UNITcode 1...16, ALL

A1...P16       - HOUSE UNIT combined

4.1.3 Coordinate tokens:

{LATITUDE d m} - your LATITUDE in Degrees and Minutes

{LONGITUDE d m}- your LONGITUDE in Degrees and Minutes

{TIMEZONE x}   - your TIMEZONE (Standard/Daylight Savings time)

4.1.4 Clock tokens:

{SYNCHRONIZE PC}  - set PC time based on X10 time

{SYNCHRONIZE X10} - set X10 time based on PC time

4.1.5 Day tokens:

SUNDAY,    SUN - Specific day for event to occur

MONDAY,    MON -     "

TUESDAY,   TUE -     "

WEDNESDAY, WED -     "

THURSDAY,  THU -     "

FRIDAY,    FRI -     "

SATURDAY,  SAT -     "

WEEKDAYS       - Event occurs only on weekdays

WEEKENDS       - Event occurs only on weekends

EVERYDAY       - Event occurs everyday

TODAY          - Event occurs once today only

TOMORROW       - Event occurs once tomorrow only

XA Command Interpreter                                                   PAGE 9

4.1.6 Time tokens:

{TIME hh:mm AM} - Schedule event time

{TIME hh:mm PM}

SUNRISE         - Calculate time of sunrise

SUNSET          - Calculate time of sunset

{OFFSET +/-mm}  - Event + or - 60 mins of programmed time (dawn)

NORMAL          - Event occurs at specified time

SECURITY        - Event occurs within the hour of programmed time

4.1.7 Miscellaneous tokens:

{BASECODE x}   - change house BASECODE of interface to A...P

{PORT COMx}    - send commands via PORT COM1..4

{IRQ x}        - customize IRQ level for port.

{IO x}         - customize port address (must be in hex).

{DEFINE x z}   - substitute string 'x' for string 'z'

{EVENT xxx}    - Place event xxx in a known location of CP290 memory

{CLEAR xxx}    - Delete event xxx from CP290 memory

DIAGNOSE       - Invoke CP290 self-test routine

MONITOR        - watch events as they happen

XA Command Interpreter                                                   PAGE 10

4.2) Tokens in Detail

4.2.1 Action Tokens

ON

This token informs the interface to turn ON the selected

modules.

Example: XA "A1 ON"

OFF

This token informs the interface to turn OFF the selected

modules.

Example: XA "A1 OFF"

{DIM %%}

These two tokens specify the intensity level of the selected

lamp module or wall switch module. The modules will turn on,

brighten to full intensity, then dim to the percentage

specified in the second token. There are 16 different

brightness levels available, the percentage you choose is

mapped to one of these levels.

Example: XA "A1 DIM 50"

Notes:

1) Only one token in this category should appear in a command. If

two or more of these tokens appear, only the last one will be

acted on. For example:

XA "A1 ON OFF" - will be interpreted as:

XA "A1 OFF"

If you were trying to flash the lights, the proper way to perform

this action is:

XA "A1 ON" "A1 OFF"

2) You may activate as many modules as you want in a single command

providing these modules share the same HOUSE code:

XA "A1 A2 A3 A4 A5 A6 ON"

To activate modules with different HOUSE codes, use separate

commands:

XA "A1 A2 A3 ON" "B1 B2 ON" "C1 OFF"

3) If you download a timed event that contains the DIM token, the

X10 interface program (X10.EXE) will report the action taken on

the module as DIM 90%.

XA Command Interpreter                                                   PAGE 11

4.2.2 Module Address tokens

{HOUSE x}

These two tokens specify the HOUSE code of the module you

want to select. There are 16 HOUSE codes available, lettered

A through P. This method of selecting address requires the

use of the UNIT token. Only one HOUSE token should appear in

a command.

{UNIT y}

{UNIT ALL}

These two tokens specify the UNIT code of the module you want

to select. There are 16 UNIT codes available, numbered 1

through 16. Specifying UNIT ALL operates on all 16 modules

addressed by the HOUSE token. This method of module addressing

requires the use of the HOUSE token.

Examples: XA "HOUSE A UNIT 1 ON"   or,

XA "UNIT 1 HOUSE A ON"

Multiple UNITs may appear in a single command.

Example:  XA "HOUSE A UNIT 1 UNIT 2 UNIT 3 ON"

Turn off all units on HOUSE P.

Example: XA "HOUSE P UNIT ALL OFF"

A1...P16

This is a shortcut method of specifying both HOUSE and UNIT

tokens. The house code must appear first, followed by the

unit code. Specifying ALL is not permitted.

Example:  XA "A1 ON" "P16 OFF"

Notes:

1) You may activate as many modules as you want in a single command

providing these modules share the same HOUSE code:

XA "A1 A2 A3 A4 A5 A6 ON"

To activate modules with different HOUSE codes, use separate

commands:

XA "A1 A2 A3 ON" "B1 B2 ON" "C1 OFF"

XA Command Interpreter                                                   PAGE 12

4.2.3 Coordinate tokens

These tokens enable the precise calculation of sunrise and sunset

times for your location. If your events don't require these

functions, then don't worry about them. This information can be

found in almanacs, or extrapolated from maps. Your local library or

city hall may also have the precise coordinates.

{LATITUDE d m}

Your LATITUDE in degrees (0...+/-90) and minutes (0...59).

Example: LATITUDE 41d35m

{LONGITUDE d m }

Your longitude in degrees (0...+-180) and minutes (0...59).

Longitudes West of the prime meridian are positive, those East

of the prime meridian are negative. In the US, all longitudes

are positive.

Example: LONGITUDE 81d20m

{TIMEZONE x}

Exact calculations depend on the local time. Use the

following table to determine your time zone.

Zone                     x

====================     =========

Eastern Standard Time:        5

Eastern Daylight Time:        4

Central Standard Time:        6

Central Daylight Time:        5

Mountain Standard Time:       7

Mountain Daylight Time:       6

Pacific Standard Time:        8

Pacific Daylight Time:        7

Example: TIMEZONE 5

XA Command Interpreter                                                   PAGE 13

Notes:

1) Since all three tokens in this group are required, it is easier

to include these tokens in a file and invoke the XA utility in

the file mode:

Example: XA f=rise_set.cmd

where the file "rise_set.cmd" contains the following

commands:

:

LATITUDE 41d35m

LONGITUDE 81d20m

TIMEZONE 5

:

4.2.4 Clock tokens

{SYNCHRONIZE PC}

These tokens instruct the XA utility to set your computers

clock based on the current time retrieved from the CP290. It

will not affect the computers date.

Example: XA "SYNCHRONIZE PC"

{SYNCHRONIZE X10}

These tokens instruct the XA utility to set the time and day

of the CP290 internal clock based on the current time

retrieved from your computer.

Example: XA "SYNCHRONIZE X10"

Notes:

1) You might want to include the command:

XA "SYNCHRONIZE PC"

in your AUTOEXEC.BAT file to set your PC's internal clock if

your system lacks a on-board battery backed-up clock. Note that

this command will not set up the date since X10 does not

contain a real-time calendar.

2) Use:

XA "SYNCHRONIZE PC"   or,

XA "SYNCHRONIZE X10"

in your AUTOEXEC.BAT file if either your PC or CP290 do not

keep accurate time. My CP290 tends to keep better track of time

than my PC does.

XA Command Interpreter                                                   PAGE 14

4.2.5 Day tokens:

Use of these tokens indicate a timed event, ie, the complete command

will be downloaded to the interface and stored there for later

activation.

SUNDAY,    SUN

MONDAY,    MON

TUESDAY,   TUE

WEDNESDAY, WED

THURSDAY,  THU

FRIDAY,    FRI

SATURDAY,  SAT

These tokens specify the specific day of the event. Multiple

day tokens may appear within a command.

Example: XA "MONDAY FRIDAY A1 ON TIME 7:30 AM"

WEEKENDS

Event occurs on SATURDAY and SUNDAY

WEEKDAYS

Event occurs MONDAY TUESDAY WEDNESDAY THURSDAY FRIDAY

EVERYDAY

Event occurs MON TUE WED THU FRI SAT SUN

TODAY

Event will occur today only at the specified time. It is then

deleted from the interface after midnight.

TOMORROW

Event will occur tomorrow only at the specified time. It is

then deleted from the interface after midnight.

Notes:

XA Command Interpreter                                                   PAGE 15

4.2.6 Time tokens:

Use of these tokens indicate a timed event, ie, the complete command

will be downloaded to the interface and stored there for later

activation.

{TIME hh:mm AM}

{TIME hh:mm PM}

Schedule event at a certain time.

Example: XA "SUNDAY ON TIME 12:30 AM D1"

SUNRISE

SUNSET

These tokens will calculate the sunrise or sunset for the day

token in the command. The use of these tokens also requires the

LATITUDE, LONGITUDE, and TIMEZONE tokens to be included.

Therefore, these tokens should be used in the file mode.

Example: MONDAY ON SUNSET D1

If multiple day tokens appear in the command line, the

sunrise or sunset will be calculated for the last day token

that appears in the command line.

Example: MONDAY TUESDAY WEDNESDAY ON SUNSET D1

If more accuracy is needed, then break each daily event into

its own command.

Example: MONDAY ON SUNSET D1 EVENT 0

TUESDAY ON SUNSET D1 EVENT 1

WEDNESDAY ON SUNSET D1 EVENT 2

{OFFSET +-mm}

Push or postpone the scheduled event by up to 60 minutes. This

token is useful if you want to activate lights at dusk rather

than sunset, or to deactivate lights at dawn rather than

sunrise. The '+' sign is the default condition, you must

specify the '-' if needed.

Example: Turn on lights 30 minutes after sunset

MONDAY ON SUNSET D1 EVENT 0 OFFSET 30

Turn lights off 45 minutes before sunrise

MONDAY OFF SUNRISE D1 EVENT 1 OFFSET -45

XA Command Interpreter                                                   PAGE 16

NORMAL

This token activates the event at the specfied time.

It is the default mode, therefore it does not need to be

specified.

SECURITY

This token allows the interface to randomly select the

activation time of the event. This activation occurs within the

hour of the programmed time. Therefore, if the downloaded

time was for 7:18 PM, actual activation time could range from

7:00 PM through 7:59 PM. This mode provides a security

feature giving your lighting a more lived-in look.

Example: XA "A1 ON TIME 7:30 PM SECURITY EVERYDAY"

4.2.7 Miscellaneous tokens:

{BASECODE x}

These tokens change the base housecode of the interface

WITHOUT losing the data stored in the interface. Prior to

changing the housecode, a complete upload of all events

stored in the interface is performed. The housecode is

changed, then all events are downloaded back into the

interface. Valid housecodes are A...P.

Example: XA "BASECODE D"

During upload, the interface sends all 128 possible events that

are stored in its memory. If an event actually exists, then an

up arrow is displayed, otherwise a dot (�) is displayed. During

download, a down arrow is displayed. Downloads will take

significantly longer due to the overhead involved with sending

synch bytes for each event.

XA Command Interpreter                                                   PAGE 17

{PORT COMx}

These tokens allow you to specify which comm port the

interface is attached to. Legal ports are:

COM1 = I/O address 0x3F8   Interrupt 4

COM2 =      "      0x2F8       "     3

COM3 =      "      0x3E8       "     4

COM4 =      "      0x2E8       "     3

These tokens were intended to be placed in command files.

They will take precedence over the 'c=x' command line

argument (see section 3).

Example line in a command file:

PORT COM3

If your port assignments do not match those in the above

table, then use the command line parameters [i=x] and [o=xxx],

or the file mode tokens "IRQ" and "IO" (see below) to customize

the communication characteristics.

{IRQ=x}

Use these tokens in your command file only if your

communications port uses a different IRQ level than what is

"standard" (see the above table for the default assignments).

The default IRQ level is 4. If your serial port uses IRQ 5,

then use the following line in your command file:

IRQ=5

These tokens were intended to be placed in command files.

They will take precedence over the 'i=x' command line

argument (see section 3).

{IO=xxx}

This option allows you to select a different IO address from

the default 3F8 hex. If your serial port is setup for 2E8 hex,

then use the following line in your command file:

IO=2E8

These tokens were intended to be placed in command files.

They will take precedence over the 'o=xxx' command line

argument (see section 3).

XA Command Interpreter                                                   PAGE 18

{DEFINE x z}

These tokens were designed to be used in command files. They

allow you to substitute several tokens into a single token of

your choice. They help to make command files more readable. Up

to 50 custom tokens are allowed. For example, you can assign a

token "CHRISTMAS_ON" to an equivalent group of tokens "HOUSE A

UNIT 4 ON TIME 8:00 PM EVERYDAY" as follows:

DEFINE CHRISTMAS_ON HOUSE A UNIT 4 ON TIME 8:00 PM EVERYDAY

============ =======================================

x                          z

The 'x' and 'y' strings can be up to 60 characters each.

Notice that the 'x' portion uses underscore characters (_) to

separate the words but to keep the string contiguous. This is

important because the XA interpreter looks for a SPACE

character to separate the 'x' and 'y' components.

Later in the command file, you can use the following command

to activate your Christmas lights:

CHRISTMAS_ON

Internally, XA will convert this string to its equivalent:

HOUSE A UNIT 4 ON TIME 8:00 PM EVERYDAY

More Examples:

:

DEFINE PLANT_GROW_LIGHTS A8

DEFINE OUTSIDE_LIGHTS A1

DEFINE ALL_ON HOUSE A UNIT ALL ON

PLANT_GROW_LIGHTS ON TIME 6:00 AM EVERYDAY

PLANT_GROW_LIGHTS OFF TIME 12:00 AM EVERYDAY

OUTSIDE_LIGHTS OFF SUNRISE EVERYDAY

ALL_ON SUNDAY TIME 7:30 PM

The use of DEFINES is helpful if you want to the XA utilty

monitor the CP290 for events as they occur. See Section 6

on Monitoring.

**** Important Note ****

DEFINEs must be declared before being used. The following

would cause an error message because the DEFINE was placed

out of order:

OUTSIDE_LIGHTS ON SUNSET EVERYDAY

DEFINE OUTSIDE_LIGHTS A1

XA Command Interpreter                                                   PAGE 19

{EVENT xxx}

The CP290 can store up to 128 events in its memory. Each event

occupies fixed locations in memory. When you download events

to the interface, an event number will be assigned to the

command. The first event will have an event number of 0, the

second will be 1, etc. Therefore, any event that was

previously stored at those locations will be overwritten. The

token {EVENT xxx} allows you to override the number assigned to

this command.

If you normally use one command file then the need to use

this token is probably unnecessary. If multiple command

files are used, then this token is helpful.

Example: MON ON SUNSET D1 EVENT 100

TUE ON SUNSET D1 EVENT 101

WED ON SUNSET D1 EVENT 102

{CLEAR EVENT xxx}

These tokens delete the specified event from the computer

interface. See description for EVENT xxx above. Only 1 event

may be cleared per command.

Example: XA "CLEAR EVENT 100" "CLEAR EVENT 101"

DIAGNOSE

This token triggers the X10 diagnostic routine. You will be

informed of the results after a few seconds.

*** WARNING ***

The diagnostics on the CP290 will

destroy all the event information stored

in its memory. Make sure you have all

the event information contained in a

command file so that you can restore

this data once the diagnostics have

completed.

MONITOR

This token informs XA to continuously monitor the

communications port for any messages sent by the CP290 when

it executes a stored event. See (Section 6 - Monitor Events)

for more information.

XA Command Interpreter                                                   PAGE 20

5)  Command files

Since the CP290 is only capable of storing a single weeks worth of

events, command files were created to allow easy update of the events

that may need to revised due to ever-changing conditions, such as sunrise

and sunset times.

Command files should only be created with an editor or word processor

that saves files in pure ascii format. Any extraneous characters or

control codes will cause the XA interpreter to ignore your commands.

You may place comments in your command file. A comment is delineated by

the pound sign character '#'. This character must appear in the first

column of the line.

# This is an example comment.

Comments are ignored by the interpreter. Therefore, comments can be

placed in front of commands you do not want to download to the interface

at this time. For example, you can hide your Christmas lighting commands

from the interpreter once the holidays are over:

#DEFINE XMAS HOUSE A UNIT 2

#XMAS ON EVERYDAY SUNSET OFFSET 20

#XMAS OFF WEEKDAYS TIME 10:00 PM

#XMAS OFF WEEKENDS TIME 12:00 AM

Blank lines are permitted in the command file.  These allow you to

group similar events together.

See the sample command file "XA.CMD" (included in this package) I use for

automating events in my home.

XA defaults to reading a command file if there are no commands in the

command line. The default command file is called 'XA.CMD'. You may keep a

number of command files around. To use a different command file, use the

'f=' parameter when starting XA. For example, if you wanted to execute

the commands found in a file called 'MONITOR.CMD', you would start the XA

utilty as:

XA f=MONITOR.CMD

The next section discusses the use of monitoring and uses a command

file called MONITOR.CMD to show how this is done.

XA Command Interpreter                                                   PAGE 21

6)  Monitor Events

The CP290 sends a message to your computer after it executes an event

stored in its memory. Most software packages (including X10.EXE shipped

with the CP-290) will ignore these messages. XA allows you to monitor

these events if desired.

XA will enter its monitor mode after all other commands have been

processed. You can activate the monitor feature in two ways, as a

command, or as a command line parameter.

Command example:

XA "MONITOR"

Command line parameter example:

XA +m

In the first example, XA "MONITOR",  the token MONITOR informs XA to

watch for any transmissions from CP290. The second example use command

line options and has the same effect, but remember that if no commands

were actually present, the XA utility will try to open the command file

'XA.CMD' and process the commands found in there. If you don't want that

to happen, use the '-e' option which tells XA not to execute (or

download) those commands, as in the next example:

XA +m -e

When XA begins to monitor, it displays the following message:

Monitor events. Press <ESC> when finished...

XA waits for a message from the CP290. You may exit this function by

pressing the <ESC> key.

When the CP290 executes an event it sends a message to the PC with the

following information; the HOUSE code, UNIT code, and the event

FUNCTION (ON, OFF, DIM). XA takes this information and adds the time

and date information from your PC's clock and prints the following

message:

Sun Mar 24 07:00:13 1991 HOUSE A UNIT 5 ON

XA Command Interpreter                                                   PAGE 22

This message is semi-useful; it describes what just occurred but if you

forgot what was addressed as HOUSE A UNIT 6 then the message is pretty

meaningless. You can give this module a more descriptive name by using

a command file that includes DEFINE statments for all your modules.

Below is sample command file (MONITOR.CMD) that describes what we're

talking about:

DEFINE Porch/Garage_Lights HOUSE A UNIT 1

DEFINE Plant_Grow_Lights HOUSE A UNIT 5

DEFINE Family_Room_TV HOUSE A UNIT 6

DEFINE Basement_Computer HOUSE A UNIT 7

Now when a message is broadcast by the CP290, a scan of all the

definitions will be made to see if the HOUSE and UNIT codes match. If

so, then the description will be used instead of the codes. Therefore,

you would get the following message on your screen.

Sun Mar 24 07:00:13 1991 (A5) Plant_Grow_Lights ON

Sun Mar 24 18:43:13 1991 (A1) Porch/Garage_Lights ON

In the above messages, a combined House and Unit code (A6) is

automatically provided - you don't have to include it as part of the

DEFINE token.

Also, the DEFINE must include BOTH the HOUSE and UNIT tokens when

describing an object. You can not use the short cut method, such as:

DEFINE Porch/Garage_Lights A 1             <=== will NOT work.

DEFINE Porch/Garage_Lights HOUSE A UNIT 1  <=== will work.

If different modules are controlled by a single event, the module which

has the lowest UNIT code will be displayed. Therefore, if modules C9 and

C10 are programmed as a single event, only C9 will be displayed.

Monitoring is useful when running on a multitasking system. It has been

tested as a background task under Windows 3.0 (386 enhanced mode). It

should work for Desqview as well.

XA Command Interpreter                                                   PAGE 23

7)  POWERUP Utility

The POWERUP utility is provided in this package to help you perform

unattended, and automatic event downloads to the interface. POWERUP

should be placed in your AUTOEXEC.BAT file so it will always be executed

after your computer boots. This way, it can decide whether or not it's

time to perform any updates to your interface, and it prevents you from

needlessly sending commands when you don't need to.

The POWERUP command has the following form:

POWERUP d=day s=hh:mm e=hh:mm

The command line parameters may appear in order, and they must all be

present:

d=day     - This is the text string for day you want to check

for. Legal strings are: MONDAY, TUESDAY, WEDNESDAY,

THURSDAY, FRIDAY, SATURDAY, SUNDAY, WEEKDAYS,

WEEKENDS, and EVERYDAY.

s=hh:mm   - This is the "window" start time. Hours must be

entered in 24 hour format only.

e=hh:mm   - This is the "window" end time. Again, 24 hour

format is required.

Example: POWERUP d=SUNDAY s=2:55 e=3:05

If the "day" specified matches the day reported by DOS, and the time

reported by DOS falls within the specified "window", then POWERUP will

return an ERRORLEVEL of 1, otherwise a 0 is returned. Therefore, by

placing the proper batch file commands in AUTOEXEC.BAT, you can now

execute a variety of functions at certain times.

You may enter multiple 'd=day' parameters. For instance, to have Powerup

check for a window begining at NOON and lasting until 1:00pm every

MONDAY, WEDNESDAY, and FRIDAY, you would enter:

POWERUP d=MONDAY d=WEDNESDAY d=FRIDAY s=12:00 e=13:00

if errorlevel == 1 echo It's time!

If your computer is controlled by an Appliance module, you can now

automatically download revised event times. In addition, you can even

perform other actvities such as backups, communications, etc. without

being at your computer. I perform my backups and event revisons on

Sunday mornings at 3:00 AM. This way, I never forget about backing up and

now I don't have to sit around and wait for the process to finish. See

the sample AUTOEXEC.BAT file included in this package that I use to

accomplish this task.

XA Command Interpreter                                                   PAGE 24

8)  License Agreement and Registration

You may make copies of this program, manual, and other files and give it

to your friends, upload it to bulletin boards, or include it in the

library of a non-profit computer club.

I expressly forbid any for-profit venture from selling this software and

documentation, either separately or as part of a "library" diskette.

I disclaim any liability for its use, misuse, or abuse, including any

direct or indirect actions. The user accepts full responsibility for his

or her use of this program.

SUPPORT SHAREWARE!

If you find this software has any value for you, please send a

contribution. Contributions of $20 or more will entitle you to (2) updates

of this program as new enhnacements are added. Also, a completely new

human interface is being designed. Your contribution will enable me to

notify you when it's complete.

Whether or not you contribute, please share this software with others.

Your suggestions are always welcome. Send any comments to the address

given below, or you may contact me on Prodigy.

Bruce Christensen

6594 Hudson Ave.

Mentor, OH. 44060-4545

Prodigy ID:  MHNC39A

This software will be uploaded to the PC-OHIO BBS as new updates become

available. Their number is: (216) 381-3320.

Prodigy is a registered service mark and trademark of Prodigy Services Co.

Desqview is a trademark of Quarterdeck Office Systems.

Windows is a trademark of Microsoft Corporation.

X-10 is a registered trademark of X-10 (USA) Inc.