PRINTGL 1.09 REFERENCE

**** TEST VERSION *** TEST VERSION ****

(C) Copyright Ravitz Software Inc. 1990

Ravitz Software Inc.

P. O. Box 25068

Lexington, KY  40524-5068

g-1

CONTENTS

INTRODUCTION ..................................................... 2

LICENSE AND REGISTRATION                                           3

ASSOCIATION OF SHAREWARE PROFESSIONALS ........................... 3

WHAT IS NEW                                                        4

INSTALLATION ..................................................... 5

OPERATION                                                          6

SYNTAX ........................................................... 7

/A - LOCATION OPTION                                               8

/C - PEN COLOR OPTION ............................................ 8

/D - OUTPUT DESTINATION OPTION                                     9

/F - OUTPUT FORMAT OPTION ....................................... 10

/I - SCALING POINT LOCATION OPTION                                11

/L - PAGE LAYOUT OPTION ......................................... 12

/M - MAGNIFICATION OPTION                                         13

/N - PAGE NUMBER OPTION ......................................... 13

/O - ORIGIN AND ORIENTATION OPTION                                13

/P - PLOTFILE OPTION ............................................ 14

/S - PEN SHADING OPTION                                           14

/T - TEMPORARY FILE OPTION ...................................... 14

/W - PEN WIDTH OPTION                                             15

/X - SUPPRESS MESSAGES .......................................... 15

/Y - Y/D SWITCH SETTING OPTION                                    15

/Z - CHORD ANGLE OPTION ......................................... 15

MESSAGES                                                          16

SERIAL PRINTER INTERFACE ........................................ 16

EPSON 9 PIN PRINTERS                                              17

EPSON AND NEC 24 PIN PRINTERS ................................... 17

HP LASERJET                                                       18

HP PAINTJET ..................................................... 18

IBM LASERPRINTER                                                  19

IBM PROPRINTER .................................................. 19

IBM PROPRINTER X24 COMPATIBLE PRINTERS, IBM QUICKWRITER           19

IBM PROPRINTER X24 .............................................. 19

IBM QUIETWRITER 2 AND 3                                           19

CGA DISPLAY ..................................................... 20

EGA DISPLAY                                                       20

HERCULES DISPLAY ................................................ 20

MCGA DISPLAY                                                      20

VGA DISPLAY ..................................................... 20

BIT MAPS                                                          21

GEM .IMG BIT MAPS ............................................... 21

PRINTGL MENU INTERFACE (PMI)                                      22

PMI SPECIFIC FUNCTIONS .......................................... 22

PMI INPUT AND OUTPUT CHOICES                                      23

PMI PLOT OPTIONS ................................................ 23

PMI PEN OPTIONS                                                   23

PMI PAGE LAYOUT OPTIONS ......................................... 23

PMI OTHER OPTIONS                                                 24

HP-GL ........................................................... 25

HP-GL CHARACTER SETS                                              33

COMPATIBILITY AND TECHNICAL INFORMATION ......................... 37

SUPPORT BULLETIN BOARD                                            38

SUPPORT BY MAIL ................................................. 38

g-2

INTRODUCTION

PrintGL prints an HP-GL (Hewlett-Packard Graphics Language - HP 7475

subset) plotfile  on a matrix printer.  HP-GL is widely supported by

graphics programs  including  AutoCAD,  Generic  CADD, MathCAD, SAS,

MICROCADAM, Schema, and  many more.  PrintGL has native mode drivers

for  Epson  and NEC compatible 9 and  24 pin dot matrix printers, HP

Laserjet, Deskjet, and  PaintJet, and IBM 9 and 24 pin  Proprinters,

Quietwriter 2 and 3,  and LaserPrinter.  It will also  display plots

with a CGA, EGA, VGA, enhanced VGA, or HGC and output  a bit map  or

GEM .IMG file.

Even  if your graphics program  supports your printer, you may  find

that PrintGL  gives  better  print quality.  PrintGL uses  the  best

graphics modes available for each printer that  it supports.  If you

need  printed graphics output from personal software, you can output

HP-GL  and use  PrintGL to  do the  printing.  This  gives immediate

support to a wide range of printers.

PrintGL interprets all of the HP 7475 command set that is applicable

to plotfiles except  for character set  8  (Katakana).  None of  the

digitize,  output, or device control commands (except plotter on and

off) are supported as these are only  used with direct control  of a

plotter. And the error mask command is not supported. In addition to

the HP  7475 commands,  PrintGL handles paper feed  commands so that

you can put multiple plots in one plotfile.

PrintGL Menu Interface (PMI) is a menu driven program that lets  you

choose any PrintGL options,  select  a  list  of  plotfiles, and run

PrintGL. This simplifies the selection of PrintGL's many options and

lets you run multiple plotfiles without intervention.  PMI  lets you

save three  configurations,  so  after  initial  setup you  can  run

PrintGL with minimal effort.

g-3

LICENSE AND REGISTRATION

PrintGL is  distributed  as  shareware.  You may use PrintGL without

charge on a trial basis to determine its suitability for you. If you

continue  to use  it  after your  evaluation, a  $25 registration is

requested.  There is no cost to upgrade from any 1.** version to any

other 1.** version.

PrintGL and PrintGL Menu Interface are provided as is.  There are no

warranties expressed or implied.

You may distribute PrintGL if you keep  the entire package together,

unchanged,  including this  license explanation, and  do  not charge

more than $10.

To  register your copy of PrintGL,  send your name, address, and $25

check, and specify PrintGL 1.09, to:

Ravitz Software Inc.

P. O. Box 25068

Lexington, KY  40524-5068

You will  not receive  a new  copy  of PrintGL.  The bulletin  board

distribution  package  is the complete,  full  function  program.  A

receipt will be sent, usually by postcard. Include an additional $10

to get the current version  of  PrintGL (or you may specify the next

significant release) on diskette (3 1/2" unless you specify 5 1/4").

If  you are interested in using PrintGL  as  a printer  driver  in a

commercial  program,  send a note.  Special  license agreements  and

special versions are available for this purpose.

ASSOCIATION OF SHAREWARE PROFESSIONALS

This program is produced by a member of the Association of Shareware

Professionals  (ASP).  ASP  wants  to  make sure  that the shareware

principle  works  for   you.   If  you  are  unable  to  resolve   a

shareware-related  problem  with  an ASP  member  by contacting  the

member directly, ASP may be able to help. The ASP Ombudsman can help

you resolve a  dispute or problem  with an ASP member, but  does not

provide technical support for members' products. Please write to the

ASP Ombudsman  at  P.O.  Box  5786,  Bellevue, WA  98006 or  send  a

Compuserve message via easyplex to ASP Ombudsman 70007,3536

g-4

WHAT IS NEW

For version 1.09:

three configurations are contained within PMI

HP 7475 support

HP-GL reference section in PRINTGL.DOC

GEM .IMG output format

colors may be mixed with any PrintGL shading pattern

For version 1.07:

plotfile masks are supported

improved messages

slightly revised font (*,f,t,j,0)

unsupported HP-GL commands are noted (and then ignored, as before)

/X[S[F]][U] option suppresses messages

PrintGL Menu Interface is included with the PrintGL package

For version 1.05:

DOS environment variable PRINTGL can be used to set defaults

slightly revised command line parser

PrintGL Menu Interface is available to registered PrintGL users

For version 1.04:

/F9* supports NEC 24 pin printers in 360x360 dpi mode

For version 1.02:

/FVP, /FVT replaced by /FV*88, /FV*91

any 720x540 VGA is supported with /FV+modenum

any 800x600 VGA is supported with /FV*modenum

slightly changed line type patterns for improved speed

The latest version of PrintGL is available from the support bulletin

board at  606-268-0577  (1200/2400,N,8,1  24  hours/day,  XMODEM  or

YMODEM (1K XMODEM) download), packed in .ZIP format.

g-5

INSTALLATION

The PrintGL package includes of these files:

PRINTGL.EXE  program

PRINTGL.DOC  documentation

CHARSET.PLT  HP-GL character set plotfile

PMI.EXE      menu interface program

SAMPLE.PLT   sample plotfile

To  install PrintGL copy the distribution files to your working disk

or directory.

PRINTGL.EXE is a stand alone executable  file.  There  are no  extra

printer driver files or overlays.

PRINTGL.DOC is this manual.  You can print it with "COPY PRINTGL.DOC

PRN" (39 pages).  The HP-GL character set should be printed with the

command PRINTGL CHARSET.PLT /M1 /AO0,0 (and a /F  option specific to

your printer) and inserted in the manual.

PMI.EXE is the executable file for PrintGL Menu Interface. PMI needs

to be able to  find  itself  (PMI.EXE)  and  PRINTGL.EXE, so both of

these files should be in a directory that is in your DOS PATH.

SAMPLE.PLT is a sample plotfile to help you try out PrintGL.

g-6

OPERATION

PrintGL reads  a plotfile, does arc, character,  fill, etc.  to line

conversion,  stores the data  in a coordinate  list,  and if needed,

puts the list on disk. It then reorients the data and determines the

plot size.  Next  the  data is  resized and  relocated.  Finally the

coordinate list is rasterized and printed in swaths.  You are likely

to see  the  printer  print  a  swath  and  stop  while  PrintGL  is

processing the next swath.

When you display a plot, press Esc to return to the DOS text screen.

Ctrl-Break  stops  PrintGL at the next break  point.  Ctrl-Break  is

ignored when PrintGL is waiting for user input.

If your  plotfile contains any IW (input window) commands  and there

is  data  outside the clipping window,  then if  PrintGL's automatic

sizing or centering is in effect (/A and /MF options), PrintGL sizes

and centers assuming that you want to see the entire window, whether

is contains data or not.

PrintGL works within a printer's current margins  and  starts at the

current print  position.  If you decline  the form feed suboption of

the /L option  then PrintGL  leaves the  printer at the start of the

line directly below the plot.  All printing is done in printer lines

that cover the full width of the printhead.

Internally PrintGL uses two byte coordinates on a 1016 dots per inch

grid. This allows for plots with coordinates of over 30 inches - far

larger than an HP 7475 can handle.  However if you are using PrintGL

to emulate a large plotter, you may overflow the internal coordinate

system.  This  condition is not  checked, and it will cause  garbage

output.  You can change the internal resolution to 508 dots per inch

(giving 60 inch  maximum plot  sizes)  by editing PRINTGL.EXE with a

binary editor -  search  for the string "indpi"  and change the next

two bytes from hex F803 to hex FC01.

g-7

SYNTAX

The calling syntax of PrintGL is:

PRINTGL plotfile /option1 /option2 ...

Spaces between parameters  are  not required.  The  plotfile  may be

specified with  the /P option if it  is inconvenient to make it  the

first parameter.

If a given option is entered more  than once, only the last entry is

used.  And if  an  option  is invalid  (but  not  incorrect),  it is

ignored.  This  lets  you set  up a  batch  file with your preferred

options  but override them as desired.  For example,  create PHP.BAT

with the line:

PRINTGL /FL /M1 /P %1 %2 %3 %4 %5 %6 %7 %8 %9

Then  PHP can be called just  like PRINTGL  but  it  defaults to  HP

LaserJet format with magnification 1.

The DOS  environment variable PRINTGL is examined  by PrintGL and if

it  is  nonblank  then  it  is  put  in front  of  the  command line

parameters.  If  you put "SET PRINTGL=/FL/M1"  in your  AUTOEXEC.BAT

then PrintGL will default to Laserjet format with magnification 1.

If you do not enter any parameters or if any parameter is incorrect,

PrintGL displays its option reminder screen.

Below  [ and ] enclose optional  suboptions and � separates mutually

exclusive suboptions. The options and suboptions may be in any order

except that multiple numeric suboptions are ordered. "c" refers to a

suboption  character,  "x"  and  "y"  to  suboption  floating  point

numbers, "i" and "j" to suboption integers, "b" to a suboption byte,

and "f" to a DOS file or device name.

For  floating  point numbers a  decimal  point is  allowed  but  not

required, and scientific notation is not allowed. "-" is allowed but

not  "+".  For integer input, only base ten integers from 0 to 32767

are  allowed.  A  byte must be a base ten integer from 0 to 255 or a

hexidecimal number prefixed  with $ from $00  to $FF.  Further range

checking is done for most options to flag unreasonable input.  If an

option has multiple numeric  suboptions, these may be separated with

commas or blanks.

********************************************************************

It  is very important to choose the right output format (/F option).

It is likely  that  the  default  output  format will  work on  your

printer, but it is equally likely  that it  will not produce optimal

results.  Please look carefully at the entire list of output formats

before  choosing one, and do not assume that because a  format works

that it is the best one to use.

********************************************************************

g-8

/A - LOCATION OPTION

/A[O][x,y] - location - default /A

The A option determines  the  location of the plot  in  the  printer

window, by specifying a point on the plot that is to be aligned with

a point in the printer window. The point on the plot is normally its

center,  but  you can specify  a point with x,y  in  inches from the

plot's  origin.  The  point  in  the printer window is  normally the

center, but O says to make it the origin.  The  default /A  puts the

center of the plot at the center of the printer window.

When PrintGL is  finding the center of a plot, the  entire rectangle

defined  by any IW command  is included  in the overall  size of the

plot.

/A      puts the center of the plot at the center of the print

/AO0,0  puts the plot 0,0 at the print origin

/C - PEN COLOR OPTION

/Cc.. - color - default /CL

The C  option specifies  the color of each  of the eight  pens.  The

colors are  B  for blue, C for cyan, G for green, L for black, M for

magenta, R for red, and Y for yellow.  Unspecified pens use the last

specified color  (/CRGB is the  same as /CRGBBBBBB).  If  the output

format does not allow color then this option is ignored.

Additional colors can be created with shade mixing. These colors use

the pen's specified  shading pattern  (/S option)  to mix  the seven

pure colors.  If the shade value is 0  (solid)  it  is replaced by 2

(one pel checkerboard) which produces the most uniform mixed colors.

The colors may not look good with multipass dot matrix  formats, and

they need line widths  of at least 2 (4 for 2 pel shading patterns).

Mixed colors may  have  visual artifacts on edges  near  45  degrees

angle. The colors are:

.bc  blue cyan         .lb  black blue          .wb  white blue

.bm  blue magenta      .lc  black cyan          .wc  white cyan

.gc  green cyan        .lg  black green         .wg  white green

.gy  green yellow      .lm  black magenta       .wm  white magenta

.rm  red magenta       .lr  black red           .wr  white red

.ry  red yellow        .ly  black yellow        .wy  white yellow

.wl  white black

The following output  formats handle colors:  /F9 (NEC 24  pin), /FN

(Epson 9 pin), /FP (HP PaintJet), /FT (Epson 24 pin), /FB (bit map),

/FG (GEM .IMG),  /FE  (EGA), and /FV (VGA).  Shading may  be used to

emulate color on black only printers (see /S option).

/CL          sets pen 1..8 to black

/CLRG.BM.LY  sets pen 1    to black

pen 2    to red

pen 3    to green

pen 4    to blue/magenta checkerboard

pen 5..8 to black/yellow checkerboard

g-9

/D - OUTPUT DESTINATION OPTION

/Df - destination - default /D1

The D option specifies the output printer port, device, or file.  1,

2, or  3 causes PrintGL  to use the BIOS printer routines, bypassing

DOS.  This is  normally faster than specifying the DOS device (LPT1,

LPT2, LPT3), but  it  may  cause  problems with  print  spoolers  or

multitasking systems. Use COM1, COM2, etc. for serial printers.

Display output  can only go to the screen and causes this  option to

be ignored.

/D2     sends output to the second parallel port via BIOS

/DLPT3  sends output to the DOS device LPT3

g-10

/F - OUTPUT FORMAT OPTION

/Fc[+[b]�-�*[b]�=�i,j] - output format - default /FN

The  F option  specifies which output format to use.  The  format is

chosen  with  a  device  character  and,  optionally,  a  resolution

modifier (-, +, *)  or alternate modifier  (=), and in some  cases a

display mode.  *, +, and  -  are  allowed even where  they  have  no

affect.  For  bit maps you can choose a specific  resolution in dots

per  inch.  Resolutions here  are  given  in horizontal  x  vertical

dots/inch.

To return to DOS from a displayed plot, press Esc.

/F1     IBM Proprinter ..................................... 120x72

/F1+    IBM Proprinter (2 pass)                              120x144

/F2     IBM Quietwriter 2 .................................. 240x240

/F3     IBM Quietwriter 3                                    240x240

/F4-    IBM LaserPrinter ................................... 150x150

/F4     IBM LaserPrinter                                     300x300

/F4=    IBM LaserPrinter ................................... 300x300

/F5     IBM Proprinter X24 compatible printers               180x180

/F5+    IBM Proprinter X24 compatible printers (2 pass) .... 360x180

/F6     IBM Proprinter X24                                   180x182

/F6+    IBM Proprinter X24 (2 pass) ........................ 360x182

/F9     NEC 24 pin compatible printers                       180x180

/F9+    NEC 24 pin compatible printers (2 pass) ............ 360x180

/F9*    NEC 24 pin compatible printers (4 pass)              360x360

/FB     bit map ............................................ 100x100

/FBi,j  bit map                                                ixj

/FC     CGA display ......................................... 62x25

/FE     128K EGA color display                                62x45

/FG     GEM .IMG bit map ................................... 100x100

/FGi,j  GEM .IMG bit map                                       ixj

/FH     Hercules graphics card display ...................... 70x45

/FL-    HP LaserJet compatible printers                      150x150

/FL     HP LaserJet compatible printers .................... 300x300

/FL=    HP LaserJet compatible page printers                 300x300

/FM     128K EGA monochrome display ......................... 62x45

/FN     Epson 9 pin compatible printers                      120x72

/FN+    Epson 9 pin compatible printers (3 pass) ........... 120x216

/FN*    Epson 9 pin compatible printers (6 pass)             240x216

/FP     HP PaintJet ........................................ 180x180

/FT     Epson 24 pin compatible printers                     180x180

/FT+    Epson 24 pin compatible printers (2 pass) .......... 360x180

/FT*    Epson 24 pin compatible printers (4 pass)            360x360

/FV     VGA display ......................................... 62x62

/FV+    enhanced VGA display 720x540 mode - mode $63          70x70

/FV+b   enhanced VGA display 720x540 mode - mode b .......... 70x70

/FV*    enhanced VGA display 800x600 mode - mode $64          78x78

/FV*b   enhanced VGA display 800x600 mode - mode b .......... 78x78

The display mode  with /FV+ and /FV*  may  be  specified  in  hex by

prefixing it with $, for example /FV*$64 is the same as /FV*100.

g-11

/I - SCALING POINT LOCATION OPTION

/IA�R�W[N] - IP point location - default /IA

The I option  sets the initial scaling points  (P1, P2 - the scaling

points may  be  set  in the  HP-GL  file  with  the IP  command).  A

(absolute) sets  the points to the default  for  the paper size (see

below).  R fits P1 and P2 to the plot window with at least .25  inch

margins and an x/y ratio of  10/7.2  (the same ratio as the  HP 7475

with small paper).  W fits P1 and P2 to the plot  window with  a .25

inch margin. N makes the margin 0 for R or W and is ignored with A.

The default scaling points depend on the paper size  (/L option) and

the plot orientation (/O option).

paper  options              default scaling pts  with RO90 command

A      /O1 /L 7.84,10.20    250,596 10250, 7796  154,244  7354,10244

A4     /O1 /L 7.60,10.87    603,521 10603, 7721    0,610  7200,10610

B      /O1 /L10.20,16.38    522,259 15722,10259  283,934 10283,16134

A3     /O1 /L10.87,15.90    170,602 15370,10602  607,797 10607,15997

A      /O2 /L10.20, 7.84    250,596 10250, 7796  154,244  7354,10244

A4     /O2 /L10.87, 7.60    603,521 10603, 7721    0,610  7200,10610

B      /O2 /L16.38,10.20    522,259 15722,10259  283,934 10283,16134

A3     /O2 /L15.90,10.87    170,602 15370,10602  607,797 10607,15997

anything else               250,279 10250,7479   279,250  7479,10250

/O3 and /O4 gives the same default scaling points as /O1 and /O2.

The default paper size is 7.53 by 10.14, the HP 7470 standard  paper

size.  This causes "anything else" values.  This is because a number

of  printers  can't handle the slightly larger HP 7475 A size plots.

You must use the /L option to get HP 7475 standard paper sizes.

/IA   uses the default default scaling points

/IWN  sets the scaling points to fit the print window

g-12

/L - PAGE LAYOUT OPTION

/L[B][F][R][x,y[,x,y]] - page layout - default depends on printer

The L option defines the page layout.

B boxes the plot at the hard clip limits using pen 8. This generally

increases the amount of data  sent to a printer, but not bit maps or

displays.

F  causes a form feed after the plot.  This  is ignored for bit maps

and display output.

R causes reverse  image (black on white)  on one color display plots

and bit maps and is otherwise ignored.

The first optional x and  y are the  plot width and height in inches

rounded to .01.  These define the hard clip limits. These values are

not checked against your printer's capabilities.  If  you specify  a

page  layout larger than the  output  device can handle, the results

are unknown.  You may need to reset your  printer's margins, maximum

line length, or page length to get large plots.  The size needed for

a given width and height exceeds x and y by 8 pels to allow for line

widths.

The second optional x and y  are the  left and top margins, measured

from the printer's left margin  and current vertical position to the

hard  clip limits.  The left margin is rounded to .01 inch.  The top

margin  is rounded to a 1/6 inch and line feeds (assumed  to  be 1/6

inch) are used to  position the paper.  The margins  are  subject to

printer  resolution  and  efficiency considerations, and may  not be

exact. Margins are ignored for display output and bit maps.

Each printer has a default page layout. The default width and height

are  7.53x10.14  for  printers  and  bit  maps  and  10.14x7.53  for

displays.  These  correspond  to  HP  7470  A  size paper,  which is

slightly smaller than  HP 7475 A  size paper.  This  is  the default

because some printers can't handle the extra .06 inch  length of the

HP 7475 A size.  The margins are printer  specific to compensate for

different  default  margins.  In  evaluating user  options,  PrintGL

processes each option and when finished reprocesses the default page

layout  for  the last chosen  printer and then reprocesses  the last

user /L option.  This procedure  gives  good default values  so that

your /L option can  change the letter or  size suboptions  and still

have good margins.

/LB7.5,3,.5,0   makes the  print window 7.5 inches wide and 3 inches

high,  with a .5 inch left margin and no top margin,

boxes the window, and does not do a form feed

/LF7.53,10.14   sets HP 7470 A size paper

/LF7.53,10.73   sets HP 7470 A4 size paper

/LF7.84,10.20   sets HP 7475 A size paper

/LF7.60,10.87   sets HP 7475 A4 size paper

/LF10.20,16.38  sets HP 7475 B size paper

/LF10.87,15.90  sets HP 7475 A3 size paper

g-13

/M - MAGNIFICATION OPTION

/M[F]x - magnification - default /MF.9

The M option  specifies  the magnification.  F fits the plot  to the

printer  window size.  The default is /MF.9 which makes the plot 90%

of the largest  size that will fit in the printer window.  /M1 gives

the same size plot as an HP 7475 plotter.

/MF.9  fits the plot to the print window and shrinks it 10%

/M1    plots at the same size as an HP 7475

/M.5   plots at half the size of an HP 7475

/N - PAGE NUMBER OPTION

/N[i] - page number - default /N

Ni says to print only page i of the plotfile.  If you do not specify

i then all pages in the plot will be printed (this is the default).

/N   prints all pages in the plot file

/N3  prints only page 3

/O - ORIGIN AND ORIENTATION OPTION

/O1�2�3�4[L] - origin, orientation - default /O1 or /O2

The O options sets the printer's origin and orientation.  The origin

is  one of four corners numbered  1..4 for upper  left, lower  left,

lower  right,  and upper  right.  The  L suboption  specifies a left

handed coordinate  system (this should never be  needed).  1  is the

default  origin for  printers and  matches the  plotter's coordinate

system.  2 is the default origin for displays which also matches the

plotter's coordinates due to the reversed width/height ratio.

Here are pictures of PrintGL's view of  the output media compared to

an HP 7475's view, to help with choosing a /O option.

HP 7475 A        HP 7475 B          PrintGL

y��������Ŀ    0,0��������Ŀy     ul ������Ŀur   ul��������Ŀur

�        �       �        �         �paper �       �screen  �

�        �       �        �         �      �       �        �

0,0����������x      �        �         �      �     ll����������lr

�        �         �      �

�        �      ll ��������lr

�        �

x����������

/O1  puts the print window's origin in the upper left corner

/O2  puts the print window's origin in the lower left corner

g-14

/P - PLOTFILE OPTION

/Pf - plotfile - no default

The plotfile may  be specified as the first  parameter  without  any

prefix,  or it may be specified anywhere in the option list with the

/P prefix.  The plotfile may be a standard DOS mask with "*" and "?"

characters.  In this case each matching file is  processed, but  the

first failure or user break stops all processing.

/S - PEN SHADING OPTION

/Sc.. - pen shading patterns - default /S0

This option assigns a shading pattern to each pen.  Unspecified pens

use the last specified shade.  The shade values are  0..8.  0 is the

default solid line.  1..4 are 1 dot patterns (75%, 50%,  25%, 12.5%)

and need line widths of  at least  three to give good looking lines.

5..8 are  2x2  dot patterns (75%, 50%,  25%,  12.5%)  and need  line

widths  of at least five to give  good looking lines.  Some printers

may  wash  out shading because they  have a dot size that  is  large

relative to the dot spacing.  This is often the case with dot matrix

printers in multipass modes.

1                   2                   3                   4

��������            ��������            � � � �             �   �

��������            ��������            � � � �               �   �

��������            ��������            � � � �             �   �

��������            ��������            � � � �               �   �

5                   6                   7                   8

��������            ��  ��              ��  ��              ��

��  ��                ��  ��

��������            ��  ��              ��  ��                  ��

��  ��                ��  ��

/S0236  sets pen 1    to solid

pen 2    to 1 pel checkerboard

pen 3    to 1 pel dots

pen 4..8 to 2 pel checkerboard

/T - TEMPORARY FILE OPTION

/Tf - temporary file - default /TPRINTGL.TMP

PrintGL uses a temporary file when it does not have enough memory to

handle  a plotfile.  It does  not use extended or expanded memory so

you can best utilize this memory by putting the  temporary file on a

RAM disk in the extra memory. If you have 640K of memory and average

size  plotfiles  then  it is likely  that  PrintGL will never use  a

temporary file.

g-15

/W - PEN WIDTH OPTION

/Wc.. - pen widths - default /WA

The W option assigns a line width to each pen (1..8). Each width may

be 1..8 pels or A..H for the default width of the output format plus

0..7 (maximum 8). Unassigned pens use the last assigned width.

The default line  width  is 1 for all output formats except  240 and

300 dpi printers which have a default line width of 2.

/WA       sets pen 1..8 to the default width for the chosen printer

/WABC123  sets pen 1    to the default width for the chosen printer

pen 2    to the default width + 1 pel

pen 3    to the default width + 2 pel

pen 4    to 1 pel

pen 5    to 2 pels

pen 6..8 to 3 pels

/X - SUPPRESS MESSAGES

/X[S[F]][U] - suppress messages - default /X

The  X option  lets  you  suppress  some  PrintGL  messages.  The  S

suboption suppresses location and size messages. With S you can also

specify   F,   which  suppresses  file  names   and  page   numbers,

substituting "Plot  1", "Plot  2", etc.  The  U suboption suppresses

unsupported command messages.

/X   gives all the normal messages

/XS  gives only the most important messages

/Y - Y/D SWITCH SETTING OPTION

/YD�Y - Y/D switch setting - default /YD

The Y option sets the plotter's rear panel Y/D switch to D  or Y.  Y

starts with the plotter off and responds to esc.(, esc.Y, esc.), and

esc.Z  commands.  D  starts with  the plotter  on and  ignores these

commands.

/YD  puts the YD switch in the usual D position

/Z - CHORD ANGLE OPTION

/Zi,j - default and minimum chord angle - default /Z15,5

The HP 7470 defaults  to a  chord  angle of  5  and has  no minimum.

Because  PrintGL stores each  chord  in a  circle  in  its  internal

database, this can be  very  time consuming.  And because of the low

resolution   of  printers  it  does   not  produce  a  corresponding

improvement in print quality. The Z option specifies the default and

minimum  chord angles.  The default is /Z15,5.  For better emulation

use /Z5,2.

/Z5,2  sets the default chord angle to 5 degrees, and the minimum to

2 degrees

g-16

MESSAGES

PrintGL  displays a number of messages to  keep you informed of what

it is doing. The output looks like this.

Reading SAMPLE.PLT

Plotfile has 1 page(s)    Processing page 1    2283 data records

Overall magnification    0.9964

Print window   Horizontal   0.00   7.53    Vertical   0.00  10.14

Plot area      Horizontal   0.42   7.13    Vertical   0.61   9.54

Output format /FN    120x72 dots/inch (HxV)

Writing to device LPT3#

The first line  is the plotfile  that is  currently being processed.

The second line tells how many pages are  in the plofile, which page

is being processed, and the number of data points in that page.  The

number  of data points is the number of pen moves in the page and is

shown to give you an indication of the plot's complexity.

The  overall magnification  is either  the  magnification  that  you

specified  with the  /M option or  the effective magnification after

fitting to  the  page  with  /MF.  The  print window line  gives the

horizontal  and vertical  coordinates of  the  print window,  always

starting at 0,0 and in inches.  The  plot area gives  the horizontal

and vertical coordinates of a box that just covers the plotted data.

This uses the same coordinate system as the print window, so you can

tell where the plot is relative to the print window.

The  output format  and resolution  are  given and  then the  output

device is  noted (except for screen plots).  If three plane color is

being used, the word "color" is displayed.  If you have a black only

printer, but the output format allows  color (this is  common with 9

and 24 pin  printers), using color  will triple the PrintGL run time

with no improvement in print quality.

During  processing, PrintGL informs  you when  it is  processing and

writing data,  and you can watch the progress from 0% to 100%, along

with the elapsed time.

SERIAL PRINTER INTERFACE

If your  printer  is  attached  to  a  serial port, specify  /DCOM1,

/DCOM2, etc. PrintGL cannot set up the port. This is usually done in

the AUTOEXEC.BAT with MODE (probably MODE COM1:9600,N,8,1,P).

At 9600 baud, the normal printer interface speed, a serial interface

is  far  slower  than a parallel port.  A full  page  of 180x180 dpi

graphics takes about 5  minutes.  And if you are  using color, which

requires sending  three independant color planes, this  jumps  to 15

minutes.

g-17

EPSON 9 PIN PRINTERS

/FN covers a wide range  of  9  pin dot matrix printers.  You need a

cyan/magenta/yellow ribbon  to  get  color  prints.  /FN+ uses 1/216

indexing and /FN* uses 1/216 indexing and 1/240 graphics.

The  IBM  9 pin  Proprinters  do not  do  1/216 indexing and  do not

correctly handle 1/240 graphics. /FP is a special Proprinter driver.

The /FN drivers use the following printer escape codes:

esc J - index

esc L - graphics command for /FN and /FN+

esc Z - graphics command for /FN*

esc r - set ribbon color (only if color is specified)

EPSON AND NEC 24 PIN PRINTERS

/FT covers the entire line of Epson LQ printers  and many compatible

printers (Panasonic, etc.). You need a cyan/magenta/yellow ribbon to

get color prints.  /FT+  uses  1/360 graphics mode.  /FT* uses 1/360

graphics mode and 1/360 indexing  and resets the printer's line feed

distance  to 1/6 inch on completion.  A clean paper path  with equal

tension on both sides is needed for good 360x360 graphics.

/F9  and  /F9+ are  identical  to  /FT  and  /FT+.  /F9* is the  NEC

equivalent to /FT*. The above notes apply to the /F9 drivers also.

These  drivers will not  work with IBM  Proprinter X24s in alternate

graphics mode because they use the absolute tab command.

The /FT and /F9 drivers use the following printer escape codes:

esc $   - absolute tab

esc * ' - graphics command for /FT, /F9

esc * ( - graphics command for /FT+, /FT*, /F9+, /F9*

esc +   - set n/360 indexing for /FT*

esc 2   - start esc A indexing for /FT*, /F9*

esc A   - set n/72 indexing for /FT*, /F9*

esc J   - index

esc r   - set ribbon color (only if color is specified)

fs  3   - set n/360 indexing for /F9*

g-18

HP LASERJET

/FL works with  any LaserJet or  DeskJet.  This driver assumes  that

your printer is in portrait mode.

At  300 dpi (the default), if you have just 512K  in a LaserJet then

your  plot can  cover about  half  the area of  the  page.  1.5M  is

sufficient for  a  full page of graphics.  At  150  dpi (/FL-)  512K

handles  a full  page.  (All  DeskJets  can  handle a full  page  of

graphics at 300 dpi.)

/FL- defaults to a page  height of 10.11 inch instead of 10.14 inch,

so that plots fit on a page without changing margins.

/FL= is the same as  /FL  except that it reduces the amount  of data

sent to the printer at the expense of processing time.  This may run

faster than /FL and may allow larger plots on 512K printers. It only

works on page printers - not the DeskJet.

HP PAINTJET

/FP should work with  any 180  dpi HP-PCL  printer, including the HP

Rugged Writer, but this has not been tested.  If your PaintJet has a

serial interface, please see SERIAL PRINTER INTERFACE.

g-19

IBM LASERPRINTER

/F6 drives the  LaserPrinter  in  native mode.  /FL will work if the

LaserPrinter is in HP LaserJet mode.

At 300 dpi (the default), if you have just 512K in your LaserPrinter

then your plot can cover about half of the page.  1.5M is sufficient

for a full page of graphics.  At 150 dpi (/F6-) 512K  handles a full

page.

/F4= is the  same as /F4 except that it reduces  the  amount of data

sent to the printer at the expense of processing time.  This may run

faster than /F4 and may allow larger plots on 512K printers.

IBM PROPRINTER

The /FP driver  is  for all 9 pin  Proprinters.  The high resolution

driver (/FP+) uses  1/144  inch indexing  and it does  not work with

Epson 9 pin compatible printers that have 1/216 inch indexing.

There  is no  /FP*  resolution,  which  would  be  240x144,  because

Proprinters do not handle 1/240 graphics correctly.

IBM PROPRINTER X24

The /F5 driver covers all Proprinter  X24s.  These printers  can use

the X24 compatible driver (/F4), but because they do 1/144 indexing,

the print will have horizontal white streaks every 2/3 inch. The /F5

drivers  compensate  for  the  indexing  by  defining  the  vertical

resolution as 182 dots per inch  and  indexing 19/144 inch per line.

To work properly the printer  must be  on a  1/144 boundary when the

graphics are printed.  If you keep the line feed distance a multiple

of 1/144 (1/6, 1/8, 1/9) then this will always be the case.

Proprinter X24 and  XL24  are very slow with  graphics,  making many

passes per line.  This  limitation cannot be overcome with software.

The  X24E and XL24E are significantly faster, and are much preferred

to the original X24s.

IBM PROPRINTER X24 COMPATIBLE PRINTERS, IBM QUICKWRITER

The IBM Quickwriter is  a  Proprinter X24 compatible printer  except

that  it does 1/180 indexing.  This is  probably the  case with most

Proprinter X24 compatible printers. /F4 is the correct driver to use

for  such  printers.   /F5  (the  Proprinter  X24   driver)  is  not

recommended.

IBM QUIETWRITER 2 AND 3

Line  widths  should  be  at  least  two  for  these  printers.  The

Quietwriter  drivers reset the line feed  distance to 1/6 inch  upon

completion.

g-20

CGA DISPLAY

The  CGA  driver (/FC)  does not give  color.  The  CGA card has  no

graphics mode that supports the 7 colors used by PrintGL.

EGA DISPLAY

The EGA drivers (/FE, /FM) require a 128K or 256K EGA. If you have a

64K EGA, the CGA (/FC) driver will work.

/FM is for an EGA card with  a digital monochrome  monitor attached.

If  your display  adaptor supports  Hercules  modes, /FH  will  give

better resolution.

HERCULES DISPLAY

/FH supports the  Hercules  monochrome graphics card.  There  is  no

support for the Hercules InColor card.

MCGA DISPLAY

/FV will work with an MCGA if you do not use color.

VGA DISPLAY

The /FV+ output  format requires  a VGA  card that  supports 720x540

mode.  /FV* requires a VGA  card and multisync  monitor that support

800x600  mode.  PrintGL sets up the screen by setting  BIOS  display

modes $63 and $64 ($ means hex).  Your VGA card must use these modes

to work with /FV+ and /FV*.

If  your  VGA  card  uses other  mode numbers to select  720x540  or

800x600 mode, you can specify this number in the /F option.  Here is

a list of /FV options for common  VGA cards.  You may need to  run a

special driver that is supplied with the card to get these modes.

ATI                     /FV*$54

Genoa                   /FV*$29

Paradise                /FV*$58

Renaissance   /FV+$63   /FV*$64

Trident                 /FV*$5B

Video Seven             /FV*$62

g-21

BIT MAPS

Color bit maps are two pels per byte.  Each  pel nibble  = 1*blue  +

2*green + 4*red +  8*intensity.  The  intensity is 0 for black and 1

otherwise.  The  background is  15  - white,  which  is  not  a user

selectable color.

/CL  (default) bit  maps are 8  pels/byte with 1 meaning black and 0

background.  This may  be  reversed  with  the R suboption of the /L

option.

The  map size is the size specified  by the L  option plus 8 pels in

each direction to give room for line widths of up to 8 pels.

GEM .IMG BIT MAPS

Here is the .IMG format  used by  PrintGL.  It uses only a subset of

the available commands.  The file starts with a header composed of 8

word values (high byte first):

1  1

2  8

3  number of color planes - 1 or 3

4  1

5  pel width in microns - round(25400 / horizontal dots/inch)

6  pel height in microns - round(25400 / vertical dots/inch)

7  scan line width in pels (always a multiple of 8)

8  number of scan lines

The rest of the file has the scan lines in order from top to bottom.

For  color  plots,  each complete scan line consists  of  a red scan

line, a green scan line, and  then a blue scan line.  Each scan line

is composed of these data commands:

$01..$7F - from 1 to 127 * 8 pels off

$81..$8F - from 1 to 127 * 8 pels on

$80 n b1 b2 .. bn - n bytes of bit mapped data

Single plane files use 0 for the background. Three plane files use 1

for the background (red + green + blue = white).

The  map size is the size specified  by the L  option plus 8 pels in

each direction to give room for line widths of up to 8 pels.

g-22

PRINTGL MENU INTERFACE (PMI)

To  run  PrintGL  Menu Interface,  enter PMI  at the DOS prompt, and

press Enter. There are no options. The PMI main menu is displayed at

the  top  of the  screen,  and  the  bottom  of the screen  is  used

differently for each main menu option.  The PMI main menu is divided

into  six  groups  -  the top row  and five columns.  Each group  is

described below.

When the cursor is on a main  menu item,  the  current value of  the

corresponding option is diplayed.  To modify that value, press Enter

(or  the left  mouse button).  The  options are modified with cursor

movement  and sometimes  entering  numeric  values.  The  Enter  key

returns you to the main menu with the new option values. Esc (or the

right  mouse button) returns you  to the main menu  with  the option

values unchanged.

Whenever a cursor  bar has a notch, then the  value within the notch

may be edited.

PMI SPECIFIC FUNCTIONS

The top row of  main  menu  options is  dedicated to PMI  functions.

These  include  running PrintGL,  switching  to  alternate settings,

exiting  PMI, saving  the  current  PMI  settings, and  setting  PMI

options.

R  runs  PrintGL from  any main  menu cursor  location, but  PrintGL

messages will not be saved unless the cursor is on "run PrintGL".

The "alt settings"  item rotates  between three  PMI configurations.

The current configuration number is on line 2 column 1.

The "save settings" item saves the current  PMI configurations.  All

of the current PrintGL  and PMI options for  each configguration are

saved.  The plotfile list is not saved.  PMI saves its configuration

by writing to PMI.EXE.  To find  PMI.EXE  it looks  in  the  current

directory and  then  searches  the DOS  PATH,  saving  to the  first

PMI.EXE that it finds.

The  "menu options"  item lets you configure  PMI mouse sensitivity,

screen colors, and up to five initial plotfile masks.  Screen colors

are specified with the standard IBM BIOS attributes.

For color displays, the left hex digit is the background color and

the right hex digit is the foreground color. Bright colors are not

available in the background.

0 black      4 red          8 gray (bright)     C bright red

1 blue       5 magenta      9 bright blue       D bright magenta

2 green      6 brown        A bright green      E yellow

3 cyan       7 white        B bright cyan       F bright white

For digital monochrome displays, these attributes are useful.

$01 underscored     $09 bright underscored     $70 reverse image

$07 normal          $0F bright

g-23

PMI INPUT AND OUTPUT CHOICES

Column  one of the main menu lets you choose the  plotfiles  to plot

(the /P option  in PrintGL), the plot page  number (/N),  the output

destination (/D), and the output format (/F).

You may enter up to 99 plotfiles in the "plotfile" main menu option.

To  choose from a  list of files, enter a  mask name and press Enter

(the cursor must  be on the  mask).  PMI will then display a list of

corresponding  files.  Select  any number of the files by moving the

cursor  to the desired file and pressing Space  (or the middle mouse

button). If no files have been selected, then pressing Enter selects

the file under the cursor.  The selected files are added to the file

list at  or below  the  cursor.  Files that would overflow the  99th

position on the plotfile list are dropped.

When you run PrintGL, PMI runs PrintGL for each file on the plotfile

list.  Masks are ignored.  For any  plotfiles that PrintGL does  not

successfully process, PMI inserts an arrow in front of the  plotfile

name.  You  can view the results from the "plotfile" main menu item.

The arrow is not considered  part of the file name by PMI, so you do

not need to remove it to retry PrintGL.

Printers in the output format list  that support  color have a color

option.  This lets you specify a black/white printer even though you

may have  set up colors with the "pen color"  menu.  Output  formats

that do not  support color do not have this option and always ignore

the selected colors.

PMI PLOT OPTIONS

The second column  of main menu  options  define how  the  plot will

appear  on  the  page.   The  options  include  magnification  (/M),

orientation (/O), and position (/A).

PMI PEN OPTIONS

The   third   column  of   main  menu   options   covers   the   pen

characteristics:  color  (/C), shade (/S), and width (/W).  Each  of

these menus use the up and down cursor keys to select a pen, and the

left and right cursor keys to choose an attribute.

For  color  selection,  the  one  character  colors  (l,b,c,g,m,r,y)

represent pure colors, and the two character colors are checkerboard

mixes.  The checkerboard mixed colors disable any shading option for

that pen.

PMI PAGE LAYOUT OPTIONS

The fourth column of main menu options covers  the page layout (/L).

These are box, form  feed, reverse image, print window, and left and

top margins.

g-24

PMI OTHER OPTIONS

Options  that are generally set  once and then never touched  are in

column  5.  These include  the temporary file name (/T), default and

minimum chord angle  (/Z), plotter scaling  points (/I), and plotter

D/Y switch setting (/Y).

g-25

HP-GL

PrintGL's HP-GL specification is taken from the HP 7475A Interfacing

and Progamming Manual, HP part number 07475-90001.  PrintGL supports

this HP-GL  subset except for  character set  8 (Katakana), digitize

commands, output commands,  the error mask  command, and some device

control commands. Supported commands are listed below. Commands that

are not applicable to printing, such as pen velocity, are considered

supported  but  are  completely  ignored.   Commands  that  are  not

supported are flagged and then ignored.

Some extensions are supported. Form feed commands are allowed and 15

pens are allowed instead of 6. To take advantage of the this, it may

be useful  to  use  a 7550 driver to create your plotfile.  And fill

angles (FT command) are not restricted to multiples of 45.

In reading a plotfile, PrintGL ignores carriage returns, line feeds,

and nulls  except where  they may be valid characters (LB,  SM,  DT,

esc.), and ASCII EOF (dec 26) is interpreted as an end of file mark.

AA x_center , y_center , arc_angle [, chord_angle]

arc absolute

AA draws an arc starting at the current pen position  with the pen

in the current up/down state.

AF

advance full page (extension)

AF starts a new plot.

AP

automatic pen pickup - IGNORED

AR x_center_delta , y_center_delta , arc_angle [, chord_angle]

arc relative

AA draws an arc starting at the current pen position  with the pen

in the current up/down state.

CA character_set

designate alternate character set

See HP-GL CHARACTER SETS for a list of character sets.

CI radius [, chord_angle]

circle

CI draws a  circle centered at the current position.  The  current

up/down state and position are unchanged by a CI command.

g-26

CP [spaces , lines]

character plot

CP  moves the pen by  character spaces (+ is along the  text)  and

lines  (+  is  up), using  the  current  up/down state.  Using  no

parameters is the same as including a carriage return/line feed in

an LB command.

CS character_set

designate standard character set

See HP-GL CHARACTER SETS for a list of character sets.

DF

default values

This sets default values for the following commands:  CA,  CS, DR,

DT, FT, IW, LT, PA, PT, SC, SM, SL, SR, SS, TL.

DI [run , rise]

absolute character direction

DI sets the baseline direction for labels.  Using no parameters is

the same as DI1,0.

DR [run , rise]

relative character direction

DI  sets the baseline  direction  for labels.  Run  and rise are a

percentage of P2X-P1X and P2Y-P1Y. Using no parameters is the same

as DR1,0.

DTt

define label terminator

The label  terminator denotes  the end  of each  LB  command.  The

default is ASCII 003.

EA x_opposite_corner , y_opposite_corner

edge rectangle absolute

EA draws a rectangle with corners defined  by the current position

and the command  parameters.  The current pen position and up/down

state are unchanged.

EC

enable cutter - IGNORED

g-27

ER x_opposite_corner_delta , y_opposite_corner_delta

edge rectangle relative

ER draws a rectangle with corners defined  by the current position

and the command  parameters.  The current pen position and up/down

state are unchanged.

EW radius , start_angle , sweep_angle [, chord_angle]

edge wedge

EW draws a wedge centered at the current position. The current pen

position and up/down state are unchanged.

FT [type [, spacing [, angle]]]

fill type

These are  the  fill types.  Type  2  is identical to  type 1  for

PrintGL.

1  solid lines spaced at the pen thickness (see PT command)

2  unidirectional solid lines spaced at the pen thickness

3  lines at the specified spacing

4  cross hatched lines at the specified spacing

5  type ignored

A spacing parameter of 0 sets  the spacing  to  1%  of  the  P1-P2

diagonal. Spacing is in x axis units (this is only relevant if the

axes have differnet  scaling).  PrintGL supports any integer angle

for  fill  (the HP 7475 requires multiples of 45  degrees).  Cross

hatched lines are at angle + 90.

IN

initialize

IP [p1x , p1y [, p2x , p2y]]

sets scaling points P1 and P2

IP sets the scaling points (P1 and P2) in plotter units. P1 and P2

are used  by a number  of  commands  to help with size independant

plots.

IW [x_lower_left , y_lower_left , x_upper_right , y_upper_right]

input window

IW sets a  clipping window in plotter units.  With  no parameters,

the  clipping window is turned off.  When a clipping  window is in

effect  and  there  is  data  outside  the  window,  if  PrintGL's

automatic  sizing or centering is  in effect,  PrintGL  sizes  and

centers assuming  that you  want to see the entire window, whether

is contains data or not.

g-28

LB...text...terminator

label

Labels use  the current character set, based on CA, CS, SA, and SS

commands. See HP-GL CHARACTER SETS for font specifics.

LT [pattern_number [, pattern_length]

line type

PrintGL's line type  patterns are close to, but not  identical  to

HP's.  Line  type patterns  are dynamically  limited to a  minimum

length of 16 pels.

0            at endpoints only

1            �       �       �       �       �

2            ����    ����    ����    ����    ����

3            ������  ������  ������  ������  ������

4            ����� � ����� � ����� � ����� � ����� �

5            ���� �� ���� �� ���� �� ���� �� ���� ��

6            ��� � � ��� � � ��� � � ��� � � ��� � �

no parameter ����������������������������������������

Pattern  length  is a  percentage  of the  P1-P2  diagonal with  a

default of 4%.

Line  type is  a property  of the line that is being drawn - it is

not a property of the pen that is being used to draw the line. You

cannot specify that a certain pen should use a specific line type.

(Color, shading, and width are pen  properties and can be assigned

to pens.)

PA [x , y] [, x , y] ...

plot absolute

PA moves the pen in current up/down state.

PD [x , y] [, x , y] ...

pen down

PD sets  the pen  down and optionally  moves  it with  the current

absolute/relative mode.

PG [i]

new page (extension)

PG starts a new plot. The parameter is ignored.

PR [x_delta , y_delta] [, x_delta , y_delta] ...

plot relative

PR moves the pen with the current up/down state.

g-29

PS

paper size - IGNORED

PrintGL ignores this command. You must set the paper size with the

/L option.

PT [thickness]

pen thickness

The pen thickness  is  the line spacing used in  solid fills.  (It

does not cause a pen to be a certain width.)

PU [x,y] [,x,y] ...

pen up

PU  raises  the  pen  and  optionally  moves it with  the  current

absolute/relative mode.

RA x_opposite_corner , y_opposite_corner

shade rectangle absolute

RA  fills a rectangle with corners defined by the current position

and the command parameters.  The current pen  position and up/down

state are unchanged. The rectangle edge is not drawn.

RO [angle]

rotated coordinate system

RO with 0 or no parameters sets up a normal coordinate system.  RO

with  90,  the  only other allowed value, sets up a system that is

rotated  90 degrees.  The method of rotation depends on  the paper

size.  The RO command does not change the scaling points, so an IP

command is useful after an RO.

The HP 7475 specifies two different meanings for RO90 depending on

the paper  size (A/A4  or B/A3).  PrintGL  decides  which  type of

rotation to do  based  on  the sum of the print  window  width and

height.  If this sum is greater than 23.75 inches then large paper

rotation is used. Otherwise, small paper rotation is used.

Small paper rotation:              Large paper rotation:

RO 0             RO 90             RO 0             RO 90

y��������Ŀ   0,0��������Ŀy     0,0��������Ŀy    y��������Ŀ

�        �      �        �         �        �      �        �

�        �      �        �         �        �      �        �

0,0����������x    x����������         �        �      �        �

�        �      �        �

�        �      �        �

�        �      �        �

x����������   0,0����������x

g-30

RR x_opposite_corner_delta , y_opposite_corner_delta

shade rectangle relative

RR  fills a rectangle with corners defined by the current position

and the command parameters.  The current pen  position and up/down

state are unchanged. The rectangle edge is not drawn.

SA

select alternate character set

SA specifies the alternate character set as current. The alternate

character set is chosen with CA.

SC [xmin , xmax , ymin , ymax]

scaling

SC  sets user units for subsequent drawing.  The units are defined

by  user values for P1 (xmin,ymin)  and  P2 (xmax,ymax).  Using no

parameters  turns  scaling off, and  subsequent drawing is done in

plotter units (1/1016 inch).

SI [width , height]

character size absolute

SI sets the capital letter  box  width and height  in centimeters.

Letter  spacing  is  1.5  *  width and line spacing is 2 * height.

Using no parameters with B or A3 paper is the  same as SI.285,.375

and any other paper is the same as SI.187,.269.

SL [tan_angle]

character slant

SL sets the character slant.  The parameter is  interpreted as the

tangent  of  the angle from vertical.  Using no parameter sets the

slant to 0 degrees.

SM[c]

symbol mode

SM sets symbol mode, which centers  the  symbol mode character  at

any  plotted  point.  Any character from ASCII  033  to  ASCII 126

except a ";" sets symbol mode.  Any  other  character turns symbol

mode off.

SP pen_number

select pen

SP selects a pen.  Using 0 or no parameter stores the pen  without

selecting a new pen.  This is usually done at the  end of a  plot.

PrintGL ignores any drawing with pen 0.

g-31

SR [width , height]

character size relative

SR sets the capital letter box width and height as a percentage of

P2X-P1X  and P2Y-P1Y.  Letter  spacing is 1.5  *  width  and  line

spacing  is  2  *  height.  Using no  parameters  is the  same  as

SR.75,1.5.

SS

select standard character set

SS specifies the standard character  set as current.  The standard

character set is chosen with CS.

TL [positive_length [, negative length]]

tick length

TL sets the tick lengths  used by XT and YT.  The  parameters  are

lengths above  and  below the current position as a  percentage of

P2Y-P1Y for x-axis ticks and  a percentage of  P2X-P1X  for y-axis

ticks.

UC [[pen_control ,] x_delta , y_delta] ...

user character

UC specifies a  user defined character.  Pen control number are 99

for pen down, and -99 for pen up. Pen movements are relative, with

x_delta in units of 1/4 the character  width  and y_delta in units

of 1/8 the character height.

VA

adaptive velocity - IGNORED

VN

normal velocity - IGNORED

VS

pen velocity - IGNORED

WG radius , start_angle , sweep_angle [, chord_angle]

shade wedge

WG fills a wedge centered at the current position. The current pen

position and up/down  state are unchanged.  The wedge  edge is not

drawn.

g-32

XT

x-axis tick

XT draws a vertical line, with length specified by the TL command,

at  the current position.  The pen position  and up/down state are

unchanged.

YT

y-axis tick

YT draws  a  horizontal  line,  with length  specified  by  the TL

command, at the  current position.  The  pen position  and up/down

state are unchanged.

esc.(

esc.Y

plotter on

Esc.(  and esc.Y turn the plotter on.  This is only relevant  with

the /YY option. It is not flagged as unsupported if /YD is set.

esc.)

esc.Z

plotter off

Esc.) and esc.Z turn the plotter off. They are only effective with

the /YY option.  The plotter on  and off commands let you  include

comments  or  data  to  pass through  to  another  device  in  the

plotfile. These are flagged as unsupported commands if /YD is set,

as an indication that you should set /YY.

esc.@, esc.H, esc.I, esc.M, esc.N, esc.R

RS-232 device control commands - IGNORED with /YY

g-33

HP-GL CHARACTER SETS

The HP 7475 includes the following character sets.  PrintGL supports

all of these except Katakana.

0  ANSI ASCII

1  9825 character set

2  French/German

3  Scandinavian

4  Spanish/Latin American

6  JIS ASCII

7  Roman Extensions

8  Katakana - NOT SUPPORTED BY PRINTGL

9  ISO International Reference Version

30  ISO Swedish

31  ISO Swedish For Names

32  ISO Norway Version 1

33  ISO German

34  ISO French

35  ISO United Kingdom

36  ISO Italian

37  ISO Spanish

38  ISO Portuguese

39  ISO Norway Version 2

The character sets specify a font for codes 33..127.  Codes 000..032

and 128..255 are independant of the character set and are all no ops

except these.

003 default label terminator

008 backspace

009 half backspace

010 line feed

011 reverse line feed

013 carriage return

014 select alternate character set

015 select standard character set

032 space

g-34

The HP-GL characters cannot  be  printed  with  the  standard  ASCII

character set.  The  file  CHARSET.PLT  is an HP-GL file  with three

pages that can be used to print these pages of PrintGL's manual. Use

the command PRINTGL CHARSET.PLT /M1 /AO0,0 with a /F option specific

to your printer to do this.

g-35

g-36

g-37

COMPATIBILITY AND TECHNICAL INFORMATION

PrintGL will run on any IBM compatible MS-DOS computer. It will work

in  a  region  as  small as 92K but  may need  more memory for  high

resolution  or wide prints.  116K should suffice for the  worst case

black/white prints  -  13.2 inches  wide, 360x360 dpi.  Color output

requires  an additional  32K  or more  for high  resolution  /  wide

prints. 196K is needed for the worst case.  A larger region may make

things go faster by allowing larger buffers and averting the use  of

a spill file.

By default PrintGL uses the BIOS printer interrupt to send data to a

printer.  If  a file  or device is specified for output then DOS 2.0

file handling is used instead.  Messages  are  written to the screen

with  standard  DOS  CON  output.   Graphics  screen  modes  (except

Hercules) are set up with BIOS, and graphics screen updates are done

by writing directly to the display buffer.

PrintGL takes the BIOS break interrupt (hex 1B) and the DOS critical

error interrupt (hex  24) in addition  to  those  taken by the Turbo

Pascal 5.5 startup code.

PMI will run on any IBM compatible MS-DOS computer.  It will work in

a  region as small as 152K.  When running  PrintGL,  all but 40K  is

given to PrintGL. More memory may allow PrintGL to run more quickly.

PMI runs PrintGL via a DOS shell.  It searches the current directory

and then the DOS PATH, calling the first copy of PRINTGL.EXE that it

finds.  PMI  sets up a new environment  with just a PRINTGL value to

set PrintGL parameters.

PMI  can use any text or graphics display  mode that allows at least

80 columns and 20 lines of text.  In text modes, PMI writes directly

to  the  display  buffer, and  in  graphics modes, BIOS  is  used to

display text.  When under DESQview, PMI should be started in a  text

mode, and it writes directly to the DV selected screen buffer.

The Microsoft mouse interface (interrupt  hex 33) is  used for mouse

handling.  Function 0 for reset, 3 for button presses, and hex B for

motion are the only functions used.

PrintGL and PMI  were  written  by  Cary Ravitz  and  compiled  with

Borland's Turbo Pascal 5.5 and Turbo Assembler 1.01.

g-38

SUPPORT BULLETIN BOARD

The Ravitz Software support bulletin board is at:

606-268-0577  1200/2400,N,8,1  24 hours/day.

To ask a question (or make a comment or suggestion), first prepare a

file with the details of your question. Please be very specific, and

include  any  related  information on your  hardware, software,  and

operating environment.  Then  call the bulletin board and choose the

upload question option.  XMODEM file  transfer protocol is  required

for this.  Your question will be assigned a number, (such as 1015) -

remember this number. An answer file (with the assigned number) will

be posted, usually within 24 hours.

To get an answer, call up  the bulletin board,  choose  the download

option, then the answer option, and then  enter  the  answer number.

XMODEM is also required for downloading answers.

All question and answer files are public.

The latest versions of PrintGL and Ravitz Editor are available  from

the  bulletin board.  These  are  packed in  .ZIP format and  may be

downloaded with XMODEM or YMODEM (1K XMODEM).

Anyone  may use the bulletin board, however priority may be given to

registered users of PrintGL and RE.

SUPPORT BY MAIL

You may send questions by mail to:

Ravitz Software Inc.

P. O. Box 25068

Lexington, KY  40524-5068

It is  best to  send questions  on diskette  (360K, 1.2M,  720K,  or

1.44M, 720K  is  best) in a standard ASCII file.  The answer will be

put on your diskette and  it will be mailed back  to you.  This will

make  it  easy  to include  example files  or a  new version  of the

software with the answer.

Return postage is appreciated.