BiosFix was originally written to correct two bugs that exist in

Tandy 1000 computers. If you are not aware, all Tandy 1000s

report an invalid display adapter and monitor installed in your

system. Under most circumstances this doesn't cause a problem.

However certain software will not run properly because of this,

especially if you have installed an EGA or VGA card. Fortunately

both bugs are easy to fix and a method has been found so that the

"patch" is compatible on all IBM PC-compatibles.

When you run the program two windows will appear on the screen.

The top window displays a copyright notice along with your

current video setup according to the BIOS. First the Video

Display Combination Code (Video DCC) is examined at segment

address 0040:008Ah in the BIOS Data Area. This byte should be

from 00h to 0Fh to indicate the video cards currently installed

in the system. A value of 00h indicates no video card is

installed. The Tandy BIOS sets this byte to 3Fh, which is

invalid.

The Primary and Secondary Display Code is obtained via INT 10h,

Function 1Ah, Subfunction 00h. The value in the BL register

indicates the primary display code and the value in the BH

register indicates the secondary display code. These values

should be between 00h and 08h, where 00h indicates no video card

is installed. The actual value returned depends on the video card

you have installed.

Previous versions of this program did not report the current video

settings. This has been added so that you can confirm that your

changes had been made and so that you could confirm whether or not

your System BIOS or Video BIOS is plagued by this bug. Also, the

first revision of this program had a minor bug itself. Instead of

patching the byte at segment address 0040:008Ah, it patched a word

at that address. This effectly changed the Diskette Data Rate at

segment address 0040:008Bh. OOPS! But this probably does not affect

PC/XT-compatible systems since that byte is normally 00h anyway.

Once these values are determined and displayed in the top window,

a second window is created on the bottom of the screen. All valid

video combinations are displayed as shown:

��������������������������������������������Ŀ

� No Display Adapter          MDA            �

� CGA                         MDA + CGA      �

� EGA                         EGA + MDA      �

� Mono EGA                    Mono EGA + CGA �

� PGC                         PGC + MDA      �

� PGC + Mono EGA              Color VGA      �

� Color VGA + MDA             Mono VGA       �

� Mono VGA + CGA              Mono VGA + PGC �

����������������������������������������������

Use the arrow keys to highlight the actual video card(s) you have

installed in your system and press ENTER to make the selection.

You can also press ESC to exit the program without making any

changes. This is useful if you want to determine whether or not

your BIOS is returning the proper display combination codes. This

is also useful if you want to verify that the selection you made

previously has been accepted. If the display combination codes

are correct, press ESC to exit.

The acronyms used in the menu stand for the following terms:

��������������������������������������������������������Ŀ

�MDA      �Monochrome Display Adapter                    �

�CGA      �Color Graphics Adapter                        �

�Color EGA�Enhanced Graphics Adapter with color monitor  �

�Mono EGA �Enhanced Graphics Adapter with mono monitor   �

�PGC      �Professional Graphics Card                    �

�Color VGA�Video Graphics Adapter with color monitor     �

�Mono VGA �Video Graphics Adapter with monochrome monitor�

����������������������������������������������������������

As the first step you will need to determine whether you have one

or two video cards installed in your system. If you have two

video cards installed then choose the item in the menu that

matches both cards. For example, if you have a VGA card with a

monochrome monitor and a CGA card with a color monitor, choose

"Mono VGA + CGA" from the menu. If you have a VGA card with a

color monitor and an MDA card with a monochrome monitor, choose

"Color VGA + MDA" from the menu. The dual display combinations

require one video card to be color capable and the other to be

monochrome. Also, the color-capable video card will usually be

the primary video card.

If you have one video card installed in your system, choose only

a single video card from the list. For example if you have a VGA

card with a monochrome monitor, choose "Mono VGA" from the menu.

If you have a VGA card with a color monitor, choose "Color VGA"

from the menu.

Use the arrow keys to highlight the video combination that

matches your system and press ENTER. BiosFix will make the

necessary patches to both the System BIOS and the VGA BIOS and

then return to the DOS prompt. These changes will be active until

you power off or reboot the system.