CAMP.DOC  1

- CAMP -

version 1.1

(John's Happy Camper)

90-July-4

A program to capture and analyze 2-way communications

(C) Copyright 1990, D. John Repici

All rights reserved.

CIS: 73577,337

GE: J.REPICI

The author of this program will assume no liability for losses caused by its

use.  This responsibility rests solely with the end user.

CAMP is a copyrighted collection of program and documentation (the

"package") produced by D. John Repici.  The program and document may be

copied and distributed for personal use provided that all components of the

package remain together, and provided that the copyright notices are NOT

REMOVED.

CAMP or any part of CAMP may not be sold to produce income, or included as

a component in a product or service which is intended to be sold to produce

income, without permission from the author.  The author reserves the right to

withdraw any permission given. CAMP may not be distributed in any government

or military sponsored database or network, nor may any part of CAMP.

**A "Starving artist" donation of $15.00 will be gratefully accepted.

Send to:

John Repici; 120 Jefferson St.; Riverside, NJ 08075

CAMP.DOC  2

Camp is:

A program to allow your computer to monitor two-way RS-232 data transfers

between a host and a terminal, or between any two RS-232 ports.  You run it on

a computer connected between the two ports.  To the two devices it is connected

to, it looks like a straight cable.  It captures data and stores it in two

separate files, one for data going from the terminal to the host, and another

one for data going from the host to the terminal.  There is byte-for-byte

representation of the activity on the ports in these files.

graphically:

com1                         com2

|                            |

[ HOST ]<------->[ your computer running CAMP ]<-------->[ TERMINAL ]

|                                      |

|                                      |

|                                      |

RS-232                                 RS-232

To run camp:

Properly connect your computer between the two devices to be monitored

(discussed later) and type CAMP at the DOS prompt.  You will get CAMP's opening

screen with the "top" menu.  There are two menus in CAMP, this top one and the

menu to change the com-port settings.  To select an item on these menus, use

the left and right arrow keys to move the pointer over the item and then hit

the <enter> key.  Next is a discussion of each item on both menus.

CAMP.DOC  3

The Top Menu:

Record:     Opens the files named with the "File" selection (discussed

bellow) and begins monitoring the ports.  If no file-name has

been specified, this selection will not be made.  When the

files are opened, they are not zeroed.  Any port activity

previously recorded to the file will remain, new activity will

be added on to the end of the file.

F10 = Stop "Record" mode and return to the top menu.

This is the only active key during recording. Note that the Esc

key is reserved in the "Record" mode (future release).

Playback:   Allows you to play the files back side by side.  Hitting the

down-arrow while in this mode will display the next 12 bytes of

the files being played.  Hitting the PgDn key will display the

next 36 bytes of the files.  The F1 key will toggle between hex

and ascii representations. Lastly, hitting the Esc key will

stop the playback and take you back to the top menu.  You can

not rewind (at this release).  In order to see the files again

you must hit Esc, to go back to the top menu and then choose

playback in order to view the files from the beginning.

F1 = Hex/ascii              Esc = return to top menu

Com..       This will take you to the com-port menu, allowing you to change

the port settings and to make those setting permanent.  The

com-port menu is discussed bellow.

Hex/ascii   This allows you to toggle between hex and ascii representations

of the information displayed in the windows.

File        Lets you choose a file name to record in.  The filename must be

entered without an extension.  Two files with the extensions

.XX1 (for com1 input) and .XX2 (for com2 input) will

automatically be created.  If the file already exists they

will not be destroyed.  Any new data recorded into them will be

recorded at the END of the existing data.

Buf         In order to maintain byte-for-byte synchronization between the

recorded input of the two ports a special "buffer" byte must be

added to take up space when one device is sending data and the

other device is not.  The byte used to pad the files can be

selected by you using this menu-selection.

erase       As stated, a file that has already been recorded to will not be

recorded over.  All new data will be placed at the end of the

file.  If you want to start fresh with a filename that has

already been used you can "erase" it.  This selection will

erase the file specified with the "File" menu item.

Esc         To exit the program, just hit the Escape key.  You will be

given your dos screen back.

CAMP.DOC  4

The Com-port Menu:

Baud        Use to set the baud rate.  place the pointer over this

selection and hit enter repeatedly until the baud rate you

want appears.

Datab       To select the number of data-bits (also called "word length")

you want.  Use the same way as the "Baud" selection.

Par         To select the correct parity: Odd, Even, Mark, Space, or None

Stopb       To select how many stop bits.  If you select a word-length of

"5" this will automatically be set at "1.5".  The setting of

1.5 stop-bits is only permitted for 5-bit bytes.

1-name      While displaying the files, it is convenient to not have to

keep reminding yourself which port is connected to which

device.  This allows you to enter a label (typically the name

of the device) for com1.

2-name      Same as "1-name" but for com2.

Altexe      Alter-executable.  This allows you to make CAMP.EXE come up

with the settings you have selected for: Baud, Datab, Par,

Stopb, 1-name, 2-name, and Buf (note that "Buf" can be changed

from the top-menu).  After selecting Altexe, the present

settings will be made part of CAMP.EXE.  Whenever you invoke

CAMP from the DOS prompt in the future, it will have those

settings.  CAMP.EXE must be in your current-working-directory,

for this option to work.  Only the copy of CAMP.EXE that is in

the current-working-directory will be updated.

Esc         Hitting the Escape key will take you back to the top menu when

you are finnished making adjustments.  Also, if you select

"Altexe" it will automatically return you to the top when it

has completed changing the executable file.

CAMP.DOC  5

Recording:

To the disk file:

All data saved to the disk files is synchronized byte-

for-byte using the "Buf"fer byte.  This synchronization

occurs AT THE INTERRUPT routine.  The EXACT activity at

the ports is maintained in perfect fidelity.

video output during recording:

The data shown in the windows while CAMP is recording

should not be taken too seriously.  It is not

synchronized, and worse, the ring buffers that service

the video output during recording will "spill over"

(they will ignore, and therefor lose, incoming data) if

they become full.  This will NOT EFFECT THE RECORDED

DATA.  only what's going past the windows while in

"record" mode.  The video output for recording is only

there so you can see activity on the ports, and make

some determination of what's going on.

Playback:

Using CAMP:

When you play the files back using CAMP, they are

displayed in perfect synchronization, side-by-side on

the two windows.  You can toggle between hex and ascii

representation while in the "Playback" mode by hitting

the F1-key.

Using other techniques:

For more convenient viewing (you can't move backwards

when viewing a file in CAMP) of either the com1 input

or the com2 input use a file viewing program such as

"List", or a binary editor.  The files are composed of

only the port data (and the Buffer bytes to synchronize

them).

CAMP.DOC  6

RS-232 connections:

For asynchronous communication the usual configuration of an rs-232 port is as

a DTE or a DCE.  The problem comes from the fact that the rs-232 standard is

full duplex, meaning it sends data out one line, and receives data back in on a

different line.  A DTE port will send data out on line 2 and receives data in

from line 3.  A DCE port will send data out on line 3 and receive data in from

line two.  Note how this makes the two complimentary.  Your computer is

"almost" ALWAYS a DTE, while, a modem for example, is a DCE.  This makes

connecting a modem to a computer very easy.  The data output by the computer on

line two is received in by the modem on its line-two, while the data output by

the modem on line three is received in by the computer on the same line.

graphically:

_____________________                       _____________________

|                     |                     |               DCE   |

| DTE                 |pin 2           pin 2|                     |

|       data out ---> |-------------------->|--> data in          |

|                     |                     |                     |

|       data in <-----|<--------------------| <---data out        |

|                     |pin 3           pin 3|                     |

|                     |                     |                     |

|_____________________|                     |_____________________|

The problem arises when you try to hook two DTEs or two DCEs together.  For

example, two DTEs (two computers for example).

_____________________                       _____________________

|                     |                     |               DTE   |

| DTE                 |pin 2           pin 2|                     |

|       data out ---> |---------------------| <--- data out       |

|                     |                     |                     |

|       data in <-----|---------------------|------> data in      |

|                     |pin 3           pin 3|                     |

|                     |                     |                     |

|_____________________|                     |_____________________|

Note that the data going OUT of one piece of equipment is going to the pin on

the other piece of equipment that is _itself_ sending data OUT.  Likewise with

the data in pins.  Here's where you need a "null modem", or "null cable" which

crosses the two lines so that two DCEs (or DTEs) can talk to each other.

CAMP.DOC  7

So we can make these assumptions:

------------ = existing cable           =============== = added cable

(not null)

+@ = added null modem adapter to camp

(null)

dTe=========================+@CAMP--------------------------dTe

(null)

dTe---------------------------CAMP+@========================dTe

(straight)

dTe=========================+@CAMP--------------------------dCe

(straight)

dTe-------------------------+@CAMP==========================dCe

Carry a null adapter and a straight cable to cover all bases.  Better have a

couple of gender changers in the kit too.

CAMP.DOC  8

History:

Release     Date        Particulars

1.0         7/3/90      First release.

1.1         7/4/90      First bug:  When you playback a new file that you

haven't recorded to yet v. 1.0 would creat files

with attributes of read/only.  This would place

two files on your disk of zero length that could

not be written to or deleted.  Bug fixed.

#