===============================================

Ven-Tel Inc. Default Settings for Exec-PC Users

With Error Correcting Modems

===============================================

The popularity of Ven-Tel modems in the bbs world is growing daily!  Ven-Tel is a leader in technical support of their modems, and is pleased to be able to offer users a listing of proper default settings for use when connecting to Exec-PC's BBS.

In addition to a five-year warranty (unmatched in the industry), Ven-Tel also offers its own technical support BBS for end-users and distributors.  For further information, please call the Ven-Tel BBS (T I P S) at (408) 922-0988 or (408) 922-0989, 24 hours a day, 8-N-1, 9600 bps max. v.32.

General Information

-------------------

Exec-PC supports two phone lines.  Both lines support RTS/CTS hardware handshaking, and XON/XOFF is not recommended.  Line 2 operates at a max bps rate of 9600 and supports v.32 & HST protocols as well as being v.42 capable.  As of this writing,  MNP levels through level 4 are available and plans to implement MNP level 5  are underway.  When using an error correcting modem, care should be taken to assure that it is set in auto-reliable mode so that a connection can be established even though compression may not be available.

When calling with an error correcting modem locked into XON/XOFF flow control, up/download errors or aborts may result.  The following information is provided to allow a smooth transfer and maximum throughput for your Ven-Tel modem.

9600 Plus/9600 Plus II - EC9600/EC9600II/PCM-96

-----------------------------------------------

AT &F \Q3 %C0 %F2 \N3 &Wn

Sets modem to factory defaults, then selects CTS/RTS hardware

handshaking, auto-reliable mode for error-correction.

Writes to NVRAM (where n is the mode) for default setting.

Change other registers as required by your terminal

software.

2400 Baud with Error Correction - PCM-2400E/EC2400-33/MCM-24E

-------------------------------------------------------------

AT \N3 \Q3 &W

Selects CTS/RTS hardware handshaking, auto-reliable mode for

error-correction.  Writes to NVRAM for default setting.

NOTE:  At publication date, Exec-PC expressed plans to implement MNP level 5 error correction.  When MNP 5 is available, change %C0 setting to %C1.

Internal Macintosh II Modem - MAC2400E

--------------------------------------

AT \Q3 \N3 &W

Sets up for an error-corrected call (v.42 compatible) and

writes to NVRAM.

NOTE:  If you experience difficulties up/downloading, set up your configuration file to disable XOn/XOff (allow hardware handshake).

FINALLY...

==========

Ven-Tel has been a leader in online communications since 1975.  In 1980, Ven-Tel was the originator of the full duplex auto-answer/auto-dial 1200 bps modem as well as the pioneer of the Half Card for the IBM PC.  With new internal modems for both the PS/2 and the Macintosh, Ven-Tel remains committed to providing technical advancement and unsurpassed support.

If you are thinking of buying a new modem for your PC, PS/2, Macintosh II, or other computer, think about the Ven-Tel advantage.  All new Ven-Tel modems feature a full five year warranty and technical support, including access to the T I P S BBS.  Most new error correcting modems will feature both v.42bis and MNP5 for compression ratios surpassing 3:1 and massive savings in online connect fees.  For the location of a dealer near you, call the Ven-Tel Inc. Marketing Department at

(800) 538-5121 from 8 a.m. to 5 p.m. PST.

=== END ===