CW-2-K1.DOC                      19-Sept-89                      Page 1 of 4

----------------------------------------------------------

CAKEWALK TO KAWAI K1 SYSTEM EXCLUSIVE DATA TRANSFER MACROS

----------------------------------------------------------

by Doug Fetter

CONTENTS:

---------

The CW2K1.ARC contains two files:

CW-2-K1.DOC - The documentation file that you are currently looking at.

CW-2-K1.ASC - The ASCII Macro information to be added to the CAKEWALK.SXT

file that came with your copy of Cakewalk.

WHY?:

-----

By using the Cakewalk System Exclusive functionality, you can have all the

appropriate Single & Multi Patches loaded to the K1 without having to call

up a separate Librarian every time that you change what you are working on

in Cakewalk. This function is by no means intended to perform the functions

of a full patch Librarian. It merely simplifies the process of switching

from sequence to sequence in Cakewalk; keeping all the associated patches

automatically linked with each individual work.

TO ADD MACROS:

--------------

The CW-2-K1.ASC file can be "read" into the CAKEWALK.SXT file using a word

processor in non-document mode. It can be positioned within the file at any

location you want to place it in order to make it appear at a convenient

spot on the Cakewalk window when you actual perform a SYSEX Receive. Another

way to add it to the file is to use the DOS copy function by typing in the

command line:

COPY CAKEWALK.SXT+CW-2-K1.ASC CAKEWALK.SXT

This will cause the CW-2-K1.ASC file Macros to be added to the end of the

CAKEWALK.SXT file which will then be replaced with the combined result. I

recommend doing this with a working copy which you have backed up, not the

original (just in case).

CW-2-K1.DOC                     19-Sept-89                      Page 2 of 4

PREPARING THE K1:

-----------------

Prior to using the Cakewalk System Exclusive functions, the K1 must be set

into the correct mode to receive SYSEX messages and to properly respond.

The following steps need only be performed one time, assuming that you do

not later change the settings about to be made.

NOTE: The Patch Data Memory of the K1 is going to be placed into a

WRITE PROTECT OFF mode. This means that the patches contained

in the K1's memory are now able to be overwritten.  This is

the ONLY way to download new patches to the K1 via MIDI; there

is no temporary patch buffer in the K1 for MIDI download oper-

ations.  If you have not already saved the patches that came

with the K1 to disk, now would be a good time. However, the

K1 factory patches are available in MIDIEX format on this

bulletin board.

1)  Depress the K1 "SYSTEM" button one time.

2)  Depress the "- NO" button until the display indicates

SYSTEM/MIDI

=SYS

3)  Depress the "SYSTEM" button until the System display indicates

INT PROTECT=ON  (or OFF).

4)  Depress the "- NO" button to get the indication INT PROTECTION=OFF.

5)  If an external memory card is plugged in, repeat steps 3 & 4 to get

the System display to indicate CARD PROTECT=OFF.

6)  Depress the "SYSTEM" button until the display indicates

SYSTEM/MIDI

=SYS

7)  Depress the "+ ON" button until the display indicates

SYSTEM/MIDI

=RCV

8)  Depress the "SYSTEM" button until the MIDI display indicates

RCV CH  =  # (where # = a number from 1 to 16).

9)  Determine the channel on which you would like to send messages to

the K1 and depress the "- OFF" &/or "+ ON" buttons to obtain this

number. If in doubt, set it so the MIDI display reads RCV CH  = 1.

10)  Depress the "SYSTEM" button until the MIDI display indicates

RCV EXCL = ON (or OFF).

11)  Depress the "+ YES" button to get the indication RCV EXCL =ON.

CW-2-K1.DOC                     19-Sept-89                      Page 3 of 4

HOW TO RECEIVE:

---------------

To use any of these System Exclusive Request Macros to receive K1 patch

data, get into the Cakewalk .WRK file into which you want to incorporate

System Exclusive data. Then go through the following steps:

1)  Enter the Cakewalk View menu via the Alt-V key combination and then

depress "X" to select system eXclusive on this menu.

2)  Move the reverse video cursor line up or down to select the location

on the table into which to place the patch you will receive.

3)  Depress "R" to begin the Receive procedure.

4)  Cakewalk will prompt your for the channel number to use in

communicating with the K1; enter a number to agree with the number

determined in step 9 of the PREPARING THE K1 section.

5)  If you are performing a "One Int Patch Dump" or "One Ext Patch Dump",

Cakewalk will additionally prompt you for the Number of the patch.

Remember that on the K1:

Single(Upper-case)     single(Lower-case)      Multi

A 1-8           0 - 7                 32 - 39            64 - 71

B 1-8           8 - 15                40 - 47            72 - 79

C 1-8          16 - 23                48 - 55            80 - 87

D 1-8          24 - 31                56 - 63            88 - 95

6)  Cakewalk will initiate the dump operation, the K1 will indicate

"MIDI    DUMP   BUSY!" followed by "MIDI  COMPLETED!", Cakewalk

indicates that some number of bytes have been received (bottom line

of screen), then press any key.

(That number is:

97 for a One Single        85 for a One Multi

2825 for an All Single     2441 for an All Multi  ).

7)  Name the SYSEX line in Cakewalk with some descriptive name and

possibly the patch number to help you work with it in the future.

8)  If you want  the SYSEX data to be AUTO-matically loaded to the K1 on

subsequent loadings of this .WRK file, mark the SYSEX line as AUTO.

9)  Save your Cakewalk .WRK file to disk.

CW-2-K1.DOC                     19-Sept-89                       Page 4 of 4

HOW TO SEND:

------------

In order to have the System Exclusive patch data sent by Cakewalk received

by the K1, the K1 must have been set up as described in the PREPARING THE K1

section above. If it is not, the patch data will not be loaded into the K1.

1)  Load the Cakewalk .WRK file for which you want to send the

previously received and stored System Exclusive patch data.

2a) SENDING AUTOMATICALLY:

- If you had selected the AUTO-matic Send feature during the Cakewalk

SYSEX Receive operation above, the patch data was sent to the K1

automatically when the .WRK file was loaded; proceed to step 3.

2b) SENDING MANUALLY:

- Enter the Cakewalk View menu via the Alt-V key combination and

then depress "X" to select system eXclusive on this menu.

- Depress "L" to send aLl patch data,

- or -

move the reverse video cursor line up or down to select the

individual patch data entry to be sent and depress "S" to initiate

the Send procedure.

- Proceed to step 3.

2c) SENDING IN A SEQUENCE:

- You can send System Exclusive patch data at any time during a

sequence. However, this takes a second or two and might result in

a glitch if anything else is playing on the K1.

- Go to the Cakewalk event view for the desired track and insert a

SysEx event, using the System Exclusive line number corresponding

to the desired patch data.

- As long as the track is not muted or archived, the patch data will

be sent to the K1 as the sequence is played.

3) The patch data has now been loaded into the K1 Memory, but it is not

in the Working (or active voice) Buffer of the K1. To get it there,

you must select the patch either at the K1 or via a Patch event in

your sequence's event stream.

- If another patch is presently selected on the K1, select the

desired new patch at the appropriate time. This causes the K1 to

load the Working Buffer with the associated patch data from its

Memory.

- If the present patch being used on the K1 happens to be the same

number at which the new patch has been sent to the K1, the patch

must be deselected/reselected to cause the patch to take affect.

First select any other patch number and then select this newly

loaded one. This causes the K1 to realize that the data it has in

its Working Buffer is not what you want but must be reloaded from

its Memory.

4) When a new patch is selected in the K1, there is a slight pause while

the K1 transfers the data from Memory to the Working Buffer. Any

notes playing (or for that matter decaying) when the Patch Change

occurs will automatically get turned off by the K1. Therefore, Patch

Changes should be performed only in quiet moments in the sequence.