CAKEWALK TO ENSONIQ VFX TRANSFER ROUTINES

Title: CW--VFX.TXT								    Date: 10/09/89

__________________________________________________________________________

PURPOSE:

1.	To backup VFX presets, Multi setups, and edited programs to

computer storage media.

2.	To allow edited programs and special Multi setups to be

downloaded to the VFX during playback of a sequence.

UTILIZATION:

1.	Load the entire contents of the file CW--VFX.TXT into the

existing CAKEWALK.SXT (see your manual for further details on

this file) using the Text In function of a word processor or text

editor.  If you use a word processor, save the modified file as a

"DOS text file", "ASCII text file", or "non-document file".  Or,

2.	Use a text editor, word processor, or line editor to add the

necessary lines to CAKEWALK.SXT manually.  The lines you need are

the ones which begin with quote marks (") instead of double semi-

colons (;;).  Be sure to type the lines EXACTLY as listed.  I

suggest that regardless of which method you use, that you place

the text before or after the entry for the Ensoniq ESQ-1.

PLEASE READ THE COMMENTS IN THE .TXT FILE FOR IMPORTANT INFORMATION!!!

RECEIVING A DUMP:

1.	Select the dump you  wish to request.  You will get the message

"Waiting for Data"

2.	Press the STORAGE button on the front panel of the VFX.  Choose

MIDI SYS-EX.

3.	Select the type of dump you want - Programs, Presets, or

Multi-A+B.  If you choose Programs or Presets you will proceed to

another sub-screen which will ask you which data you wish to

send.

4.	You will see the message "Transfering Data - Please Wait", and

the byte counter in Cakewalk will increase.  When the counter

reaches the byte count specified in the comments it will stop,

and the VFX screen will return to the PROGRAMS screen.

5.	Press any key on your computer keyboard to record the information

you have transmitted, and then name the slot you have placed it in.

Don't forget to save your Workfile!

SENDING A DUMP:

Sending a dump to the VFX requires you to simply choose the slot you wish

to send to the VFX.  If you send a program to the VFX it will ask you if

you want to write it to memory.  If you send a Preset it will ask you

which area you wish the preset to be stored in.  If you send the Multi it

will replace the current Multi A+B setup.

PRACTICAL USES:

If you use the VFX as a multi-timbral synth with Cakewalk, you can store

your special Multi-A+B settings in the sequence workfile, and have them

automatically download at the beginning of the sequence.

If you have created a special program for a sequence, you can store it in

Cakewalk to be downloaded during playback.

SPECIAL NOTES ON VFX BULK DUMPS:

I STRONGLY recommend that you backup the Internal RAM Programs to computer

disk in case you must re-initialize your VFX.  The dump size of this

feature is too large for Cakewalk to handle; therefore you must use a bulk

dump utility such as MIDIEX or QSEND to save it.  This should not be a

problem; it is unlikely that you would need to send the entire RAM

contents to the synth inside a Cakewalk sequence.

SPECIAL NOTES ON USING CAKEWALK WITH THE VFX:

The VFX fully implements Polyphonic Aftertouch and Channel Aftertouch.

You must explicitly enable the recognition of these features inside

Cakewalk with the <SETTINGS><RECORD FILTER> command.

The VFX does not handle the way Cakewalk sends Note Offs (note with

velocity of 0) properly.  In order to use Cakewalk and the VFX together

you must start Cakewalk with the -O option (the letter O, not 0), i.e. at

the DOS prompt:

C>CAKEWALK -O

This switch will not adversely affect other synths you may have.

If you have questions or comments I can be reached via the Washington MIDI

Users Group BBS (703-532-7860 - Washington, DC) or the Sound Management

Music BBS (312-949-MIDI - Chicago)

Russ Parker

Happy Sequencing!