CW-2-D10

CAKEWALK TO D-10 TRANSFER ROUTINES

(Replaces CW2D10)

==============================================================================

USES:

-----

1.  To backup presets and edited sounds to computer disk.

2.  To enable Cakewalk to send the SysEx data for any voice while playing

a sequence.  You need not worry whether the voice you want was loaded into

the D-10's RAM;  Cakewalk will handle it for you.

TO LOAD:

--------

The entire file, CW-2-D10.TXT can be "read" into the file CAKEWALK.SXT using

a word processor in non-document mode.  Alternatively, you may type the three

needed lines of text into CAKEWALK.SXT by hand.  The lines you need (from

CW-2-D10.TXT) are the three that begin with ellipses (quote marks) rather than

with semi-colans.  Make sure that you copy the text EXACTLY.  I recommend that

you put it either right before or right after the MT-32 commands.

USING:

------

To use any of these Routines, you must be in a Cakewalk file.  Type <ALT>V and

X  (for View system eXclusive).

BULK DUMPS:

-----------

1.  RECEIVING (BACKING UP):  Type R (for receive) and prepare D-10 for a One-

Way Bulk Dump (see D-10 manual on Data Transfer for details).  You may

choose to backup up sounds, rythm, or both from the D-10 pannel.  Name the

SysEx line.  NOTE:  Each Cakewalk work file can hold only 3-5 dumps

(depending on size).  BE SURE TO SAVE YOUR CAKEWALK .WRK FILE!!

2.  SENDING (to D-10):  From the SysEx screen in Cakewalk, position cursor on

SysEx line that you wish to send, and type S.  It's done!  The contents of

the dump will remain in the D-10 until you send another dump (even if you

turn it off).  MAKE SURE YOUR D-10 HAS BEEN BACKED UP BEFORE SENDING A NEW

DUMP!!!

PATCH MEMORY:

-------------

1.  RECIEVING:  - Set D-10 in the Performance Mode.

- Press the MIDI button and flip through the diplay until you

see "PATCH DUMP".  Press either VALUE button to turn ON.

(NOTE:  Each time you turn the D-10 off, PATCH DUMP is reset

to OFF.  So, you will have to turn it on every session).

- On the computer, type R (for Receive), and select "D-10

Patch Memory".  Select a channel (whatever channel your D-10

is set to transmit on) and press enter.

- On the D-10, press the Bank/Number button of the patch you

wish to send.

- Wait until Cakewalk indicates that 580 bytes have been

received (bottom line of screen), then press any key.

- Name the Sys Ex line in Cakewalk

- SAVE YOUR CAKEWALK .WRK FILE!!

2.  SENDING:    - Set D-10 in the Performance Mode.

- Select the Bank/Number that you want to hold the new patch

- In Cakewalk's Sys Ex menu, position cursor to line you wish

to send, and type S.  It's done.  The entire patch (Splits

and Duals included) will be loaded.

- NOTE:  Your patch will vanish as soon as you select another

patch on the D-10 UNLESS you write each voice to RAM (Edit

Tone, then write).  Then you can write the Patch (Edit Patch).

Patch Saves are NOT recommended for use in sequencing.  Use to back up only.

TIMBRE MEMORY:

--------------

This is an excellent way to send Sys Excl messages to determine voices rather

than program changes.  In this manner, the voices you want for your sequences

need not have been previously loaded into the D-10.  Furthermore, as Cakewalk

can send Sys Ex messages at any point in a track, you need only go through

these procedures once!

1.  RECEIVING:  - Before beginning, note on what channel in your sequence the

voice is to be used, and find the voice on the D-10.

- Make sure the D-10 is set to Multi-Timbral Mode.

- Press the MIDI button and flip through the diplay until you

see "TIMBRE DUMP".  Press either VALUE button to turn ON.

(NOTE:  Each time you turn the D-10 off, TIMBRE DUMP is reset

to OFF.  So, you will have to turn it on every session).

- Flip through the D-10 display until you are on the Part/

Channel desired.  The display MUST show the same channel

that you will be using in the SysEx command.

- On the computer, type R (for Receive), and select "D-10

Timbre Memory".  Select the desired channel (this must be

accurate) and press enter.

- On the D-10, press the Bank/Number button of the tone you

wish to send.

- Wait until Cakewalk indicates that 262 bytes have been

received (bottom line of screen), then press any key.

- Name the Sys Ex line in Cakewalk.  I recommend that you

include the channel number in the name.

- SAVE YOUR CAKEWALK .WRK FILE!!

2.  SENDING MANUALLY:

- Same as for Patch Memory (above), except the D-10 should

be in Multi-Timbral Mode.

- As above, your timbre will be lost if you select a new one

on the same channel, unless you write it to RAM (edit tone).

3.  SENDING AUTOMATICALLY:

- Select the Auto-Send feature in Cakewalk.  Each time your

work file is loaded, the Timbres will be sent.

- If you use any Patch/Program changes in the tracks, the

Timbre you sent will be lost.

4.  SENDING IN A SEQUENCE:

- You can send a new Timbre at any time during a sequence.

However, this takes a second or two and will result in

a glitch if anything else is playing on the same channel.

- Go to event view for the desired track.

- Insert a SysEx event, using the SysEx line number coresponding

to the desired voice.

- As long as the track is not muted, the new Timbre will be

sent

All Patch and Timbres will be lost once the D-10 is turned off, unless they

are saved.

=============================================================================

WARNING:  Although using SysEx sends of Timbres you programmed is a good

way to insure that other D-10/Cakewalk users will hear what you

want them to when they play your sequences, please abide by copy-

right laws.  Distributing sequences that contain the SysEx data

of copyrighted Voices/Patches may be in violation of copyright

law.

===============================================================================

If you have any questions/comments regarding these routines, I may be reached

at the WMUG BBS:  703/532-7860

--Jeffrey Morris

***

##########################################################