GetIt!

ShareWare MIDI file utility

version 3.5

by Aron Nelson

3/3/89

USERS MANUAL

DESCRIPTION

GetIt! is a MIDI file utility that can handle system exclusive

transfers between a MIDI device and your IBM computer or strict

compatible.  GetIt! can received MIDI system transfers from

a variety of MIDI devices and save the data to a disk file. This

allows you to have a library of files which contain the data for

all  of  your  MIDI  devices.   GetIt!  can  perform  some   file

manipulations  that  allow you to convert your  system  exclusive

data from one format to another by adding/removing  header/footer

bytes.

FEATURES:

Can receive system exclusive dumps as large as system memory.

System dumps can contain more than one dump.

Can transmit system exclusive dumps as large as system memory.

Can sequence up to 8 tracks of multi-channel music.

Versatile  peek mode with English/Decimal/Hex views  and  Log-to-

disk function.  User defined MIDI transmit strings.

Can  search through a file and find the beginning of  sysex  data

and begin transmitting the sysex data.

Can add sysex header/footer bytes to a binary file.

Can strip (delete) header/footer bytes from a binary file.

Can be automated through the use of script files.

Extensive script commands including CASIO mode and looping.

Can transmit many MIDI messages through built in script commands.

GETIT! MODES

1) RECEIVE MODE:  GetIt r

GetIt! is set up as a device that will wait for any system

exclusive  transfers.  System exclusive  transfers  are  received

"back  to back" in memory and stored as such. This allows you  to

receive  system transfers from a D10 and a Dx7, save them as  one

file, then transmit the whole system file as one.

When a system transfer has ended, GetIt! will display the

transfer status in kilobytes and bytes. The transfer window

will something look like this:

transfer status -> 43K 640 bytes

num sysex dumps -> 166

Press <return> only after the numbers stop moving, this will

end the transfer and take you to the file section where you will

be  prompted for a filename. Be patient as large  data  transfers

can  take some time. Because of the large amount of data that  is

being  received, the transfer status numbers only move *after*  a

system message has been received.

GetIt! checks for an overwrite when saving and will ask you  if

you  wish to overwrite anyway. The RECEIVE MODE is good for  MIDI

devices that can start MIDI transfers from the front panel like a

Dx7,  or  a D10, D20.  If you have a MIDI  device  that  requires

handshaking or prompting, then you must use script mode.

2)  TRANSMIT MODE:  GetIt t [s] <optional file to send>

To transmit a sysex data file that is stored on disk, simply type

GetIt t <file to send>. GetIt! will read the sysex file and

proceed  to send the data after reading the file. If  you

wish to be prompted through the procedure just type "GetIt t" and

GetIt!  will  prompt  you through the procedure.  If  the  second

parameter "s" is included, GetIt! will skip any extraneous header

bytes  and start transmitting the system exclusive file when  the

first  $f0 is found.  GetIt! will then transmit ALL  bytes  until

the  end of file.  Use the "s" command when you have  files  with

extraneous  header bytes before the actual system exclusive  data

starts.   One restriction, the "s" mode can only handle files  as

large  as  64K.  Any files larger than 64K will cause  GetIt!  to

stop  transmission.  As a general rule, anything smaller  than  a

sample  dump is OK. "S" mode is good for librarians such  as  the

one in CAKEWALK.  CAKEWALK adds header bytes to the beginning  of

sysex data files.

3)  PEEK MODE: GetIt p

Peek mode turns GetIt! into a midi terminal.  Incoming midi  data

is  displayed onscreen as (h)ex, (d)ecimal or (l)ine.  Line  mode

is actually english expressions telling you what type of data  is

being  sent and the following bytes associated with the  type  of

message.  The  type  of display can be changed "on  the  fly"  by

simply   typing  the  letter corresponding to  the  display  type

twice.  GetIt!  also  has a transmit mode  which  allows  you  to

transmit  MIDI  messages   to  your  MIDI device  and  view   the

responses   if   any  onscreen.  This is useful if  you  will  be

developing  script  files (mentioned  below).    By  transmitting

MIDI  data,  you  can   see   if  certain  system  commands   are

communicating to the MIDI device.

Note: A "hidden" feature allows you to display incoming clock and

active sensing bytes.  Active sensing and clock bytes disrupt the

diplay  because of the frequency at which they are sent,  however

if   you   wish to display them, just press the letters  "f"   or

"F"  then  press the type of display (D),(H),(L) and  the  active

sensing and clock bytes will be displayed. To stop showing active

sensing  and  clock  bytes, simply repeat the procedure and   the

function will be toggled off.

Version 3.5 adds 2 new features to peek mode:  Event logging  and

transmit libraries.  Event logging allows you to write everything

that  you see on screen to disk.  MIDI data is written to a  disk

file  which  can  be  viewed  with  any  standard  ascii  editor.

To activate event logging, type ALT-L.  GetIt! will prompt for  a

filename to log to.

Transmit  libraries allow you to create MIDI "strings"  that  you

can send to your MIDI device.  Up to 16 strings can be stored  in

any  one file.  Each string is composed of a list of  MIDI  bytes

that  you  would  like to send out the MIDI  port.   To  activate

transmit  mode press 't' while in peek mode.  When the  menu  for

transmit appears, type 'r' to read a .trn file. Once the file

is  read  simple point to the string you wish to  send  with  the

cursor keys and press <enter>.  Escape aborts the procedure.   If

you  would like to create your own transmit strings  simply  type

'e'  for  edit and an edit box will appear.  Type  in  your  MIDI

string  at this point.  To save your MIDI strings, type  's'  for

save  and  a dialog box will appear asking you for a  file  name.

Type in your file name and press enter to continue the save.

4) SCRIPT FILE MODE: GetIt c <script file>

Script file mode allows you to "talk" to your MIDI devices

if you have the system exclusive documentation at hand. By using

the  script  commands  in  GetIt!  you  can  initiate  a  2   way

communication  between your MIDI device and GetIt!.   The  script

commands and syntax are based loosely on Carter Scholz's MidiSave

script  language.  The  differences  are  that  in  GetIt!,   all

parameters   are preceded  by commands and that the  command  set

has   been   greatly  extended. There must  be  a  space  between

commands  and parameters. To create a script file all you  do  is

use an ascii word processor like Sidekick, and type commands into

a text document.

The commands available to you are:

A  All notes off.

Sends all notes off on every MIDI channel.

B  Blank (clear) the screen.

Clears the computer screen.

K  Wait for (IBM) keypress.

Waits for the user to press a key.  Pauses script file

execution.

M  Display message following M to (IBM) screen .

Displays message following M. Must be on a line by itself.

N  <channel> <noteX> <velX> Play note.

Sends a MIDI note on command to MIDI device.

P  <channel> <patchnumX>     Send patch change.

Sends a MIDI patch change to MIDI device.

R  <num>  Receive <num> transfers from MIDI device.

Receives <num> of complete MIDI messages from MIDI device.

S  Send a system message $F0 <messageX> $F7

Sends a system message.  Must be a complete MIDI message starting

with $F0 and ending with $f7.

W  <num>  Wait <num> milliseconds.

Waits (delays) <num> milliseconds - uses Turbo Delay procedure.

Useful for delays between notes.

Z  Save the MIDI sysex transfers.

Saves a system exclusive dump.

[  <starting index> Start loop at index.

]  <ending index>   End loop at index.

X  Replace with Loop Index.

/,\,{,},(,),;, are comment markers, follow with any comment.

Variables that end with an X can be replaced with the current

loop index.

NOTE:

The M and comments only work on a *line* basis. You can only place comments

and messages on a line by themselves on this release. I plan improve

on this in later versions. Do not mix commands and message/comments on one

line.

A loop is defined as:

[ <start number>

<any command>

] <end number>

A WORD ABOUT X VARIABLES:

X variables allow the user to substitute the LOOP INDEX  anytime

he  puts an X instead of a number.  The X variable ONLY WORKS  IN

LOOPS.  Lets say you want to transmit patch changes  from  1-127.

Create a loop [ 1   ] 127. Put this in the loop:

TR 192 X

Example:  [ 1

TR 192 X

] 127

The  X variable will substitute the loop index 1->127 and put  it

into X, looping through patches 1-127.

EXTENDED COMMANDS  (GetIt 3.5)

These extended commands allow GetIt! to send libraries that are stored

without header information.  Some librarians store just the data portion

of the system exclusive transfer and "throw away" the header info.

These commands will let you put them back in when transmitting the file.

Note:

They do not alter the files themselves.

TR  <byte..Byte..etc>

Transmit user defined bytes.

Sends  arbitrary bytes out the MIDI port.  Useful for  adding

headers   to   voice  libraries.  The  format   is   TR   xx1

xx2...xx3...xx4...etc. Cannot accept X variables.

BS  Binary Send.

Reads any binary file and sends the file as MIDI bytes.  These files

are  usually  files that do not include the MIDI  header  $f0

etc.  BS is an unusual script command because it prompts  you

for a file to send.

SS  Send Binary w/ search.

Reads  any  binary file and starts transmission  of  file  at

first  $f0  that it encounters.  Some librarians  prefix  the

binary with some header information, this command will ignore

them. SS prompts you     for a file to send just like BS.

IS   Initialize  Sysex buffer.  Use this command when  you  would

like to disregard the last sysex dump.  This initializes all

of the sysex variables to 0.  Use this when communicating to

your  MIDI kbd with the "S" command.  For example,  if  your

kbd  send an acknowledge message after you prompt it  for  a

system dump, send  the prompt command with the "S" command,

use  the  "R  1" command to wait  for  one  system  message,

initialize the sysex buffer to disgregard the message  after

receiving it with the "IS" command, then use "R <num>" to

receive the system dump.

CASIO COMMANDS:

Casio commands allow you to send and receive any type of MIDI

data.  Some  casio MIDI devices stop the  transmission  of  a

system exclusive file right in the middle of transmission and

require  a  handshake! Casio mode allows you  to  communicate

to the casio with the commands CT, CW and CR. Casio  commands

should be within the OPENCASIO and CLOSECASIO statements.

OPENCASIO  Open Casio mode.

GetIt! 3.5 now supports casio synthesizers. Full  handshaking

is supported with the CW and CR commands.

The OPENCASIO command prepares GetIt! for a CASIO script.

CLOSECASIO Close Casio mode.

This command closes the casio mode.

CT <byteX....byteX....byteX...etc>

This command tells GetIt! to transmit any bytes following the

CT command.  The CT command accepts X variables when used  in

a loop.

CW <num>  Wait for  <num> bytes.  This command will allow  GetIt!

to  wait for <num> bytes before proceeding to  next  command.

This allows you to prompt for a file using the CT command and

then wait for a casio acknowledge using the CW command. The CW

command  "throws" away <num> bytes - you don't  save  the

response  anyway.

CR Get CASIO dump.

This  command  prepares GetIt! for a CASIO dump.   All  bytes

received  through the MIDI in port are saved.  This  includes

All  MIDI information so don't press any notes or it will  be

saved too. Command terminates when it receives $f7.

CS  Save CASIO dump.

This command saves the CASIO dump that you have received.

NOTE:

Do not mix non-casio commands within the OPENCASIO and CLOSECASIO

parts of your scrips.

CAUTION:

You need to be very careful in CASIO mode when using the CW  and

CR commands.  In interest of speed, these routines will wait  for

until  they  get what they are expecting. They do not  check  for

keypresses.   When  you set CW to wait for <num> bytes,  it  will

wait there until you send the required number of bytes.  Likewise

with CR mode - it will wait until it gets an $f7.(end of sysex)

5) SEQUENCER MODE: GetIt s

GetIt! is also a simple MIDI sequencer. To start, just type

"GetIt s" on the command line and GetIt! will display an 8  track

sequencer. Type 'h' for help, 'q' to quit.

6)  FILE  PROCESSES  MODE:

GetIt  f  [h,s]  <infile>  <outfile> <strip/header file>

GetIt! can perform processes on binary files, adding or  removing

bytes from the beginning of a file and the end.

Basically, system exclusive data is required to start with a  $f0

and  end  with  an  $f7.  What comes in  between  is  up  to  the

manufacturer  of the MIDI device.  Usually the  system  exclusive

file has $f0 <manufacturers id> <etc.> header bytes *before*  the

actual  voice  data  starts.

Some  librarians remove and store system data without the  header

bytes,  only storing the voice data. When they send the  data  to

the  MIDI device, they add in the headers when sending  the  data

back.  GetIt! usually stores data with the header/footer bytes so

that  sending a file is quick and easy.  Nonetheless, GetIt!  can

convert files between the two standards.

HEADER FILES:

To  add bytes to the beginning of a system file (headers) and  to

add  bytes  to the end (footers), create a header  file  and  run

getit with these parameters:

GetIt f h <infile> <outfile> <header file>

The infile is the system exclusive file without the header/footer

bytes. Outfile is the new file containing the header/footer bytes

that you added. Header file is a ascii file containing two lines.

The first line contains the header bytes to add to the  beginning

of the outfile. The second line contains the footer bytes to  add

to the end of the outfile.

Example:   to add $f0 $43 to the beginning of a system  exclusive

file and $f7 to the end of a system exclusive file:

Create a header file with the two lines.

$f0 $43

$f7

The first line contains the header bytes to add to the  beginning

of  the new file.  The second line contains the footer  bytes  to

add to the end of the new file.

STRIP FILES:

Getit f s <infile> <outfile> <strip file>

GetIt!  can  also strip bytes off of the beginning  and  ends  of

files.  This  command is used to convert plain  system  exclusive

files containing headers to the format that some librarians use -

omitting the header/footer bytes.

To  strip  bytes off of a system exclusive file, create  a  strip

file  with  two  lines.  The first  line  contains  the  starting

position  to write from, the second line contains the  number  of

bytes to truncate off of the end of the file.

Example: To strip off the first byte and last byte of a file:

Create a strip file with these parameters:

2

1

The first line contains the starting byte position to write from,

in  this  case write from the 2nd byte of the file.   The  second

line contains the number of bytes to truncate from the end of the

file, in this case strip one byte off of the end of the file.

To  strip off only the first byte and leave the rest  alone,  use

these parameters:

2

0   <- no bytes stripped out.

To  strip  off only the end and leave the  beginning  alone,  use

these parameters.

1 <- write from first byte - leave alone.

1 <- strip off one byte off end.

ROUTINES FOR GETIT!

GetIt!  uses  the  TurboMIDI  toolkit,  available  from   N-FOCUS

Technologies.  TurboMIDI  allows you to  write  MIDI  librarians,

Simple MIDI sequencers, MIDI mappers, Ear training programs,  and

MIDI  utilities like GetIt!. TurboMIDI requires Turbo Pascal  5.0

from Borland International. The source for several MIDI  programs

are  included  free  with the TurboMIDI toolkit and  an  8  track

simple  sequencer  that  you can use in  your  own  applications.

Source code for TurboMIDI will be available at a future date.

HERE IT COMES:

Yes, GetIt! is a MIDI SHAREWARE utility, and the author <me>

requests that if you like and use GetIt! and find it useful

please send a $20 donation.

Regarding my Shareware thoughts:

If you like GetIt! (I hope you do), please send the $20 amount.

Almost  anyone should be able to "scrape" up the $20  amount.  If

you  can afford a MIDI device, then you can pay for  GetIt!.   If

you truly cannot pay for GetIt!, then just send me a note telling

me so and you will be added to the registered users list  anyway.

When  you  can pay for GetIt! I will expect  you  to  contribute.

Please do give copies of GetIt! to anyone and everyone! I want to

make  GetIt! a useful utility, let me know what improvements  you

would like to see. GetIt! is *not* public domain and is

not free.

If you donate, you will receive notices of future

updates and additional documentation for future GetIt! versions.

You will also be helping the program progress by making it

possible for me to spend some more time on GetIt!

You also get peace of mind, everytime you receive a system

transfer from your MIDI device :-)

Please send donations to:

Aron Nelson

N-Focus Technologies

1800 Stokes St. #197

San Jose,CA 95126

Software License

Getit.exe    MIDI Librarian.

Getit20.exe  MIDI Librarian/File utility.

Getit25.exe  MIDI Librarian/File utility.

GetIt30.exe  MIDI Librarian/Peek/Sequencer.

GetIt.exe    version 3.5 MIDI Librarian/Peek/Sequencer.

You are free to use, copy and distribute Getit! for

NONCOMMERCIAL use if:

**  No fee is charged for use, copying or distribution.

**  It is not modified in any way.

Clubs  and  user groups may charge a nominal fee  not  to  exceed

($10) for expenses and handling while distributing Getit!.

Site  licenses and commercial licenses for Getit! are  available.

Write  N-Focus  Technologies  for  more information.

DISCLAIMER

N-Focus Technologies hereby disclaims all warranties relating  to

this  software,  whether express or  implied,  including  without

limitation  any implied warranties of merchantability or  fitness

for a particular purpose. N-Focus Technologies will not be liable

for  any special, incidental, consequential, indirect or  similar

damages due to loss of data or any other reason, even if  N-Focus

Technologies or an agent of N-Focus Technologies has been advised

of  the possibility of such damages.  In no event  shall  N-Focus

Technologies's  liability for any damages ever exceed  the  price

paid  for the license to use software, regardless of the form  of

the  claim.  The person using the software bears all risk  as  to

the quality and performance of the software.

Governing Law:

This  state shall be construed, interpreted, and governed by  the

laws of the state of California, as well as those implications

made by the Federal copyright laws.

TradeMarks:

Turbo  Pascal  4.0  and  Turbo Pascal 5.0  are  a  trademarks  of

Borland,  MPU-401  is a trademark of Roland Music, IBM  PC  is  a

trademark of IBM Inc.

-------------------- CHANGES ------------------------------------

Getit  3.5  adds  extended  commands  to  the  script  language.

Extended commands are 2 letter commands such as TR and BS.

Extended commands  facilitate  sending  libraries   w/o   header

information.

03/04/89

Fixed bug when MIDI system commands were sent while recording.

Disabled Control-C.

Added 8 track sequencer.

Added transmit menu.

Fixed bug when decimal was selected in peek mode.

03/05/89

Added help to 'f' parameter.

Added Log to disk for peek mode.

Changed thanks notice.

Added IO checks for all file IO.

Added  help displays when partial parameters are entered  on  the

command line.

Added 60 character capability to log file string.

Added CASIO mode.

Added transmit modes in script commands.

Removed $ff requirement on transmit.

Changed  name  of  exe back to GetIt so that  the  shell  can  be

used for upcoming version.

###################