Sequencers, Midi and Music Software#

A "Sequencer" is defined as: a circuit, device, or software which records

and stores performance information so that it may reproduce the

performance; a sequence recorder. #

the Rock Synthesizer Manual

I. Sequencer History#

a).Analog sequencers#

Sequencers have been popular in electronic music since Dr. Robert Moog

introduced the System 55 synthesizer in the late 60's. This type of

sequencer was known as the analog sequencer, due to the setting of a

number of continuously variable(analog) poten tiometers to derive pitch.#

In Dr. Moogs' original design, a bank of 24 analog potentiometers could be

tuned individually, each producing a control voltage that was then patched

to the sound producing circuitry of the synthesizer. A variable clock

would step the sequencer from one potentiometer to the next, starting over

again once it reached the last position. These voltages, when patched to

the inputs of his voltage controlled oscillators, would produce varying

pitches. A repeating sequence of notes resulted and hence the device came

to be called a sequencer. This original sequencer had twenty-four pots, so

you could have a maximum of twenty-four notes or eight three-note chords

sequence over and over again. An example of an eight note sequence is the

droning pattern heard in i nk Floyds' "Dark Side of the Moon" recording of

some years back. Although other companies introduced stand alone versions

of analog sequencers about that time as well, Moogs' design is usually

thought of when the term is mentioned.#

b). Digital sequencers #

About 1975, Oberheim electronics introduced the DS-2, a "Digital"

sequencer capable of remembering one-hundred and forty-four notes.

Although it was only a monophonic device, the attraction to this type of

sequencer was that notes could be entered in both "real time" or "step

time" from the synthesizer keyboard. This was of great interest to

composers who would prefer to play notes in from the piano keyboard,

rather than having to painstakingly "tune in" one note at a time with a

potentiometer. This unit had the capacity for three sequences that could

either be played back independently or chained together to play in turn,

but there was no means of permanent storage for anything loaded into the

device. Since all information was stored in a "volatile" R A M memory,

everything was erased when power was removed.#

Shortly thereafter, Roland, a young company known mostly for their drum

machines, introduced a digital sequencer known as the MC-8 Microcomposer.

This was the first of many automated devices from the Roland company and

was considered to be clearly ahead of it's time. This device allowed

precise entry, editing and playback of a multiple of parameters including;

control voltage, step time, gate time, portamento and volume or modulation

settings. It was one of the first devices to implement tape

storage/backup-up functions, which is on nearly every

synthesizer/sequencer on the market today.   Although it could be accessed

from an external synthesizer for entry, manual step time entry was the

preferred method. It had an extremely large memory for that perio in time,

1200 notes, with an optional 16k upgrade that allowed over 5000 notes.

Designed for use with Rolands' modular studio synthesizer, the MC-8 was a

giant step forward for electronic music. An example of the precise step

entry method with the later version MC-4 is the complex synthesizer solo

of Totos' hit song, "Rosanna".  #

Using the control voltage scheme, polyphony was difficult to achieve

without the use of a modular synthesizer containing several oscillators.

Given even that, composers could enter only one single note passage at a

time. #

c.) System Common#

What was needed was a system with which a composer could play polyphonic

parts into the sequencer from the synthesizer and have it play back

verbatim. Determined to solve this problem, Oberheim Electronics

introduced in 1979 a new sequencer designed to w ork exclusively with

their OB-8 synthesizer, the DSX. The DSX allowed eight note polyphony and

full synchronization with their new drum machine, the DMX. This was a

great breakthrough,  and together these units would allow a composer to

generate fluid a d complex musical passages that could be edited, looped,

quantised, cut and pasted and heard with percussion. Soon thereafter,

Sequential Circuits followed suit and introduced a polyphonic digital

sequencer for their Prophet V and Prophet 10 synthesizers. This unit was

the first to utilize a disk drive for storage of song files. a#

II. MIDI#

Manufacturers were now slowly responding to performers' requests to find

ways allowing instruments of one brand to communicate key information with

others of different brands. The control voltage technique was

unsatisfactory because a separate communication line was required for

each notes' information. Also, there was no universally satisfactory

standard for this data transmission among the various manufacturers. #

Digitally controlled oscillators were just beginning to appear in units

such as the Roland Jupiter 8 and the Korg Poly Six. Since key information

was transmitted in the form of a number instead of a voltage, more

efficient handling of the data was possible. If all manufacturers were to

agree on the type, rate and size of the transmitted data for each key

depression, theoretically any keyboard could trigger any other. So, in

1983, with mutual cooperation between Roland, Sequential Circuits and

Yamaha, the MIDI specification was born. The Midi spec provides for the

transmission of a number of performance parameters including: note-on,

note-off, velocity, aftertouch(pressure), and controller data, such as:

pitch bend, modulation wheel, sustain pedal and others yet to be defined.

The original intention was merely to decide on a standard that would

permit transmission of data from one synthesizer to the next, regardless

of brand. Also provided for was a channel system to permit remote

selection of instruments. It was soon realized, however, that since this

data was in a form that could easily be handled by a computer, great

amounts of musical information could be stored and manipulated with a

microprocessor and RAM memory. #

III. Midi Recorders/Dedicated Computers#

The term "sequencer"  was now rapidly becoming out of date, since users

were no longer restricted to a small amount of notes repeating over and

over in a pattern. This new type of device would be dubbed a "Midi

recorder". Manufacturers rapidly began to design hardware sequencers(or

dedicated computers) to accompany their synthesizers, encouraging the user

to begin forming a Midi instrument network. One of the first MIDI

recorder/dedicated computers to appear was the Roland MSQ-700. This unit

could store over 6500 notes for up to 16 channels in its internal memory.

It had eight multi-channel tracks which could be recorded in real or step

time. These tracks could be linked together in up to 73 repetitions in

order to form a song. Also by Roland was the MSQ-100, a lesser expensive

but very capable Midi recorder. Since that time Yamaha and Korg have

introduced hardware sequencers. A tape storage back-up interface was also

provided for these units to permanently store song data for later

retrieval. The advantage of a dedicated computer such as this is the

convenience of portability and ease of use. Also of mention is the

inherent ruggedness of a 'made for portability' unit. The disadvantage of

these devices is the limitation of  expandability o f memory and

functions.#

IV. Computers and Midi #

The idea of using a micro or personal computer for the task of recording,

playback and editing of Midi events has recently become very much in vogue

due to the general public awareness of the personal computers' value in

other tasks. Musicians cannot rationally ignore the vast potential the

personal computer offers and sees the opportunity to combine a variety of

applications in one machine. Personal computers come in several different

types and with a varying amount of user memory with each. In order to make

an intelligent decision on which computer to purchase a user must decide

what software is currently available for the tasks to be performed and

what price point will be within his/her means.#

Among the least expensive, but well suited to Midi applications is he

Commodore 64 and 128 computers. While still lacking the power of more

expensive units, this computer has quite an admirable collection of Midi

software available for it. Although 64k of memory was the maximum

available for many personal computers of the seventies, the Midi data

format only allows about 5000 events to be recorded into this amount. Next

in apparent popularity was the Apple II computer. Able to display an 80

column screen and with expansion slots for adding accessories, this

computer was a big seller to many schools and individuals interested in

personal computing. An equal amount of Midi software is available for the

Apple II as is the Commodore. Although the Apple II is a much more

expensive computer, the Midi software for it remains on about the same

level as the Commodore. Moving up through the ranks and into the eighties,

the introduction of the Apple Macintosh showed much promise as a musicians

computer as well as a business machine. Even so, software for Midi and

business applications was slow in coming for the Mac. Present day

applications have proven that the Mac is indeed one of the best for use

with Midi.#

The IBM-PC had established itself in the early eighties as the machine of

choice for inexpensive business computers and was appearing on the desks

and in the homes of thousands of middle class information workers. This

computer is only now(1986) beginning to show promise as a musician's

computer, largely due to the fact that an enormous amount of work-alike

"clones" are appearing on the market at a price that is well within the

budgets of serious music hobbyists and performing musicians. Since (1.)

the bulk of modern business software has been geared for use on the IBM

and compatibles, and (2.) the wealth of peripheral equipment easily

attached to the IBM is expanding at at an alarming rate, music software

developers have naturally turned to this unit as a "standard" for which a

very large market is already established. #

Two new entries in the marketplace are showing much promise; the Commodore

Amiga and the Atari 520 and 1040ST. These computers are of the 32 bit

variety like the Macintosh, and are in a very affordable price range. The

software development for these machines is being approached very

cautiously as each companys' future is dependant on their success.

Developers are wary of investing the thousands of hours necessary to

develop Midi software for a machine that has an uncertain future. As of

this writing, to my knowledge, two companies have music software on the

market for the ST and only one for the Amiga. This is very disappointing,

especially in light of the fact that the Atari actually comes equipped

with a Midi interface as standard equipment. Both machines share the same

main processor as the Apple Macintosh for which there is a very nice

software catalog to choose from. #

V. Music Software#

Even though a special purpose computer designed exclusively for Midi

recording tends to be easier to operate, the flexibility and open

architecture of a personal computer could far outweigh that single

advantage. With the ability to provide full screen graphic displays of

Midi events and even musical notation for editing, not to mention the

storage capacity, the personal computer is a natural choice for

professional and novice composers alike.#

Even so, software writers who understand the musician's needs are very

scarce, so it is only natural to see the development of powerful, easy to

use music software as a slow process, growing more sophisticated as the

writers and the market mature. a#

a.) Software sequencers#

One of first companies to produce music software was Passport Designs.

Their first offering grew from the experience gained with their non-Midi

Soundchaser synthesizer/software system they had developed for the Apple

II in the early eighties. Passports' first package was called MIDI 4, and

was developed for the Apple //+,//e and Commodore 64.  This software

allowed 4 channels of polyphonic Midi recording/playback, but with a bare

minimum of editing capability. Each track could be looped continuously

and manually punched-in and out for recording, but single note editing was

yet to be implemented. Tracks could be merged together, but the resulting

track could only be sent out on one Midi channel. This meant that only a

total of 4 Midi instruments could be played at a time, a serious

restriction in that Midi allowed for up to 16 discreet channels. Although

not as full featured as many musicians would hope for, the feedback that

Passport and other would-be Midi developers received from this pioneer

product prompted more research into the possibilities of more powerful

music software.

#

Another company that had introduced music software about that time was

Cherry Lane Technologies, previously known for their large catalog of

sheet music and books. The packages they were offering were independently

developed but nationally distributed by Cherry Lane. Among these was

Texture, by Roger Powell. Roger had gained a lot of experience in the

sequencer field by working with the band Utopia and it's leader Todd

Rundgren.  Realizing that most songs he played were comprised of

reoccurring verses and choruses, Roger wrote this program to implement

programmed "links" or "phrases" that could be chained together to form an

entire song. This was a very useful feature and allowed more music to be

played back from less available RAM memory. To elaborate on this method of

song composition, software writers borrowed Rolands' idea (used originally

on the TR-808 drum 'computer') of saving memory by "calling" a previously

recorded sequence instead of having the computer remember all of the notes

used in each chorus or verse as it progressed, effectively having the

original sequence repeat when that part of the song came around again. In

Rogers' program, each of these "links" could contain a number of Midi

channels, so entire songs could be broken down into their basic

component parts; i.e. Intro, Verse, Chorus, Turnaround, Bridge, Ending,

etc. This method allowed more effective use of computer memory and saved

redundant passages from having to be re-recorded.#

To elaborate on this method of song composition, software writers

borrowed Rolands' idea (used originally on the TR-808 drum 'computer') of

saving memory by "calling" a previously recorded sequence instead of

having the computer remember all of the notes used in each chorus or verse

as it progressed, effectively having the original sequence repeat when

that part of the song came around again. In Rogers' program, each of these

"links" could contain a number of Midi channels, so entire songs could be

broken down into their basic component parts; i.e. Intro, Verse, Chorus,

Turnaround, Bridge, Ending, etc. This method allowed more effective use of

computer memory and saved redundant passages from having to be

re-recorded. Other companies such as Syntech, Dr.T's, Sonus, and others

have produced excellent software for the Apple and Commodore line using

the 'smart-looping' techniques. On the other hand, computers with very

large memory capacity like the IBM and Macintosh didn't have to worry

about economy in the storage of notes and use a 'straight-line' recording

method. If a previously recorded section needed to be repeated, the user

would "copy" that section and "paste" in in the track at the appropriate

time. An example of a very user-friendly program that uses this method is

Voyetra Technologies' "Sequencer Plus" program for the IBM. It displays

the command options in a below-screen menu. Using the first letter of a

command as the command itself proved to be a very intuitive and

non-redundant method of issuing the various instructions. The many

variables in this program are all assigned by either the plus and minus

keys or wit the greater-than or less-than keys. This program continues to

be one of most popular programs for the IBM today, due to its ease of use.

a#

b.) Sequencer entry methods #

Passports' Midi 4, Roger Powells' Texture sequencers and others relied on

real-time entry of notes. You either had to have your parts well rehearsed

before you began or deliberately slow down the tempo of the computers'

metronome in order to play parts in at a more relaxed pace. The other

popular type of entry method is "step-time" entry and is more commonly

used in hardware sequencers. Step-time recording does offer much more

control over the entering of very difficult to play passages, allowing the

electronic musician to enter passages that would be impossible for any

human to play. Step time entry does not rely on timing to be set by the

length of time a note was held down, rather the timing is pre-set and only

note position is needed to be entered by the performer. This allows non

keyboard players to easily construct sequences of perfect timing that

sound very smooth and even whe played back at normal speed. Each type of

entry method has its merits.     #

c.) Notation Software  #

Professional Composer, for the Macintosh is a non-Midi music notation

program. It depends on input from the mouse pointing and clicking each

note on a grand staff. This program is the most complete for music writing

at this time. It is capable is scorin g the most complex musical passages,

providing all of the general and many exotic music notation symbols, as

well as allowing the user to create his own. Full composers' scores can be

written in any format and single parts can be extracted in each

instruments' natural key and range. Text in any Macintosh font may be

inserted and even background graphics such as pictures can be inserted.

Although the music entered can be played back either from the Macintosh'

built-in speaker or external audio out, the qality of sound is very

limited and restricted to only four notes at time. Rolands' MPS on the

other hand is a Midi based program and includes both sequencing and

notation capabilities. MPS was met with much resistance due to its method

of user interface. MPS is controlled exclusively by the IBM's ten function

keys, labeled F1 through F10. This has proven to be a very unfriendly and

un-intuitive method of user interface because in one menu a given  F key

would perform one task, and on the next menu, the same F key would perform

an entirely different function. Musicians apparently desire a single

letter command for each function that will remain constant throughout the

program to perform the various tasks of recording, editing, and playback.

This program also faltered in it's notation aspects in that it could only

display a single measure of music at a time. #

Other integrated software for the IBM includes Jim Miller's Personal

Composer program. It is the most popular notation software for the IBM and

has a fairly good sequencer built-in. After music or computer keyboard

entry, the full-page music notation may be viewed and edited. Users may

also design their own symbols in addition to the full complement of

symbols provided. Although the music will not "scroll" across the screen

as it is being played, page up and down commands may be used to follow

along with the score.#

We should now distinguish between the three different types of music

software we have touched on up to this point. #

1.) Sequencing software#

2.) Notation software#

3.) Integrated software #

1.) Sequencing software generally allows the user the most  flexibility in

the type of input and in the editing of parts of the program. #

2.) Notation software is designed to provide automated print-outs of music

notation entered from the computer keyboard or mouse. #

3.) Integrated programs, such as Jim Millers' Personal Composer and

Rolands' MPS integrate portions of each type into one program. Since a

great amount of code is required in these programs in order to do both

jobs, compromises were made to both the sequencing and notation sections

resulting in functional limitations. A way around this problem is to

design each program to store their files in the same format so that each

can read the others'.

A successful example of this concept is used by Mark of the Unicorn. They

have split the functions completely apart and use a separate piece of

software to handle each job.  The sequencer companion to Professional

Composer, named Performer, is a sequencer with amazing editing and note

management routines, allowing over five hundred tracks containing up

150,000 events of Midi information to be edited and played back to the

resolution of a single 128th note.  Complete Midi event editing is

possible with this program. After composition and editing is complete, the

tracks can be re-arranged to the format of a conductors' score and then

saved to disk in a form that can subsequently be read by the notation

softw are. This two step method will probably be successfully merged into

one program as the price of memory becomes cheaper and processors become

faster. #

d.) Midi interfaces and Syncronization#

Through necessity, Passport developed the first Midi interface. These

first interfaces worked with the Apple // series and Commodore and

utilized one Midi in, one Midi out, and a 5-pin sync jack for use with

pre-Midi drum machines. This jack provided a 24 pulse-per-quarter-note

(PPQ) output or would receive same if the software was set to recognize

it. Another type of interface was required for Texture, Roland's Apple

software, and all IBM software. The interface, known as the MPU-401, is

manufactured by Roland for the Apple IIe, Commodore 64 and IBM personal

computers. It is termed a "smart" processor because it handles all of the

Midi data within itself and frees the computers' processor of these duties

in order to increase computer speed and allow more graphics. This

interface provides one midi input, two midi outputs and offers tape sync

as a standard feature. #

Syncronization with a rhythm device is important if the music sequenced is

to have any electronic percussion score within it. Since only non-Midi

drum machines such as the Roland TR-606, TR-808, Oberheim DMX, Emu

Drumulator and the LinnDrum existed at this time, some method of

syncronizing their clocks with a Midi sequencer was necessary. An

important point to consider here was that a still un-standardized PPQ

clock format existed among these manufacturers. Midi sync of 24 PPQ was

being implemented on the newer machines, but Sync clock formats of

24,48,96 and higher PPQ formats were used in these drum machines. Reliable

methods of syncronizing these machines with the computer constantly

baffled pioneers of Midi software and hardware. It bears mention here

that now prominent manufacturers such as J.L Cooper and Garfield

Electronics began their companies as "black box" interface designers for

the custom applications of touring bands attempting to solve such

interface problems. #

An optional port was now being offered by Passport

on their interfaces known as "tape sync". By taking the audio output from

this jack and recording it onto tape, a sync "tone" was created. This

tone, when routed back into the interface could control the sequencers'

start-stop functions and maintain syncronization over time. With a

multi-track tape recorder, a user could compose and edit tracks on the

sequencer, and sequentially transfer them one by one to the recorder in

perfect sync. The advantage to this method was in that an owner of only

one synthesizer could produce a tape with the sounds of many synthesizers.

This method is used quite frequently in recording studios to drive banks

of synthesizers in real time, while saving tape tracks for non-midi

instruments such as vocals and guitars. Although this interface has become

the standard Midi interface for the IBM series, very few manufacturers for

the smaller computers have adopted that interface, due to it's high cost

compared to the Passport interface.#

e.) Voice Librarian software#

One of the functions of Midi synthesizers that can easily be taken

over by computer is that of the digital storage of their parameter

settings, or "patches". By virtue of a portion of the Midi specification

known as "System Exclusive messages", each manu facturer is allowed to

specify the codes by which each model of their synthesizer can communicate

with another of the same brand and model. This was originally intended to

allow an owner of two or more of one model synthesizer to operate the

controls of one or more of the the same model remotely, i.e., change

control lever "a" and it will change control lever "a" on all of that type

connected in the chain, Midi being the networking buss. This is a nice

idea, but few musicians own duplicate instruments in their setup.#

What it has proven useful for however, is in the bulk transfer of

parameter settings of a particular model to another of the same model.

What the original engineers of the Midi spec didn't expect though, was the

use of a computer to emulate the commands of that second instrument in

order to direct the first to dump it's parameter setting onto a floppy

disk instead of into another synthesizer. With the proper software, a

computer owner could effectively bypass the use of a cartridge or tape

backup system to store an unlimited number of parameter settings (sounds).

This has become a very important and cost effective use of the computer in

a Midi setup.#

Among the first to utilize this feature was Computers and Music, Inc.

They researched the codes necessary to request system exclusive dumps from

the Yamaha DX-7 and wrote the program now licensed to Passport Designs,

Inc. The success of this program has encouraged them to write similar

programs for other popular synthesizers. On the horizon are programs

designed to contain the codes for all current synthesizers that will write

in file formats to be compatible with all popular personal computers. This

will solve the problem of owners of different computers attempting to

share data from like synthesizers, but using different librarian software.

Currently, librarians exist for all personal computers, with only one

sharing a common file format, this being SYSEX by Key Clique. the drawback

of this program is that is only acts as a 'bulk dump' utility, not

allowing individual patches to be transferred between banks.#

The use of a system exclusive software based librarian has several

advantages over the common method of cassette of cartridge based storage

methods. First, the transfer of information is instantaneous, avoiding the

hassle of level setting to a recorder, which in itself can be a major

endevor. Second, (especially evident in a cartridge system) the amount of

patches storable on the medium verses cost is substantially reduced with a

floppy or hard disk destination verses a limited cartridge destination.

Virtually unlimited resources are availa ble on the hard drive when

compared to the very limited capacity of a standard RAM cartridge. The

only advantage to the cartridge method is in it's portability. This idea

is voided if a software based sequencer is to be employed in the same

system.#

Third, the inherent full screen display of all of the voices in a

given bank is of great advantage, allowing you to view as much as 64

sounds at a time in their prospective order. By virtue of the computers'

memory, several banks can be viewed at a time, allowing a mixing and

matching scenario between the various banks of the synthesizer, through

which you may create custom banks of selected sounds on a one-by-one

basis. Although most modern synthesizers have the ability to send their

voice parameter information through Midi, it is still very confusing to

the receiving computer if all synthesizers in the system transmit the same

command at once and on the same channel. In a large system, these commands

will often be issued on a particular channel, accessing a single

synthesizer module at a time.#

f.) Patch Editing software#

Patch editors are another animal indeed, as these programs are

designed to allow the user full access to every parameter of their

synthesizer from the computer, using system exclusive messages. This

software has made the programming of synthesizers such the Yamaha DX-7

much easier as you can work with a full screen display of the information

instead of a the small window provided on the synthesizer. An excellent

example of this type of software is Opcode System's DX/TX Editor. A

computer patch editor of this type will free the fledgling synthesizer

programmer from the tedious chore of single lever digital editing system

of todays' synthesizer.#

To elaborate on that concept, once upon a time, a long time ago

(about three years now), synthesizers had a variable lever or knob for

every control function on the machine. Due to technological advances and

price cutting pressures, the number of actual control devices was cut to

one, since this one control could be 'assigned' to any function

individually. The user would select function '88', "sustain level", for

instance, and by changing the level of that control, the sustain level

would vary accordigly. Next, the user would perhaps select function '72',

modulation level, and continue to use the same control to change the

various functions of the entire synthesizer until the desired sound was

realized.  While achieving the goal of simplicity in design and

manufacture, this method has produced the unpopular side effect known as

"user-unfriendliness" and has put a damper on the enthusiasm of many a

prospective synth programmer. Voice editors have helped spark the desire

of experimentation to synth programmers and have instilled the sense of

being "in control" to new users of the otherwise confusing music

synthesizer.#

g.) Sound sampling software#

This is the newest and possibly most exciting development in music

software since Midi was conceived. For the first time, it has become

possible to plot and graph an actual sound, study it, change any portion,

and to hear the result to confirm the expected result. One of the most

difficult of all senses to understand, the acoustic phenomena, can now be

broken down into it's most basic parts, analyzed, changed and

reconstructed into a predictable whole.#

The software I am referring to is the Sound Designer series from

Digidesign, Inc. It is written to be used with the Apple Macintosh

computer in conjunction with digital sound sampling instruments by E-mu,

Sequential Circuits and Ensoniq, respectably. I n effect, the software

does not create the actual sound, it is the means by which the user can

examine and modify an already "sampled" or "captured" sound. These

instruments make a limited digital recording of an actual sound that is

then stored in on-bard memory. Through the magic of Midi, this digital

information can be transferred to the computer for detailed display.

Although very complicated to look at in it's raw form, the soundwave can

be reduced to a single vibration within thousands. The level to which

these programs can display sound data was previous only able to be

approximated by other analytical means. This advancement means that

studies in the nature of sound and the subtleties that differentiate one

sound from another can be concretely plotted, with meaningful results.#

h.)Sound Modeling#

The latest development in the field of on-screen sound research is a

new program called "SoftSynth", also by Digidesign. This program is

designed to be a tool by which a user can develop new sounds from scratch

using the additive synthesis method. This method of synthesis utilizes a

number of sine waves with different frequencies and amplitudes added

collectively to form a composite waveform. An existing instrument which

uses this method of sound generation is the Hammond organ with drawbars.

Each draw bar represents a different harmonic, and when each of the

various harmonics are added together in different degrees many complex

waves are possible, some of which resemble other instruments. The

SoftSynth program for the Macintosh, however, allows much more complex

additions of sine waves, as the user is allowed to specify the center

frequency and amplitude envelope of each harmonic, up to 32. Since it is

possible to plot any harmonics' amplitude through time, Fourier analysis

may be studied in real time and manipulated in order to better understand

how sound waves interact to produce sound of a given timbre.#

VI. Live performance#

Live performance utilizing Midi has to be approached carefully. Let

us consider what we are attempting to replace by casting a Midi sound

module in the place of a musician. When a musician plays, attention is

focused to that person. There is action throu ghout the performance and

the audience can generally identify the sounds they hear with the musician

creating it. In the case of the Midi band, however, much of that action is

missing. Even though the execution and technique may be perfectly

programmed , the slight human flaws that are inevitably included as part

of the live artists' performance are missing. That's what's wrong if

you've tried this approach and have gotten less than a standing ovation.

You cannot create a precise performance, have it executed by Midi modules

and expect to bring the house down when played. You have created no more

than an expensive "juke box" in this solo approach.#

What can be done however, is to accompany the real performers on

stage with selected Midi modules and effects. I cannot overemphasize the

possibility of having a well orchestrated performance augmented by one or

more Midi instruments as live musicians fill out the rhythm section . This

is the application of Midi that was originally intended. As we are

continually amazed by the proliferation of Midi controlled devices, we

must  decide which ones can be used expand our own abilities rather than

trying to replace them. #

VII. Midi effects#

Since Midi information is used to direct a network of instruments as

to which note to play, how long, which sound to switch to, etc., then it

seemed feasible to include a Midi jack on effects devices in order to have

them automatically switch through their functions also. Even though

effects are not instruments in themselves, when used with musical

instruments and/or voices they can enhance an otherwise dull performance

or bring realism to the most lifeless synthesizer sound. Examples of

effects that can be effectively Midi'd are digital delays, digital

reverbs, mixing and lighting consoles. Through creative use of these

"instruments", effects never possible by human adjustment are easily

attainable and repeatable.#

VIII. Education#

Not only does Midi make the realization of music infinitely easier to

produce after its original conception, it also allows beginners to learn

music and progress at their own rate without the constant supervision and

criticism of a human teacher. Modern, interactive, music software has

begun to win the affection of youngsters eager to learn music but are

apprehensive of having yet another teacher standing over them. Music will

always be difficult to master, but no one says it shouldn't be fun to

learn at the same time. With the aid of the computer, very  logical and

thorough programs of music training can be taught, repeatably,

consistently and with every bit as much interaction as could be realized

with a human teacher.#

The Future May consider the advent of Midi control to be the start

of a revolution in the musical community. Midi allows a single performer

to easily realize the sound of anything from a small ensemble to an entire

orchestra without the expense and human emotions usually associated with

groups of players. The only limiting factor in the midi composers' pursuit

of his music is imagination (and his budget). Musicians and the concept of

group playing will never become obsolete with Midi, as is often feared.

Nothing can match the feelings  of spontaneity between performing

musicians, nor can machines ever exude the warmth and feeling that a

talented player can coax from his instrument. Midi is a tool, a

multifaceted one. It must be used to enhance the talent that works with

it, not replace it. Through creative use of the many wonderful things that

are now possible with this digital standard, music can move on to the next

generation of expression, allowing us to experience musical textures that

before could only be imagined, and further on to musical frontiers that

today could not even be conceived. #

Gary L. Osteen#

##