=================================================================

SYSEX -Yet Another Sysex Program

=================================================================

This  is yet  another program  that sends/receives  SYSEX data. I

wrote it because I feel it  offers some capabilities not found in

other similar programs. It has  proved very usefull while working

with my YAMAHA TG55, for which I dont have a patch editor yet.

This software is public domain. Use  it or change it as you wish.

I am uploading the source (Turbo pascal 5) for anybody interested

to modify it. If you have  any problems, I would be interested to

hear about them, and, possibly, to suggest a solution. In fact,if

anybody  finds it  usefull, PLEASE   drop me  a note.  This would

encourage  me to  upload other   programs I  will develop  in the

future. Future versions of this program will appear if, and when,

I will  receive  any  problem   reports  and/or  suggestions  for

extensions.

(Note that I am uploading from  Greece, which means that it costs

me about  1$ per minute...  So, I would  like to know  whether my

uploads are helpfull  to anybody, or are just  falling in a black

hole.)

Overview

--------

SYSEX is  a program that can  send and receive SYSEX  data to any

synthesizer. Its main features include the capability to create

interactive  dump  request  files,  the  automatic calculation of

checksums, and the ability to work with human-readable files.

System requirements are :

1. IBM clone with at least 256K.

2.  MPU-401  compatible  interface,  standard configuration

(IRQ 2, Base 330h).

Program operation

-----------------

The command format is : SYSEX <Filename1> [<filename2>] /options

Depending  on the  required  mode,  filename2 and/or  options are

optional. If  the names have  no extension, an  extension of .SYX

will be assumed.

If SYSEX  is typed with  no parameters, a  breif help message  is

displayed.

There are 4 main modes of operation :

1. Send a file to the synth

---------------------------

Format  : SYSEX <filename1>

Options : None

The main use of this mode is  to send bulk data (like voice info)

to the synth.

The  file is  assumed (and  checked) to  be a  series of properly

formatted  sysex  messages,  i.e  any  number  of blocks with the

format : <SYSEXSTART> data bytes <SYSEXEND>.

where sysexstart is F0h, and sysexend is F7h.

The  blocks  must  be  consecutive,  without  any  intervening or

missing bytes, for example :

F0 dd dd dd F7 F0 dd dd dd dd dd F7

Missing or superflous start/end bytes will be reported as errors.

If  there are  more than  one blocks,  the program  will insert a

delay of at least 110  milliseconds between transmissions of each

block. This  is required by  some synths. If  this number is  not

enough, recompile  the program changing the  values in the <WAIT>

routine..

2. Send a dump request , and receive a sysex dump

-------------------------------------------------

Format : SYSEX <filename1> <filename2>

For <filename1>, all comments for mode 1 apply. The program will:

a. transmit the first file, as in mode 1

b. wait for sysex data to come in.

c. Store the received bytes in <filename2>

Since  there is  no reliable  way to  know when  a sysex dump has

finished, (as it may consist of more than one blocks),the program

will  continue  receiving  and  storing  bytes  until  any key is

pressed. To help you know when a sysex dump has finished, it will

display a  receive bytes count. While  this number is increasing,

the synth is  sending data. When this number  is stable, you know

that the synth has finished transmission, and you can hit any key

to end the receive and store the data.

3. Receive a bulk dump started manually from the synth pannel

-------------------------------------------------------------

Format :SYSEX <Filename1> /m

The  program works  like mode  2, but  skips the  first step  of

sending a file. It waits for the  data to come in, and the saves

them in filename1.

4. Convert between file formats

-------------------------------

Format SYSEX <filename1> <filename2> /c

In this mode, the program reads-in  the first file, and stores it

in the second file. No interaction is done with the synth at all.

This mode  is usefull for converting  between file formats, since

the input  and output formats  can be different.  See notes about

file formats.

File formats

------------

The program recognises and generates two file formats, depending

on the file name suffix:

if a  suffix of .TXT  is given, the  program assumes TXT  (Ascii)

format.

if any other suffix is  given, the program assumes sysex (binary)

format.

If no suffix  is given, the program assumes  .SYX, and interprets

the file as binary (same as above).

Format Description

------------------

a. SYSEX files.

--------------

These are  standard SYSEX (or  MIDIEX, as some  call them) files,

which are an exact binary  image of the sent/received data. There

are no headers or any other extra bytes. These "pure" sysex files

can  be handled  by most  sequencers. While  usefull, they do not

offer  any extra  capabilities. When  using this  kind of  files,

SYSEX works like any other sysex-exchange program.

b. TXT files

------------

This format is peculiar to  the SYSEX program. Conceptually it is

the same with format (a), but with two  major differences :

1. The  data are stored in  ASCII format, which means  that they

can be created/edited with any ascii word prossesor/editor.

Therefore, if you loaded the  file with a standard editor,you

would see something like :

F0 10 11 13 F7

(In contrast,  if you loaded  a format(a) file  in an editor,

you would see "garbage", like smiling faces etc)

Values are  delimited by any  number of spaces,  and the line

length  is limited  to 254  chars. There  should be  no blank

lines, since  this will terminate  file input. (This  feature

can be  used to add a  large block of comments  at the end of

the file, by  separating the comments from the  data with one

or more blank lines. Make  sure that any blank lines inserted

for this  purpose are REALLY  blank, i.e zero  length, and do

not  include any  spaces. Remember  that a  zero length  line

LOOKS the same as one that consists of spaces only).

The  error checking  is  rather  rudimentary, so  be carefull

about the data format...

2. When a file in this  format is TRANSMITTED, there is a number

of special constructions that SYSEX will recognise. These can

be embedded in the text,  and make all the difference between

using  this TXT  format and  the standard  sysex format  (a).

These are :

1. 'any text'.

If  any text  is  put  between quotes,  the corresponding

ascii values will be sent.  This helps you put ascii data

like voice names in the  text, without reverting to ascii

tables and such.

e.g. : F0 43 20 'AB' 20 21 22 F7 actually is sent as :

��

��

F0 43 20 41 42 20 21 22 F7

2. ;any text.

A  semicolon makes  SYSEX ignore  the rest  of the line.

Usefull for comments.

3. | xx xx.

The  bar (|)  will send  the logical  OR of  the next two

values. This is especially usefull with the ? construct

(see below).

E.g  F0 | 01 02 F7 =  F0 03 F7  (03= OR of 01 and 02)

4. ?Prompt

You will be prompted for  a value, and this will replace

this  construct.  Usefull  for  making  parametric  dump

request files.  For example, let's  say that your  synth

sends  a particular  part data  with the  following dump

request :

F0 43 01 2n F7 , where n is the part required.

Combining  constructs ?  and |  , you  would create the

following dump request file :

F0 43 01 | 20 ?Part F7

This line would result in the program asking :

Part :

After  you type  in the  part value (n),  this will be ORed

with 20h to give the required 2n.

Hint : If you want to type in the value in hex , precede

it by a $ sign. For example : $3F

5.  @,#  pair.  These  constructs  are  used  to  make SYSEX

calculate  7-bit  checksum  values  for  you (which some

synths require). When @ appears, the checksum counter is

zeroed. From this point onward, each byte transmitted is

calculated in the checksum, until  a # appears, which is

then replaced  by the 2's complement  of the checksum. I

have tested  this with ROLAND  and YAMAHA synths  and it

works fine, saving all the work of calculating checksums

by hand...  Remember, not ALL  bytes in a  sysex message

take  part in  the checksum,   thus the  need for  the @

character  to   signify  the  starting   point  for  the

calculation. This capability was the main reason I wrote

this program...

Example : F0 43 20 @ 01 02 03 # F7

�� start   ��� checksum

���������

results : F0 43 20 01 02 03 7A F7

Tips about using TXT files

--------------------------

1.  Since each  file name  in  a  command can  be in  a different

format, you can use SYSEX  to do format conversions. Also, if

you  want to  check your  parametric dump  files, you can use

SYSEX to see what exactly will be transmitted. For example,if

you have created a file  DUMP.TXT that has special constructs

in it, you can use the command :

SYSEX DUMP.TXT DEBUG.TXT /c

If you then  edit the resulting DEBUG.TXT file,  it will show

the  exact sequence  sent to  the synth,  with all constructs

replaced.

2.  It is  best to  keep your  dump request  files in TXT format,

since  it is  easier to  edit/maintain them.  Synth-generated

bulk dump files,  on the other hand, should  be kept in sysex

format, since there are probably no usefull substitutions you

would  want to  make during  uploading such  a file,  and the

sysex format works MUCH faster. In any case, you can convert

between formats with the /C option at any time..

Examples of usage:

-----------------

1. send file sounds1.syx to the synth

SYSEX SOUNDS1  or SYSEX SOUNDS1.SYX  (.SYX is default)

2. receive a bulk dump. Bulk request is in TXT format, save dump in

sysex format :

SYSEX REQUEST.TXT SAVED or

SYSEX REQUEST.TXT SAVED.VOI (any  suffix but TXT, means binary

file)

3. receive a manual-started  bulk dump, save in TXT :

SYSEX SAVED.TXT /m

4. convert between TXT and sysex formats :

SYSEX REQUEST.TXT REQUEST.BIN /c or

SYSEX REQUEST.BIN REQUEST.TXT /c

Limitations

-----------

The  maximum  buffer  size  is  currently  50,000 bytes, so files

larger than this cannot be sent/received.

Error messages

--------------

Input file not found

the program cannot find the file

File too large

buffer limit exceeded for input file

Too many data in

received data exceed buffer limit

Format error, missing Start of Exclusive

Blocks are not correctly formatted, see notes about transmitting

files to the synth

Cannot create dest

Error while opening  output file. Error in name,  or no space on

disk

Acknowledgments

---------------

I have used a unit downloaded from CIS, originally named MPU401,

with heavy modifications. It is quoted as created by John Sloan.

My thanks to him.

The idea of macro constructs, and the .SYX suffix come from

Greg Hendershott's fine CAKEWALK program. Special thanks.

Mike Cariotoglou,Greece

CIS 10012,1767

BIX avoreadis