Large Buffer Sound Editor 2.0

Randall M. Smith

DESCRIPTION

This program is designed to perform some of the same

operations performed by the sound editor which is supplied by

COVOX, but it is not limited by the 64000 byte boundary their

program imposes.  With this editor you can record, view, and

playback sound files up to 256000 bytes long.  However, it

supports only 8 bit sound files and does not presently allow

sound on sound or amlitude modification.

REQUIREMENTS

This program *requires* a VGA display, and a mouse is

strongly recommended.  It uses the C library routines supplied by

COVOX and thus works with the Voice Master Card and Speech Thing.

To use the new external Voice Master II (which I do not own), it

will probably be necessary to re-link this program with the new

version of the C library supplied with that product.  The

playback routine is set to use the Speech Thing.  This can be

redirected to the Voice Master using THING.COM.  Give the command

THING /P VM0 (or whichever port your Voice Master is connected

to).  If the program is re-compiled the amount of memory can be

changed very easily to fit a system with more or less free memory

available.  It should not be too difficult to adapt this for EGA,

and I hope someone will do that.  Since all of my systems are VGA

that is not very high on my list of priorities.

The code has been written and compiled using Microsoft's

Quick C 2.51, and thus should be compatible with Professional C

6.0 as well.  Previous versions were compiled with MSC 5.0 and

Quick C 2.0, so they should work as well.  I assume it can be

ported to Turbo C, but I have used a number of Microsoft specific

routines and features.

INSTRUCTIONS

The screen represents a window 640 units wide looking into

the sound file.  In the standard mode, each unit represents 100

bytes of data in the sound file.  In the zoomed mode there is a

1:1 correspondence between units and data bytes.

There is a set of buttons drawn on the screen below the

display area.  They can be used either by clicking on them with

the left mouse button or using the keyboard equivalent.  The

keyboard equivalents are:

<==       PgUp

Play      F1

Zoom      F2

Recrd     F3

Load      F4

Quit      Esc

==>       PgDn

A description of each button follows.

Left and Right Arrows ( <== and ==> )

These buttons move the window one frame to the left or

right.  In the normal mode this means one block of 64000 bytes,

in zoomed mode one block of 640 bytes.

Play

This button causes the portion of the sound file between theleft and right cursor points to be played back.  See below for

more information on setting the cursor points.

Zoom

In normal mode (1:100) this button causes the editor to

switch to zoomed (1:1) mode.  This button is also used to switch

from zoomed back to normal mode.

Recrd

This button allows an 8 bit sound file to be recorded.  This

file can be as long as 256000 bytes in the original version.

After clicking on this button the system prompts for a file name

then for a rate.  The rate number is *not* the frequency but the

one byte value which the C routines use.  If in doubt try 100;

132 seems to be close to the maximum speed of the COVOX hardware.

When ready to begin recording press <ENTER> then any other

key to stop recording.  The system will also stop recording when

the end of the allocated memory is reached.  After the file is

saved to disk, it will become the currently displayed file.

Load

This button loads a previously recorded 8 bit sound file and

displays it on the screen.  It can then be played back using the

play button.  The program prompts for the file name to be loaded.

Quit

This button quits the program.

Left and Right Cursors.

The left and right mouse buttons can be used to set left and

right cursor points (there is currently no keyboard equivalent

for this function).  To do this, put the mouse cursor in the

waveform display area and press the left or right button.  The

status line at the top of the screen will display the new value.

Note that the cursor points do not have to be in the same block

or section.  A cursor point can be set, the display scrolled to

another location, and the other cursor point set.

When the play button is clicked only the portion of the

sound file between the two cursor points is played back.  This

can be used to isolate certain sound clusters (the reason this

whole program was written in the first place).

I welcome any comments and suggestions, and I certainly encourage

people to work on the numerous shortcomings inherent in this

version of the program.  Please post your new versions on the

COVOX BBS so that all of us can benefit from them.

Randall M. Smith

Department of Classics

University of California

Santa Barbara, CA  93106

E-mail:  6500rms@ucsbuxa.bitnet