Right from the Horse's Mouth

The Ad Lib "Gold Card"

According to a slick magazine ad pre-print which I now hold in my

hand, which was sent to me by Ad Lib INC., yes, Virginia, there IS a

Gold Card. At least there will be by this summer.

Official Name: Ad Lib Gold Multimedia Stereo Sound Adapters

Model Numbers: Ad Lib Gold PC-1000 for PC,XT,AT for normal disk based

applications. Microphone jack, tabletop microphone, game and

MIDI ports, standard 1/8" stereo out/in jacks.

Ad Lib Gold AT-2000. For use with AT class machines. Same

features as above, but with SCSI interface port for use with

CD-ROM based applications.

Ad Lib Gold MC-3000. Same as AT-2000, but for Micro Channel

computers.

Board Functionality: Multivoice sound synthesizer and digital to

analog converters for stereo playback of music, sound

effects, speech.

Microphone input and tabletop microphone for voice recording

and sound sampling.

Auxiliary input for digital recordings from CDs, audio

tapes, and other analog sound sources.

Audio mixer for combining CD audio, voice,

synthesized sound, and sampled sound signals.

MIDI in/out for interfacing with any standard MIDI

device.

Game port for all standard analog joysticks.

Fully downward compatible with Ad Lib Music Synthesizer

Card.

Full compliance with Level 1 multimedia extensions to

Microsoft Windows.

Sound Quality: 20 channels of synthesized sound plus two channels of

sound sampling.

Stereo output with programmable volume and tone controls.

Enhanced FM synthesis from 36 operators in groups of

two or four using eight different wave forms.

12 bit stereo DAC accepting 16, 12, and 8 bit data.

Antialiasing filter, plus two times oversampling and

two times undersampling digital filters.

Technical Versatility: Choice of PCM or ADPCM digital sampling at

rates of 44.1, 22.05, 11.25, 7.35, or 5.5125 kHz.

3:1 hardware compression/decompression.

Two 128 byte sampling FIFO with a choice of 8 interrupt

conditions.

16 byte MIDI I/O FIFO with three programmable timers.

Zero wait time between successive chip writes.

Programmable microphone gain, card localization, DMA

channel selection, interrupt selection, sampling

filter and on-board chip select.

Available ID code plus directly accessible/readable

registers stored in non-volatile memory.

I guess for the moment, this lays to rest any speculation on whether

or not the Gold really exists.

#