-------------------------------------------------------------------

|                     Synthesizer Toolbox 1.01

| |

|

-------------------------------------------------------------------

By Larry Battraw    ---         (c) 1989-

----------------------------------------------------------------

Disclaimer:

I take no responsibility for what harm or ill that may befall

any person who uses the information and programs contained in this

product. There is no guarantee whatsoever on the the quality of

these programs. I hereby release these programs into the public

domain for whatever purposes the user deems appropriate.

Larry Battraw

Jan. 19, 1990

----------------------------------------------------------------

Index:

Introduction

Hardware requirements

Software setup and prototype description

Hardware Design

What is LBsynth?

Good question.  LBsynth is merely an acronym for a

collection of programs and utilities I have created to utilize

some of the properties of General Instruments (tm) AY-8910A,

and the SPO-256 chips. I needed software to drive these chips

in a particular way. There was no software for them.

-Therefore I have written these programs to drive them. As of

today (1/19/90), they are:

KBD.C   - First version of a

keyboard program for the AY-8910A.

KBD2.C  - Second version of a

keyboard program for the AY-8910A-

includes glissando effect.

KBD3.C  - Third version. Major revision, including

support for up to six channels (what I

have), which are utilized to better

simulate a "real" keyboard i.e. you can

play chords.

KBD3D.C - Fourth version. Now uses library, header

file. Capable of some very interesting noises,

often sounding far better than the previous

versions. Will use six channels for "frequency

shift" effect, but ok with three.

SPCH.C  - Program to drive SPO-256. Revised to

allow saving/loading of words, and direct entering

of allophone set. Awkward, and CXL (tm) windows

are in the future.

----------------------------------------------------------------

Hardware Requirements:

To really make use of the utilities as they are,

you'll need the following: an IBM (tm) compatible computer,

a board like mine, a C compiler (preferrably Turbo C (tm)),

and a decent amount of memory (256k+). I work on a

dual-monitor, IBM XT clone with 640k RAM, two 360k disk

drives, and an Okidata (tm) ML 182- and of course, my board.

I used Turbo C 2.0 with the standard libraries to compile

all the programs, and of course, my own library. I'm afraid

I'm not able to support any other compilers at this time for

two reasons: One, these are beta-test versions, and I really

don't have the time; Two, I don't have any compilers besides

TC, Power C (tm), and Desmet C. Not exactly a wide range.

I have not been able to make my hardware setup work on

any computer but an XT-compatible, due to either software

addressing problems, or slow hardware on my board- I haven't

been able to figure which. Either way, if you decide to build

a board for a '286, '386 computer you're taking an awful

chance.  In case you do decide to build one for a '286/'386,

use a 74LS373 octal latch instead of a 74LS244 octal buffer

for faster bus speeds. Connect the latch enable of the the

74LS373 to the inverted output of the AND gate which is

connected to OE and CE. The software address to PEEK at will

need to be changed to accomodate the larger-sized EPROMS

which are used a '286/'386-type computer. The way to

calculate this is by taking a base address of $f000:e000

(hex), and subtracting the size in hex bytes from it.

Remember that a 64k-bit EPROM is rated in bits- not bytes,

and you divide the size by eight to get bytes. A 64k-bit

EPROM actually has 8k-bytes, so therefore you would subtract

$2000 from the base address ($f000:e000), giving you the

correct offset (the constant in my programs is called offst)

of $f000:c000. A 128k-bit would require you to subtract

$4000 from the base address, and so on. If you'll notice in

my source code, I have my address to PEEK at set to $f000:6000

(for the files that do not use the library).  This is because

I have the plug in the fourth socket over. I have an older

XT, which has six sockets, and this is convienent for me.

If you have more than two sockets (4 in a '286/'386),

subtract the multiple of the size of your EPROM from the base

address instead of just the size.

The speech synthesizer has a companion chip which allow a

serial setup, but this complicates software, and may slown down

things a bit. It also has no option for connecting any AY-8910A's,

and would require quite a bit of work to make such a modified

design possible. I opted for a paralell design that is fast, easy

to interface, and requires minimal "software twaddling" to work.

----------------------------------------------------------------

Software Setup:

I've tried to make each program as modular as possible,

so if a person opts for another design, they can change one

part of the program- not have to rewrite everything. To this

date, my programming is a little hectic, and ill-commented,

to say nothing of using some rather anomalous integer names.

I'll work on that in the future with luck, and try for a

little more "readability". Need I restate my original

premise that this is a very early version...

In the synth.lib file, the prototypes require the

following parameters passed to function:

Prototype name:

senddt0(int reg,int data);

Uses: <reg> is the register of AY-8910A to send <data> to.

Comments:

senddt1(int reg,int data);

Uses: <reg> is the register of AY-8910A to send <data> to.

Comments: This is only used is two AY-8910A's are

connected. It writes to the second synth chips' registers.

chx(int val);

Uses: <val> is a value from 0 to 4095 to send the channel

<x> tone-period register.

Comments: Only values of 0-2 are valid if one chip is

available.

chxv(int val);

Uses: <val> is a value from 0 to 15 to send to the volume

control register of channel <x>.

Comments: Use only values of 0-2 if one chip is available.

silence(void);

Uses: none

Comments: Sets all volume registers to zero.

nphs0(int val);

Uses: <val> is an integer from 0 to 31 that sets the phase

of the noise generator on chip 0.

Comments: none

nphs1(int val);

Uses: <val> is an integer from 0 to 31 that sets the phase

of the noise generator on chip 1.

Comments: none

sendchk(int val);

Uses: <val> is an integer that is sent to port A of the

AY-8910A, where the speech synthesizer should be connected.

Comments: Also checks to see if speech synth is done

talking before returning control to program.

int xlat(char *alph);

Uses: <alph> is a string pointer to the string which is to

be translated (an allophone). It returns the appropriate

code to be sent (using sendchk).

Comments: On an error condition (allophone code not

found), an error message is printed, and it returns a

value of -1.

unsigned int readfil(void);

Uses: Requires a file called "synth.cfg" to be in the

same directory as the file you are attempting to run. This

initializes the offset so the library will write to the

correct memory address.

Comments: To use the library, this function MUST be

called, or otherwise nothing will appear to be happening,

as the library functions will use an offset of zero, where

nothing is located. If the config file is not found, the

program will abort with an ERRORLEVEL of 1. It will also

terminate if there is nothing or a 0 in the config file

(ERRORLEVEL of 2).

----------------------------------------------------------------

Hardware Design

I have created the schematic diagram for the board

using AUTOSKEM (tm), and it details the main parts that go

into the board's construction, neglecting the amplifier. If

you will download AUTOSKEM and print the diagram out, it

should be fairly clear how everything is laid out. The XT

ROM socket should be accessed by soldering a ribbon-cable

with the data lines, etc.  to a 28-pin DIP header that will

plug into the place where a second ROM can go (for those of

you with older machines like mine, it will be the one next

to the real EPROM). The cable should be kept as short as

possible, to minimize RFI (radio-frequency-interference),

and the actual board should be carefully placed where it

will not short out anything (it's a good idea to insulate it

well). I keep mine in the second drive bay, as I have two

floppies- but no hard drive.

As far as the amplifier goes, I use a low-pass-

filter/mixer layout, which is relatively simple to design.

You must NOT connect the output from the speech synth and

sound synth together- the ouput of the speech synth is held

at 5 volts when it is inactive, and the sound synth holds

its outputs at ground.  Connecting them together will likely

destroy one. Use either an op-amp mixer, or a capacitively-

coupled design. If all else fails, copy the schematics

provided with the chips, with this correction for the speech

synth: the output from it is shown as being passed through 2

33K-ohm resistors before being amplified.  This is

incorrect, and should be 33 ohms. Next version released will

include a mixer/amplifier diagram along with the main

diagram.

If there are any questions and suggestions, feel

free to direct them to me via Hackers-R-Us BBS

(602-999-999), or the address you'll find at the end of the

document.

This is not a mega-bucks project, and I've tried to

keep the list of parts down to an absolute minimum, due to

my budget considerations, and whoever else who tries this

out.  For a board with two AY-8910A's and an SPO256 chip,

the partial parts list goes as follows:

LM386 Power Amplifier

2- 74LS08 Quad AND gates

74LS04 Hex Inverter

74LS244 Octal driver- connect the inputs of this

to the address bus {A0-A7}, and the outputs to

the sound synthesizer chip {DA0-DA7}. The sound

synth loads the bus too much, and your computer

will not run without this.

2- AY8910A Sound synthesizers

SPO256 Speech synthesizer

Speaker

3.579545 MHz Color burst crystal

Besides a few miscellaneous capacitors, and

resistors, that should pretty much complete it. Remember to

place decoupling capacitors around .1 mF across all IC's,

and keep wires short.  Please note that in the above parts

list, I have specified 2 74LS08's, and to AY-8910A's. These

are not required (only one 74LS08) is needed) if you build

only the main design. The second sound chip is optional,

extending the channels of sound to six from three, and

requires another AND-gate chip to properly interface it.

Check out the addendum to the main diagram (2synth.pic) for

details on how to contruct a dual-synthesizer chip design.

This is my address for any inquiries or ideas you may

have:

Larry Battraw

1547 E. Hilton

Mesa Az., 85204