MIMIC (tm) - a Digicorder (tm) utility program

(c) 1990 Eletech Electronics, Inc. All rights UNreserved.

MIMIC and Digicorder are trademarks of Eletech Electronics, Inc.

Yes! MIMIC, although copyrighted, is devoted to public domain. You can do

anything you want with MIMIC, free of charge. Source codes are included so

rebuild MIMIC if you are afraid of virus attack.

MIMIC is a utility program that reads Digicorder recorded voice file and

outputs voice data to an IBM PC/XT/AT compatible PC's internal speaker to

reproduce the recording. Due to the simpilicity of PC's speaker circuitry the

reproduced sound quality is not nearly as good as if played from Digicorder.

None the less this capability makes Digicorder an instant standard since now

every PC can play these recordings without any extra hardware!

This package contains the following files:

MIMIC.DOC - the file you are reading now

MIMIC.EXE - MIMIC executable

MIMIC.C   - MIMIC source code, the Microsoft C (5.0) part

MINT.ASM  - MIMIC source code, the Microsoft MASM (5.1) part

*.16      - sample recordings recorded at 16K bits/second sampling rate

*.32      - sample recordings recorded at 32K bits/second sampling rate

To play a voice file, just type

MIMIC sampling_rate filename

Here filename is the recording's file name (optionally with a search path),

and sampling_rate has to be an integer between 12 and 64. For example,

MIMIC 32 news.32

will play news.32 at 32K Hz (or bits/second.) You can play a voice file at a

sampling rate other than the one it's recorded at. The effect will be like

playing a 33 RPM album at 15 or 45 RPM. Pressing any key during the playback

will terminate the execution of this program.

MIMIC reads 32K bytes of data from the recording file at a time. If the

recording file is read from a floppy disk, a short playback distortion may

be heard whenever MIMIC reads from the floppy disk. The faster the CPU, the

less significant this distortion becomes.

MIMIC achieves CPU speed independent playback rate by re-programming and

intercepting timer 0 (clock) interrupt. It also uses (and restores) software

interrupt #255, although this interrupt can be easily replaced with another

one should conflict with TSR programs results. Note that MIMIC is not a TSR.

Although the playback rate is independent of the CPU speed, higher CPU speed

is required for high playback rate. Usually an 8MHz XT can playback at 16KHz

while playback at 32KHz requires at least an 8MHz AT. If the system is a

little bit too slow for the sampling rate, the playback tone will be slightly

lower. If the system is way too slow for the sampling rate, the playback tone

will suddenly be a lot lower due to much lower playback speed. Also, the DOS

clock will run a little bit off beat during the execution of MIMIC but the

hardware clock will not be affected at all. See MIMIC.C for more details.

The operation theory is really simple. When Digicorder records, it uses the

Continuously Variable Slope Delta modulation (CVSD) technique. This CVSD

technique, simply put, records the direction of the waveform bit by bit. For

example, if the wave goes up from the previous sampling point, a bit of "1" is

recorded, otherwise a bit of "0" is recorded. Absolute silence is recorded as

"01010101...". Basically, the higher the sampling rate, the better the sound

quality, but also the more data to store. Digicorder's sampling rate is

adjustable from 12K to 64K Hz (or bits/second.)

When Digicorder plays, the process is reversed and waveform reconstructed and

output to its own speaker, bit by bit in real time. When MIMIC plays, voice

data is sent to the PC's speaker bit by bit. These bits either charge ("1") or

discharge ("0") the capacitor in parallel with the speaker, simulating the

ups and downs of the original waveform. Thus the sound wave is reconstructed

in a crude but legible form.

The relatively high background noise is an unfortunate result of not exactly

reproducing sounds the CVSD way. Consider recording silence at 16 KHz sampling

rate, an 8 KHz tone should be heard when the recording is played (by sending

"01010101..." to speaker at 16 KHz.) Saturation/Non-linearity of the capacitor

also adds to the background noise. XT's in general have more noise than AT's,

probably due to poorer motherboard grounding (nowadays XT's motherboard

usually has only two layers. Some noise reduction is possible by pre-

processing the recording, but a way to significantly reduce the noise has not

been found. Note that these noises do not occur if Digicorder is used to play.

Digicorder, a half-length 8-bit PC/XT/AT compatible voice card complete with

microphone, speaker, tape/cassette recorder adaptor cable and utility/device

driver programs, can be purchased for $99 (Californians add 6.25% sales tax)

from:

Eletech Electronics, Inc.

1262 E. Katella Ave.

Anaheim, CA 92805

(714) 385-1707

Dealer inquiry welcomed. Eletech Electronics, Inc. is a manufacturer sales

office and also carries speech processing chips, stand alone mini speech

modules, voice telecommunications equipments and voice-mail products.

MIMIC package revisions:

1.0 - original release.

1.1 - a mistake in MIMIC.DOC about MIMIC usage is corrected.

- ANTHEM.16 is excluded to reduce file size.

- except for MIMIC.DOC, all other files remain version 1.0

#####################################