DX7II MIDI Master Controller

by:  Joe Bertin & Fred Martin BBS (MIDI Conference)

These performances were assembled for use at Mustang

Studios Inc.,  Distributed for commercial, and private use

by  "F R E D    M A R T I N    B B S"   216-726-4983.

If you find this program useful, a donation of $5.00 would

be appreciated, and should be mailed to:

Mustang Studios Inc.

806 Terraview Dr.

Youngstown, Ohio 44512

216-726-5519

You will find one file:

DX7IIPRF.GET  (GETIT 3.0 format)

This file will only work in a DX7II, or DX7IIFD with an

"E!" board installed.

Here's how it works:

1) Turn on your DX.

2) Press the "PERFORMANCE" button.

3) Press the "EDIT" button on your DX.

4) Press the #32 patch button.

5) Use the "Data Entry" slider to change the device

number of the DX to "1".

6) Press #14 patch button.

7) Cursor to "INT" memory protect, turn it "OFF".

8) Press the #31 patch button.

9) Cursor to "TRNS CH", turn it "OFF".

10) Cursor to "RCV CH", turn it "OFF".

11) Press the "PERFORMANCE" button.

12) Using  GETIT 3.0, transmit DX7IIPRF.GET  file to

the DX.

13) When data transfer is complete, press the

"PERFORMANCE" button.

Press the #1 patch button, and the LCD will display

"MIDI CHANNEL 1".  Tap a few keys on the DX.  Any unit in

your setup programed to receive on MIDI channel 1 will

"play".

Press the #2 patch button, and the LCD will display

"MIDI CHANNEL 2".  Tap a few keys on the DX.  Any unit in

your setup programed to receive on MIDI channel 2 will

"play".

The receiving unit will also respond to MIDI "VOLUME"

commands when you move the slider labeled "CS1", or a foot

controller connected to the jack labeled "FC2".  This is

handy in recording real time volume changes to a sequencer,

or correcting "levels" of tracks already in the sequencer.

Neat huh?  In the performance mode, each of the top 16

patch buttons will set up the DX as a master controller on

the corresponding MIDI channel.

You'll also notice that the DX calls up "PATCH 1" in the

receiving unit, each time you change MIDI channels.  This

can be a real "pain in the butt" if your constantly

changing MIDI channels.  I have four solutions for this

problem.

1) Disable MIDI patch changes in the receiving unit.

2) Keep the patch you're currently using in the first

memory location of the receiving unit.

3) Program the DX to send the receiving unit the

proper patch change.  Try this:

a) Press the "PERFORMANCE" button.

b) Press the #1 patch button.

c) Press the "EDIT" button.

d) Press the #28 patch button.

e) Cursor over to "PATCH", and use the "DATA

ENTRY" slider to change "00" to "03".

f) Press the "PERFORMANCE" button again, and save

the edited performance.

Now each time you select "MID CHANNEL 1", your DX

will automatically change the patch on the receiving

unit to "PATCH 4".

4) Use Performances 17 through 31.  These are

programed differently, to let you change MIDI

channel alone. Here's how:

a) Press the "PERFORMANCE" button.

b) Press the #17 patch button.

c) Press the bank selector button labeled

"1 > 32" to move upward through all 16 MIDI

channels, without disrupting the settings in

any of the receiving units.

Performance 32 is a quick way of getting out of the "OCTAL"

mode, and back into the "SINGLE" voice mode to re-establish

normal play.  Try this:

1) Press the "PERFORMANCE" button.

2) Press the #32 patch button.

3) Press the "SINGLE" voice mode select button.

4) Play a few notes on the DX, you should hear only the

DX.  None of the other units in your set up should

sound.

This setup is only meant as a convenient starting point for

customizing your "E!" equipped DX7II as a controlling

keyboard.  I would suggest changing "LEVEL", "TRANSPOSE",

and "CURVE" settings to optimize the feel between the DX,

and any of the receiving units.  These settings can be

accessed by pressing the "EDIT" button, then pressing the

#28 patch button once.  Once you've got everything set up

just right, don't forget to "save" the performance.

If you have any questions, or suggestions I'm available at

"F r e d    M a r t i n    B B S"  216-726-4983 or,

Mustang Studios Inc.

5405 Market Street

Youngstown, Ohio  44512

216-726-5519