01/30/88

Roland MT-32 Performance Setup Utility for the IBM PC/MPU 401:

=============================================================

Introduction:

-------------

Having recently acquired an MT-32, I was immediately impressed by most of

its capabilities and functions. That was about a week ago and now, after

running a few sequences through the '32, I've begun to get a bit bored

with the default Part assignments (e.g., panning, reverb status, etc.). A

quick look at the MIDI implementation guide and I realized that there was

a plethora of features that this little machine possessed that would be

accessible to me if only I had the patience to study them. This program

is the result of that patience and the kindness of another MT-32 user out

there. If you like this program send me some incentive for improving it

by mailing your check for $10.00 (comments more than welcome) to:

Emilio Abello

2043 Oregon Ave.

Redwood City, CA. 94061

Disclaimer:

----------

This is the First Version of this utility. In no way does the author

warrant its performance or usability. Use it at your own risk. This

utility is to be considered "shareware" and may not be distributed for

profit. This documentation file should be included with the program if

it is to be distributed or transferred to another person.

Acknowledgements:

----------------

This program would not have been possible if not for the helpful advice

of a kind person on GENIE's MIDI RT BBS (Category 6, Topic 20). All I

know about him is his handle: MWILLIAMS; if you know him, send him my

thanks. He explained how the checksums for Roland MIDI System Exclusive

messages works.

Instructions:

------------

In a nutshell this utility will allow you to:

* Change Reverb Mode (Room, Hall, Plate, Tape Delay), Time and Level

* Reassign each part (1-8, Rhythm) to different MIDI channels

* Adjust Fine Tuning, Bender Range, Reverb On/Off, and panning for

each part.

PROBLEMS: On the Rhythm part, only the Bender range function seems to be

adjustable. I am still working on this ( give me a break, I've

only spent less than a day on the whole thing -- the reason

why I'm uploading it this early is to get feedback, and also

because I know you PC owners out there are as starved for

software as I am). Also, the bender range settings might be

changed by some sequencers, although the PAN setting seems to

be fine. The program takes some time to send each command to

the MT-32 as I had to put a delay in there -- there were

glitches when I ran it at full speed. So if you 286/386 users

have any problems, let me know.

To load the program type

MT32UTIL

from the drive and directory where it exists. Make sure that the MT32 is on

and connected to your computer. Also make sure that the UNIT # is set to 17

(see MT32 manual).

In a few seconds you should see a screen that is divided into 11 main areas

(I call them "regions" or "parameter sets"). These regions correspond to

Reverb settings, Channel assignments, and there are 8 regions for each of

the '32's 8 parts and 1 for its rhythm part. All fields are initially blank

as the program does not bother to check the MT32 for its default values.

At this point you may change the value of any parameter on the screen by

using the <PgUp> and <PgDn> keys on your PC's keyboard to select the

region where that parameter is located, using the Up and Down arrows to

select the parameter you want to edit, and using the Left or Right arrows

to decrease or increase the value of the parameter. Every time you hit

the left or right arrows, there will be a short delay and you will here a

beep as the program is telling you that the parameter/value has been sent

to the MT32. You should also see the MIDI message light go on briefly.

Note that you will not here anything from the '32 while the values are

being sent.

The arrow keys decrease or increase values by one. You may, however, use

the comma and period keys on your keyboard to decrease the values by 5

at a time.

So what can you do with it? Well, try assigning parts 1, 2, and 3 to the

same MIDI Channel for increased texture or layering effects. Try the tape

delay effects. Use the Fine Tuning to detune two parts that are on the

same MIDI channel or pan them seperately... etc.

TO QUIT THE PROGRAM TYPE <Esc>.

PARAMETERS -- EXPLAINED :

-----------------------

REVERB:

MODE  - can be ROOM, HALL, PLATe, or tape DeLAY.

TIME  - 0 to 7

LEVEL - 0 to 7

MIDI CHANNELS:

can be 1 to 16

PART PARAMETERS:

FINE TUNING   -  can be from -50 to 50

BENDER RANGE  -  can be from 0 to 12 ('32 actually allows up to 24)

REVERB SWITCH -  can be ON or OFF. If off, then no reverb for that part.

PAN POT       -  can be -7 (far left) to 0 (center) to 7 (far right)