MPU-401/MT-32 SETUP SAVER v1.2 - by John Barlos - June 1989

REQUIREMENTS *****************************************************************

IBM PC/XT/AT or compatible

MPU-401 MIDI interface or compatible

MT-32 Sound Module or compatible

WHAT DOES IT DO? *************************************************************

MTSS.EXE extracts paramters from the following MT-32 addresses:

Timbre temp - Parts 1-8         Patch temp - Parts 1-8

Patch temp  - Rhythm Part       Setup temp - Rhythm Part

System area - ALL

11 exclusive messages in all are requested from the MT-32.

If successfully received, data file will contain 2248 bytes.

File may later be sent to the MT-32 using 'Quick SEND v2.1' to

restore saved settings.

The data received is enough to restore the immediate setup

of the MT-32. The 64 tone patches in memory group are not saved

but easily could be, the author of MTSS.EXE does not find it necessary.

Doing so would increase the size of the data files by 16K.

FILE CREATION ****************************************************************

File written to disk will overwrite an existing file with same

name specification ONLY when user answers "yes" to 'Overwrite (y/n)?'.

If file name specified already exists on disk in the current

\PATH\ and MT-32/MIDI communications fails, the file is deleted

from disk. This will only happen due to USER error (cables not

connected, MT-32 : OFF, etc...).

UPDATES **********************************************************************

***** Revised v1.1 *****

Excepts up to 48 char for x:\path\file name.

User abort successful with keypress while processing.

***** Revised v1.2 *****

User may specify output file name at DOS COMMAND TAIL.

User timer tick vector (int 1CH) installed to time MT-32/MIDI

communications response. If MT-32 does not respond 10 sec. after

1st data request, program is aborted. User may still abort transmission

with keypress.

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

DISCLAIMER:

The author assumes no responsiblity for claimed damages resulting

from usage of MT-32 SETUP SAVER (MTSS.EXE v1.x).

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

If you find MT-32 SETUP SAVER to be of ANY use, a donation of $5.00

or more is requested by the author. Donations assure public release

of further development from the author, and help fight the high cost of

commercial software.

Send $5 donation to:

John Barlos

P.O. Box 1118

Maryland Heights MO.

63043