You can avoid turning your 4019 E off and on in order to reset the printer

to the desired type of emulation with the enclosed simple batch files. You can

also call the batch files from the batch file you use to run an application

program in order to make sure that the printer emulation mode is correctly

chosen before you application program takes over. For example I (almost) always

run Word Perfect 5.1 in HP emulation mode. By adding the DOS 3.3 command

"CALL PRHP" to my WP.BAT file the 4019 will (if turned on) be set to the correct

mode. If the printer is not turned on the DOS Copy command which sends the

Escape-setup stream to the printer gives me the "Abort, Retry.." command until

I turn the printer on and issue the "R(etry)" command. (If I want to run WP5.1

with the 4019 E driver I can shell form WP to Dos and type "PRIBM" ie.

Cntr F1, select DOS command and type PRIBM.BAT).

The batch files PRxx.BAT will switch the datastream mode of the IBM 4019 E

to the xx mode where xx is either IBM ( the PPDS mode ); HP (HP series II

emulation; or PLOT (HP plotter emulation).

Place the *.BAT files in one of the directories in your path statement (e.g.

C:\UTIL ). As written the Batch files assume the three *.2PR files will be

in a subdirectory named "C:\PR". If you put them in another directory edit the

batch files accordingly.

This program was really simple to write. The hard part was finding out what

escape sequence would change my 4019 between its three datastream modes. IBM's

documentation does not even hint that it is possible to switch the modes other

than turning the printer off and on. Such manual resetting often means the

wrong mode in effect when a application runs which can waste a lot of paper

before you can dive to the Start/Stop-Print/Check button.

The escape sequence can be produced from a program named "LAZRDRVR" which is an

apparently unsupported IBM program available on the EXEC-PC under the name

LAZRDRVR.ZIP. If you do not have the program GET IT since in can change modes

and do many other functions. Its only drawback is that it is completly menu

driven (ie you can't have it send a desired command sequence by issuing a

DOS command line call or by calling it from a batch file). The program's output

can be redirected to text file rather than the printer. By simply running the

program 3 time each time issuing the single mode command I desired and then

renaming the output file to *.2PR I produced the 3 files with the correct

escape sequence which when copied to the printer would switch the mode.

While I have no way of testing it (I only have the 4019 E) the batch file will probably work on the

4019 as well. If anyone can verify that it does please let me know.

I would also appreciate hearing from anyone who has bothered to set up the

PC MAG LZSELCT/SETUP (V10N2) program for the 4019 E / 4019 or even the

HP Laser II series. The LZSELECT program does much of what the LAZRDRVR

program does (but only for the HP series II commands not the IBM PDDS and

HP plotter modes). Its main advantage is that its output is in a decimal format

(ie ESC= 27) which allows the output to be used in the PC MAG SETUP2 program

which can be run as a TSR.

I frequent the EXEC-PC BBS in Milwaukee.