���������������������������������������������������������������������Ŀ

�             ASCII Documentation File for ... CAPQUANT               �

���������������������������������������������������������������������Ĵ

�                   A MS-DOS Software System for...                   �

�                                                                     �

�            ***  Quantification of Process Capability  ***           �

�                                                                     �

�           Via the Cumulative Distribution of Regret Indices         �

���������������������������������������������������������������������Ĵ

�                  A Quality Assurance Training Tool:                 �

�              Statistics Committee of the QA Section of              �

�            the Pharmaceutical Manufacturers Association             �

�                                                                     �

�                  Bob Obenchain, Research Scientist                  �

�     Statistical and Mathematical Services, Eli Lilly and Company    �

�����������������������������������������������������������������������

Any type of quantitative (variables) data measurement can be used as a

basis for Process Capability Quantification via a three-stage analysis

process:

(i) Convert each observed numeric value into a REGRET, which is a

measure of long-range economic impact or customer/regulatory

dissatisfaction that results from product deviations from an

intended TARGET value (or values.)

CapQuant allows the user to pick the regret function from

any of 9 general families and to specify the target value.

(ii) Smooth and Standardize the observed distribution of historical

regret values (via POISSONIZATION) to form a REGRET INDEX.

CapQuant determines an EQUIVALENT NONCONFORMANCE value for

each regret and the corresponding EQUIVALENT EXPECTANCY,

the expected regret within historical capability levels.

The corresponding INDEX is simply the ratio of observed

to expected regret.  An index value of 0 is ideal, while

an index value of 1 is the historical quality standard.

(iii) Generate PROCESS CAPABILITY PLOTS ...medium resolution CGA

graphical displays (320 x 200 pixels) of the Cumulative

Probability Distribution Function of the Regret Index.

CapQuant can plot the cumulative distribution function for

process history consisting of as many as 2500 observations.

Both the observed (raw) and Poisson smoothed forms are

displayed.  CapQuant also allows direct graphical

comparisons between "new" sample results (most recent

history) with the that portion of older history deemed most

relevant to current operations.

CapQuant software provides the computational tools needed to implement

specific analyses of the above type.  CapQuant creates an ASCII output

file of extremely detailed results that, following minor manual editing,

provides an ideal data input file to statistical and/or presentation

graphics systems, such as SAS.  But only a small fraction of CapQuant's

outputs (those which are displayed on your computer screen as well as

written to the ASCII output file) are needed to characterize the

(historical) capability of a manufacturing process.

TABLE-OF-CONTENTS

Usage of the CapQuant Pop-Up Menus . . . . . . . . . . . . . . . . . 2

REGRET FUNCTIONS: Quantify Impact of Deviations from Target. . . . .10

Equivalent Nonconformance and Expectancy . . . . . . . . . . . . . .15

References . . . . . . . . . . . . . . . . . . . . . . . . . . . . .19

Appendix: Technical Descriptions of the Poissonization Process . . .20

�����������������������������Ŀ

�  *** USAGE of CAPQUANT ***  �

�������������������������������

The CapQuant software system is invoked from MS-DOS by entering its

8-character name, CAPQUANT, at your DOS prompt...

Prompt> capquant

CapQuant does not expect or accept any "command-line" arguments.

Instead, CapQuant will prompt you to provide all information it needs to

perform analyses.  And each such prompt usually displays a [default value]

in square brackets that you may accept by simply pressing the ENTER key.

CapQuant Input/Output File Naming Conventions...

------------------------------------------------

The ASCII (text) files used by CapQuant for data input and output

consist of a filename (of at most eight characters) specified by the

user followed by a period (.) and one of four specific 3-character

filename EXTensions.

BILINEAR.HST  ...Batch Input of Historical Data (at most 2500 obs.)

BILINEAR.NEW  ...Batch Input of New Data (at most 250 observations)

BILINEAR.OUT  ...Batch Input of Historical Data (at most 2500 obs.)

BILINEAR.XXX  ...CapQuant Output  for Automatic REGret Calculations.

BILINEAR.REG  ...Edited .XXX File for Automatic REGret Calculations.

Example CapQuant Walk-Through...

--------------------------------

The CapQuant start-up screen will appear as follows...

����������������������  Capability Quantification �����������������ͻ

�                                                                   �

�                      CAPQUANT.EXE...Version 9010                  �

�                                                                   �

�                   A Quality Assurance Training Tool:              �

�           Statistics Committee of the QA Section of the PMA       �

�                                                                   �

� ���������������������������������������������������������������ͻ �

� �  Will Process HISTORY be Input via K = Keyboard ? ...or       � �

� �                                    B = Batch File ?           � �

� �                                                               � �

� �  Press the   K   or   B   key now -->                         � �

� ���������������������������������������������������������������ͼ �

�������������������������������������������������������������������ͼ

If you select Keyboard input, you will be prompted to type in all

historical data values using your keyboard.  But, in this case, CapQuant

will also be creating a "Batch Input" file that you could use in future

CapQuant runs to repeat and/or modify your initial analyses.

On the other hand, if you are familiar with a "screen oriented" personal

computer (ASCII) text editor, you might prefer to start by creating your

own "Batch Input" file.  This strategy can give you more flexability in

data entry and validation than is possible inside of CapQuant.  Simply

mimick the format of an example CapQuant Batch Input file.  For example,

the Batch Input file could consist for the following 7 records (rows),

each starting in column 1:

1        ...Number of Data Values per Observation

THICKNES ...Name of Variable ONE

1.09

2

13

4.7

3

q

where the letter Q at the start of the last record signals the "end of

data entry."

Once you respond to the initial CapQuant prompt by pressing either the K

or the B key, the "Window" will clear itself, and you will be prompted

to specify a MS-DOS filename, consisting of at most 8 characters.

If you have selected Keyboard input, this filename will be used to

capture the data you will be entering.  Or, as shown below, that

filename (with a MS-DOS filename extension of ".HST") will be read in as

Batch input.  For either Keyboard or Batch input, the CapQuant Output

file can have either the same filename or a different filename (8

characters maximum.) But the 3-character MS-DOS filename EXTension will

always be ".OUT".

�������������������  Capability Quantification ������������������ͻ

�                                                                 �

�  Batch File Input Selected...                                   �

�                                                                 �

�  At colon Prompts : ...simply press ENTER to get the [default]. �

�  -------------------------------------------------------------- �

�                                                                 �

�  Specify filename for Batch Input [capquant] : batfname         �

�  The Batch Input file is to be: batfname.hst                    �

�                                                                 �

�  Specify filename for Detailed Output [batfname.out] :          �

�  The CAPQUANT Output Save file is to be: batfname.out           �

�                                                                 �

�          Press Q to QUIT now...Other Key to Continue...         �

�����������������������������������������������������������������ͼ

A [Black on Green] window for Parameter Specification appears next...

��������������������    CapQuant  Parameters    �����������������ͻ

�                                                                 �

�  Does data consist of Single Values or Pairs (1 or 2) :         �

�  Number of Data Values per Observation =  1.                    �

�                                                                 �

�  What is the NAME of Variable ONE ? [variable1] :               �

�  First  Variable Name: DISSOLUT                                 �

�                                                                 �

�  Historical Data, in any sequence, are to be entered            �

�  in the Next Window...  Press Q to QUIT now...                  �

�����������������������������������������������������������������ͼ

A [Blue on Cyan] Data Entry window now appears.  If you are using

Keyboard input, CapQuant will continue prompting for either single

values or data pairs until you ENTER the letter Q to signal an end to

data entry.  The number of observations entered will then be

displayed...

����������������������   Data Entry Window    �������������������ͻ

�                                                                 �

�        List Historical Data in Output File? [y|n] : n           �

�                                                                 �

�        Number of Historical Observations =  264                 �

�                                                                 �

�        Data Entry Complete...                                   �

�        Press Q to QUIT now...Other Key to Continue...           �

�����������������������������������������������������������������ͼ

Small windows will now Pop-Up within the [Blue on Cyan] Large Window.

You first selection from the left-hand "Process Capability MENU" should

always be "R".  (...Unless you wish to "eXit" without making any

calculations!)

You first selection from the middle "REGRET Analysis MENU" should always be

"S".  (Unless you wish to Return without making any calculations!)

Then you choose a Regret-Function-Family from the right-hand "REGRET

Function MENU."  (Unless you wish to Abandon/Quit without making

any calculations!)

�����������������������������ͻ

�  Process Capability MENU    �

�                             �

�     R = Regret Analysis     �

�                             �           �������������������������ͻ

�     C = Capability Plot     �           � REGRET Function MENU    �

�             ���������������������������ͺ                         �

�     M = Moni�     REGRET Analysis MENU  � 1...ATTRIBUTES          �

�             �                           � 2...QUADRATIC           �

�     X = eXit� S = Specify Regret Functio� 3...ABSVALUE            �

�             �                           � 4...1-SIDED QUADRATIC   �

� Choice --> R� I = Regret INDEX Computati� 5...BILINEAR            �

�             �                           � 6...END-POINT QUADRATIC �

�������������ͺ L = Poisson Lack-of-Fit Te� 7...BOUNDED MAXIMUM     �

�                           � 8...RANGEMAX            �

� A = Automatic Regret Calcs� 9...NONCONFORM/EXPTANCY �

�                           � 0...Abandon/Quit        �

� R = Return to Main Menu   �                         �

�                           � Choice -->              �

� Choice --> S              �                         �

�                           �������������������������ͼ

������������������������������ͼ

The regret functions which can be defined in CapQuant belong to one of

nine general families.

1) attributes regret, which is zero inside an interval and one

outside that same interval,

2) quadratic regret, where the regret is the square of the deviation

of the measurement from its target value,

3) linear regret, in which the economic impact of a measurement is

directly proportional to the measurement's value,

4) one-sided regret, which is quadratic on one side of a target

and zero on the other side of that target value,

5) opportunity regret, in which the economic impact of a demand

for a perishable product is measured relative to the

inventory on hand,

6) end-point quadratic regret, in which the minimum and maximum

observed values in samples of fixed size are monitored, and

regret is the sum of the quadratic regret at the two end-points,

7) bounded max regret, in which regret follows a double-logit

pattern that is V-shaped in its center (at the target) but

flat like attribute regret in its tails, and

8) range (linear) regret, in which the minimum and maximum

observed values in samples of fixed size are monitored, and

regret is the difference, maximum minus minimum.

9) an unspecified pattern called "nonconformance-expectancy."

In this case, the data are assumed to have been "Poisson"ized

outside of CapQuant, but you can still use CapQuant to make

Capability Plots and/or to test Lack-of-Fit.

See the section of this CapQuant documentation, starting on page 10,

that discusses REGRET FUNCTIONS for detailed information on these nine

families.

The numerical example used here is based on...

Regret Type:    ONESIDED

TARGET = Value corresponding to ZERO REGRET = 100.00

PARM1  =   -1  ... ZERO REGRET to LEFT of TARGET.

Once you have specified a functional form for your Regret Function and

have selected values for its Parameters (TARGET, PARM1, PARM2), the

"REGRET Analysis MENU" will re-appear.  One's second selection from this

menu is usually the second item, "I"...

������������������������������ͻ

�     REGRET Analysis MENU     �

� S = Specify Regret Function  �

� I = Regret INDEX Computations�

� L = Poisson Lack-of-Fit Test �

� A = Automatic Regret Calcs   �

� R = Return to Main Menu      �

� Choice --> S                 �

������������������������������ͼ

������������������   Regret Calculations  ���������������ͻ

�                                                         �

� Observations (Periods) = 264                            �

� Regret Type   =  4                                      �

�                                                         �

� Observed Average (Expected) Regret =   51.608           �

�                                                         �

� Observed Std. Error of Regret =  88.6406                �

�                                                         �

� Corresponding Equivalent Expectancy = 0.3390            �

�                                                         �

� Enter Objective Expected Regret [   1.000] : 50         �

�         using...   50.000                               �

�                                                         �

� Enter Objective Std. Error [  85.879] : 100             �

�         using...  100.000                               �

�                                                         �

� Equivalent Expectancy per Observation =   0.2500        �

�                                                         �

� End of Regret & Equivalent Nonconformance Calculations. �

���������������������������������������������������������ͼ

As in the above example, the "Objective Expected Regret" value is

usually taken to be a convenient, "rounded" value (50, here) that is

fairly close to the observed, sample value (51.6).  The initial default

value for Expected Regret will always be [ 1.000].  But, if you select

item "I" on the "REGRET Analysis MENU" more than once, the subsequent

default value will be the value you entered in the previous iteration.

The default value [85.879] for the "Objective Std. Error," is the

numerical value that (given an objective expected regret of 50 rather

than the observed 51.6) would equate the objective "Equivalent

Expectancy per Observation" to the observed value of 0.3390.  This

default value will not, necessarily, also equate the mean and variance

of the rescaled regret distribution...unless the objective and observed

expected regret values are exactly equal.

In the above example, a convenient, "rounded" value (100) is also

entered for the "Objective Std. Error,"  and the resulting "Equivalent

Expectancy per Observation" is computed by CapQuant to be 0.25.  Later,

if you use item "L" to test Lack-of-Fit of rescaled regrets to the

Poisson distribution, the Poisson "intensity" parameter ( = mean =

variance ) will be the "Equivalent Expectancy per Observation" value

(0.25, here.)

The "REGRET Analysis MENU" will re-appear again.  As implied above, your

selection might again be the second item, "I".  For example, you would

re-run "I" if you were unhappy with the "Equivalent Expectancy per

Observation" ( .25 above ) that resulted from your two choices...for

"Objective Expected Regret" and "Objective Std. Error" (50 and 100,

respectively.)  See the section of this documentation, starting on page

14, that discusses Equivalent Nonconformances and Expectancy for

detailed information on this CRITICAL phase of the analysis.

Here... EXPREGRET objective =    50.0 ...observed =   51.6075

VARREGRET objective = 10000.0 ...observed = 7857.1484

The third item ("L") on the "REGRET Analysis MENU" provides optional

(rather than mandatory) calculations.  But statistically significant

Lack-of-Fit revealed via this "L" item can suggest the need to change

one's regret mean and variance objectives ( item "I" ) if not the very

form of that regret ( item "S" ).

NOTES:  Always re-run item "I" after each change you make to the regret

function via item "S".  And never select item "L" except after item

"I"...

������������������������������ͻ

�     REGRET Analysis MENU     �

� S = Specify Regret Function  �

� I = Regret INDEX Computations�

� L = Poisson Lack-of-Fit Test �

� A = Automatic Regret Calcs   �

� R = Return to Main Menu      �

� Choice --> L                 �

������������������������������ͼ

�����������������Poisson Lack-of-Fit Test����������������ͻ

�                                                         �

� Kolmogorov-Smirnov Statistic =   0.03938                �

� 5% Critical Value =  0.08                               �

� 1% Critical Value =  0.10                               �

� Distribution is not significantly                       �

� different from the fitted Poisson.                      �

�                                                         �

� Value, Frequency, CumFreq, and Poisson Fit...           �

�    0   0.81818   0.81818   0.77880                      �

�    1   0.14773   0.96591   0.97350                      �

�    2   0.03030   0.99621   0.99784                      �

�    3   0.00379   1.00000   0.99987                      �

�    4   0.00000   1.00000   0.99999                      �

���������������������������������������������������������ͼ

NOTE: All "Equivalent Nonconformance" values were rounded to the nearest

full integer value for the purpose of the above Lack-of-Fit test because

Poisson random variables take on only integer values.  But this rounding

really represents an unnecessary loss-of-precision in most quality

monitoring applications.  Thus CapQuant does not round nonconformance

numbers to integers before invoking the CPQ algorithm.  In fact,

CapQuant lists nonconformance/expectancy results to 2 decimal places in

the ASCII output file.

Let us suppose that you now select item "R" on the REGRET MENU followed by

item "C" on the Main MENU...

�����������������������������ͻ

�  Process Capability MENU    �

�     R = Regret Analysis     �

�     C = Capability Plot     �

�     M = Monitor Capability  �  <--- 2 nd Selection

�     X = eXit CAPQUANT       ����������������ͻ

� Choice --> C                �ysis MENU       �

�����������������������������ͼoss Function    �

� A = Automatic Regret Calcs  �

1 st Selection --->  � R = Return to Main Menu     �

� Choice --> R                �

�����������������������������ͼ

��������������������� CPQ Trend Calculations �������������������ͻ

�                                                                �

� Observations (Periods) = 264                                   �

�                                                                �

� View Listing of Results Now? [y|n] : y                         �

����������������������������������������������������������������ͼ

Graphics HELP Screen...

�����������������������������������������Ŀ

�  This HELP screen .... Space Bar or F1. �

�                                         �

�  ERASE PLOT......press ENTER or ESCape. �

�    Note: 2nd KeyPress sometimes needed. �

�                                         �

�  On-Screen Cursor & VALUES ... press V. �

�  Move Right/Left with ArrowKeys by 0.01 �

�             or... Tab/BackSpace by 0.25 �

�                                         �

�  Screen PRINT/SAVE Keys:  First Press S �

�  and then press a Second Special Key... �

�   L or F10 dumps to HP LaserJet/DeskJet �

�   I or F9  dumps to IBM Graphics Matrix �

�   E or F8  dumps to Epson FX, JX, or LQ �

�   O or F7  dumps to Okidata Microline   �

�   S saves screen to disk in PCX format. �

�                                         �

�  CapQuant, ver9010.......Press any key. �

�������������������������������������������

One iteration through the three-stages of quality monitoring

calculations is now complete.  You now have the options of either

pressing "L" to return to the REGRET Menu for further calculations or "X"

to exit CapQuant and return to DOS.  (It would be an absolute waste to

repeat CPQ Trend Calculations, item "C", without first changing either

the regret function or the objective regret mean or variance.)

�����������������������������ͻ

�  Process Capability MENU    �

�     R = Regret Analysis     �

�     C = Capability Plot     �

�     M = Monitor Capability  �

�     X = eXit CAPQUANT       �

� Choice --> M                �

�����������������������������ͼ

��������������������    Capability Monitoring   ������������������������ͻ

�                                                                        �

�  Display Historical Capability in R = Raw     ...or                    �

�                                   S = Smoothed Form ?                  �

�                                                                        �

�  Press the   R   or   S   key now -->  S                               �

�                                                                        �

�  Will NEW Process Data be Input via K = Keyboard ? ...or               �

�                                     B = Batch File ?                   �

�                                                                        �

�  Press the   K   or   B   key now -->  B                               �

�                                                                        �

�  Batch Input of New Data Selected...                                   �

�                                                                        �

�  Specify filename for New Data Batch Input [batfname] :                �

�                                                                        �

�  The New Data Batch Input file is to be: batfname.new                  �

�                                                                        �

�  Pause at each New Data Value [Y|n] :                                  �

������������������������������������������������������������������������ͼ

When you do eXit from CapQuant, you screen will display...

�����������������������������ͻ

�  Process Capability MENU    �

�     R = Regret Analysis     �

�     C = Capability Plot     �

�     M = Monitor Capability  �

�     X = eXit CAPQUANT       �

� Choice --> X                �

�����������������������������ͼ

REMINDER(S):

CAPQUANT created an Output File named... batfname.out

Use the DOS invocation... LIST batfname.out

to review detailed computational results from

this CAPQUANT run.

-----------------------------------------------------------------------

REGRET FUNCTIONS

We need a rule for transforming every possible value for the variable

into its corresponding regret value.  In other words, we need to specify a

"functional relationship."  The variable may assume negative values, but

the corresponding regret must be zero or positive.  This usually means

that the regret function is "non-linear" and that more than one value for

the variable can yield the same value for the regret (i.e. regret may be a

non-invertible function.)  All that is really necessary is that the

regret

function provide some sort of common-sense "proxy" for the economic

impact of deviations from intended target value(s).

In CapQuant, you may select a Regret Function from any of nine general

families:

ATTRIBUT        QUADRATC        ABSVALUE

ONESIDED        BILINEAR        ENDPOINT

BOUNDMAX        RANGEMAX        NONCEXPT

Each of these Regret Functions can have as many as three Parameters:

TARGET (the value of the variable resulting to Zero-Regret), PARM1,

and PARM2.

REGRET Type:    ATTRIBUT

Description:    Economic Impact is either 0 (negligible) or 1

(complete).  A variable measurement outside of an

acceptable range indicates a nonconformance (defect.)

Variables Measurement -->

|                   |

Regret = 1  ************|                   |*************

|                   |

|                   |

|                   |

Regret = 0  ------------*********************------------

|                   |

Lower               Upper

Bound               Bound

Parameters:     No single TARGET value is specified.  Regret is zero

over the interval from  PARM1 = Lower Bound

to    PARM2 = Upper Bound.

REGRET Type:    QUADRATC         (Taguchi's "Loss-to-Society")

Description:    Economic Impact is assumed to be proportional

to the SQUARE of the deviation of the measured

value from its (zero nonconformance) TARGET value.

*      Variables Measurement -->  *

|

*              |              *

*             |             *

**           |           **

**         |         **

***      |      ***

***   |   ***

Regret = 0  ------------------*******---------------------

|

TARGET

REGRET = ( Value - TARGET )^2

Parameters:     TARGET = Value corresponding to ZERO REGRET.

No PARM1 or PARM2 values need be specified.

-------------------------------------------------------------------

REGRET Type:    ABSVALUE    (...symmetric BILINEAR )

Description:    Economic Impact is assumed to be directly

proportional to the measured value.

|    Variables Measurement -->

***                         |                         ***

***                      |                      ***

***                   |                   ***

***                |                ***

***             |             ***

***          |          ***

***       |       ***

***    |    ***

*** | ***

Regret = 0 -------------------***------------------------------

|

TARGET = 0

Parameters:     No TARGET, PARM1 or PARM2 values need be specified.

Restrictions:   Variable measurements must be non-negative.

TARGET is implicitly ZERO.

REGRET Type:    ONESIDED

Description:    Economic Impact is ZERO on one side of the TARGET

and proportional to the SQUARE of the deviation

from TARGET on the other side.

Variables Measurement -->

*               |

|

*              |       PARM1 = +1

*             |

**           |       Zero Regret on

**         |       Right Side.

***      |

***   |

Regret = 0  ------------------**********************************

|

TARGET

Parameters:     PARM1 is a LEFT(-1) or RIGHT(+1) indicator.

PARM1 = -1 indicates zero regret left of TARGET.

No PARM2 value need be specified.

-----------------------------------------------------------------

REGRET Type:    BILINEAR  (...asymmetric ABSVALUE)

Description:    Two variables measurements are required, a DEMAND

level & an INVENTORY level for a perishable product.

Economic Impact is proportional to production cost per

unit when inventory > demand, but there is a Profit

(Opportunity) Loss per unit when demand > inventory.

****            Demand -->

****                                        **

****                                  **

****                            **

****               |      **

****           |    **

****       |  **

****   |**

Regret = 0  ------------------------****-----------------------

|

Inventory

Level

Parameters:     PARM1 = Regret per Unit of Excess Inventory.

PARM2 = Regret per Unit of Inventory Shortage.

No TARGET value need be specified.

REGRET Type:    ENDPOINT (quadratic)

Description:    Two variables measurements are required, the MINIMUM

and MAXIMUM observed values in a random sample of fixed

size (say, 10 measurements.)  Economic Impact is assumed

to be the SUM of the QUADRATC regrets at the min. and max.

values.

*---------------|               *

|*              |              *

| **            |            **

|   **          |          **

|     ***       |-------***

|        ***    |    ***|

Regret = 0  -----+-----------*********---+----------------

MIN              |      MAX

|            TARGET     |

REGRET = ( MIN - TARGET )^2 + ( MAX - TARGET )^2

Parameters:     TARGET = Value corresponding to ZERO REGRET.

No PARM1 or PARM2 values need be specified.

---------------------------------------------------------------------

REGRET Type:    BOUNDMAX

Description:    Economic Impact follows a double-logit pattern that

is V-shaped at the target but flat like ATTRIBUT

absolute DEVIATIONS of values from the TARGET.

Measurement-->

PARM2     ------------|---------|---------|------------

*        |         |         |        *

*    |         |         |    *

* |         |         | *

|*        |        *|

|  *      |      *  |

|    *    |    *    |

|      *  |  *      |

|       * | *       |

|        *|*        |

Regret = 0  ----------------------*----------------------

|      Target       |

Target-PARM1        Target+PARM1

REGRET = ( PARM2 * DEVIATION ) / ( PARM1 + DEVIATION )

Parameters:     TARGET = Value corresponding to ZERO REGRET.

PARM1  = DEVIATION from TARGET with regret = half MAX.

PARM2  = MAXIMUM possible regret.

REGRET Type:    RANGEMAX   (...linear in Max - Min )

Description:    Two variables measurements are required, the

MINIMUM and MAXIMUM observed values in a random

sample of fixed size (say, 10 measurements.)

Economic Impact is assumed to be proportional to the

RANGE from the min. to the max. values.

|    Range = ( Max. - Min. ) -->

|

|                          ***

|                       ***

|                    ***

|                 ***

|              ***

|           ***

|        ***

|     ***

|  ***

Regret = 0  ---------***------------------------------

|

TARGET = 0

Parameters:     No TARGET, PARM1 or PARM2 values need be specified.

Restrictions:   Range must be non-negative.  TARGET is implicitly ZERO.

---------------------------------------------------------------------

REGRET Type:    NONCEXPT

Description:    Two variables measurements are required, the

EQUIVALENT NONCONFORMANCES and the EQUIVALENT

EXPECTANCY for each observation (production lot,

period, etc.)

With input data of this type, no Regret Function is

actually specified.  Similarly, no Regret Mean or

Variance are needed to Poissonize the regret.

Parameters:     No TARGET, PARM1 or PARM2 values need be specified.

-------------------------------------------------------------------------

Equivalent Nonconformance & Expectancy

The process of converting Regret into "Equivalent Nonconformances" and

"Equivalent Expectancy" is called "Poissonization"...

REGRET = Long Range Economic Impact and/or Customer/Regulatory

Dissatisfaction associated with Deviations from intended

Target Value(s).

EXPREGRET = Expected Value of REGRET within the Historical Distribution

of Process Capability.

VARREGRET = Variance of REGRET within the Historical Distribution of

Process Capability.

EN = Equivalent Nonconformances = REGRET * EXPREGRET / VARREGRET.

EE = Equivalent Expectancy = EXPREGRET * EXPREGRET / VARREGRET.

Note, in particular, that this conversion process involves nothing more

than a simple "Change-of-Scale."  REGRET is multiplied by the EXPREGRET

/ VARREGRET ratio to compute Equivalent Nonconformances, EN.

Multiplication by this constant value assures that the mean and variance

of EN are equal (to EE = Equivalent Expectancy.)  This equality of mean

and variance is an essential characteristic of Poisson distributions.

The two key parameters needed to establish a successful application of

variables data to regret quantification are thus the expected value

(EXPREGRET) and the variance of the regret (VARREGRET) at standard quality.

Many different shapes for regret functions might be appropriate for any

particular variable in any particular production process.  The exact

form of the regret function chosen is sometimes not terribly important.

As a result, even after the regret function and all of its describing

parameters (TARGET, PARM1, and PARM2) have been specified, the critical

final phase of customizing a CPQ application still remains...choice of

numerical values for EXPREGRET and VARREGRET.

While it might be possible to specify the distribution of regret at

standard quality using only technical assumptions and theoretical

calculations, the more standard approach is an empirical one in which

actual data on the operation of the process are converted to regrets,

and the statistical distribution of those sample regrets is then studied.

If one has data on only a dozen or so lots, it could be misleading to

use the observed sample values of the mean and variance of regret as

EXPREGRET and VARREGRET, respectively, because small sample estimates

are imprecise.

On the other hand, even if one had data for hundreds of lots, it could

still be misleading to use the observed sample mean and variance of

regret to establish CapQuant parameter values because even large sample

results can be biased.  There could be several different causes of bias.

The data at hand may not represent the results of the process operating

at (or even near) an industry-wide "standard" quality level.  The

history being used might, in fact, reflect an operation in which

quality is either well above or well below standard.  Modern, new

manufacturing equipment (using, say, robotics) or a change in ingredient

formulations may have been introduced without changing the quality

standard that was barely within the capability of the old technology.

If you aren't an expert on the operation of the manufacturing process in

question, consult with an expert.  Develop opinions about the quality

level of the process as reflected in its history.  Ask yourself, "How

did I react to these quality monitoring results when I first saw them?"

(Or, "How would I have reacted if I had been the manager in charge at

that time?")  Were you ever concerned that the process might be out of

"statistical control?"  Did you ever become alarmed because rejection

rates jumped at final inspection or because in-process yield dropped?

Might you have reacted quite differently if the data had been only

slightly different?  Or, would your reactions have been basically the

same even if the data had shifted even more than it did in level and/or

were even more highly variable from lot-to-lot?

A common situation is that all the observed history reflects data taken

from the process when it was operating well within its current process

capability.  In other words, the variable could have deviated much more

from its target without management having been alarmed at all!  This

means that the values for EXPREGRET and VARREGRET that should be specified

to CapQuant will indeed differ from what is observed in the history.

General rules-of-thumb for this well-within-capability situation are...

...the EXPREGRET parameter should usually be chosen to be rather close

to the observed value of the sample mean regret if the history that

is available consists of 50 or more observations.

...the VARREGRET parameter, on the other hand, might be selected to be

10 or even 100 times larger than the observed regret variance of the

available data. (Note: This corresponds to a "Standard Error"

value between 3 and 10 times larger than that actually observed.)

There are two primary reasons for using a rather large VARREGRET value.

Again, the history data may be very well behaved, and management would

not have been alarmed even if the deviations of measurements from the

target had been much larger than observed.  The second reason for using

a large variance is that variables data are very rich in information.

(Variables data are usually much more informative than nonconformance

data.)

��������������������������������������������������������������Ŀ

� NOTE:  CapQuant does not actually accumulate results across  �

� variables, across products or over time to form either       �

� composite statistics or reporting-period summaries.  But, by �

� using one or more CapQuant output files, you can accumulate  �

� results any way you wish.  Because of "Poissonization", the  �

� composite Equivalent Nonconformances and Expectancies are    �

� both simply the respective SUMs over all relevant parts.     �

����������������������������������������������������������������

The CapQuant philosophy is to fine-tune parameter values by making

repeated "iterations" as follows:

(i)   The general form and specific parameter values of the Regret

Function are set using item "S" on the Regret Menu.

(ii)  Using item "I" on the Regret Menu, the sample mean and variance

of regret are displayed and you are prompted to enter values

for EXPREGRET and VARREGRET.  As explained above, you may wish to

consider using EXPREGRET and VARREGRET values that are not very

close, numerically, to the values in your sample.

(iii) Goodness-of-Fit of sample results to a Poisson distribution

(with "intensity" EXPREGRET*EXPREGRET/VARREGRET) can then be checked

using a Kolmogorov-Smirnov test (item "L" on the Regret Menu.)

Statistically significant Lack-of-Fit is NOT fatal.  On the

other hand, insignificance of distributional differences is

always a welcome indication that your choices are supported

by the available data.  [Of course, you could return to step

(ii) now to see if different values of EXPREGRET and VARREGRET

reduce Lack-of-Fit.]

(iv)  Make a detailed Process Capability plot using item "C" on the

Main MENU.  Study these results, using the on-screen cursor

activated by the "V" key, and determine whether your results

are consistent with the opinions of subject matter experts.

(v)   If you are unhappy with the outcome of step (iv), you should

either (1st choice)

...return to step (ii) and try out different values of

EXPREGRET and VARREGRET

or (2nd choice)

...return to step (i) and try different regret function

parameters or even a different regret function family.

Once you are happy with the outcome of step (iv), the modeling

process is complete.

REFERENCES

CONOVER, W. J. (1980). Practical Nonparametric Statistics, 2nd. Ed.,

Wiley [Kolmogorov-Smirnov table and approximation, page 462.]

HOADLEY, Bruce. (1981). "The Quality Measurement Plan, QMP."  Bell

System Technical Journal, 60, 215-273.

HOADLEY, Bruce. (1986). "QUALITY MEASUREMENT PLAN(QMP)."  Encyclopedia

of Statistical Sciences. (Kotz, Johnson and Read, Editors)

Volume 7, pages 393-398.  New York, John Wiley.

INTRODUCTION TO QUALITY TREND CHARTS.  Bell Communications Research,

Inc., January 1986 (18 page booklet.)

LINK, P. A., and RIPACKI, B. R., "Quality Assurance Approach to Fiber

Optic Cable," International Wire and Cable Symposium

Proceedings, 1985.

OBENCHAIN, Robert. "Regret Indices and the Quantification of Process

Capability."  Submitted to Technometrics. 1990.

QUALITY MEASUREMENT PLAN (QMP).  Bell Communications Research Technical

Reference, TR-TSY-000438.  Issue 1, April 1987.

-----------------------------------------------------------------------

APPENDIX ONE

Poissonization:  Conversion of Economic Impact Measures into

Equivalent Nonconformances and Equivalent Expectancy.

Section A.1    Nonconformance Expectancy

Attributes Data

Definition:     Nonconformance Expectancy is the product of the Sample

Size times the Quality Standard (expressed in expected

nonconformances per unit.)

Example:        When the quality standard is 0.004 nonconformances per

unit, one needs to inspect 250 units in order to expect

to see 1 nonconformance when quality is at the standard

level.

Information

Content:        Under a Binomial model, the "moments" of the sampling

distribution would be given by the familiar expressions:

mean = n*p and variance = n*p*(1-p).  QMP (the Quality

Measurement Plan) is based upon the Poisson (maximum

variance) model with mean = variance = n*p, which is the

nonconformance expectancy.  Expectancy is the primary

measure of "information content" used in QMP and CPQ for

sampling inspection data. [See Section A.3, below, for

details on information content.]

Variables Data

Definition:     Equivalent nonconformances and equivalent expectancy can

be defined by "Poissonizing" any continuous measure of

regret, R.  [See Section A.2, below, for details on

poissonization.]

Reporting       Experience with use of QMP at both Bell Communications

Period:         Research (Bellcore) and at AT&T Bell Laboratories

suggests the following "Rule-of-Thumb" for aggregating

sampling inspection data over "reporting periods"

(usually either the 12 calendar months or 8-per-year):

�������������������������������������������������������������Ŀ

� An effective quality monitoring program requires sufficient �

� sampling inspection to produce a nonconformance expectancy  �

� of at least 2 nonconformances per reporting period.         �

���������������������������������������������������������������

If production is so low or nonconformances are so rare

that one would have to inspect more than 20% of total

production to achieve a nonconformance expectancy of at

least 2 nonconformances per reporting period, then

"quantitative" quality monitoring is probably

impractical.  (After all, QMP is the "most powerful"

statistical method currently available; if QMP can't get

the monitoring job done, what method can?)

Section A.2    Equivalent Nonconformances & Expectancy

Definition:     Equivalent nonconformances and expectancy can be defined

by "Poissonizing" a continuous measure of regret, R.

For example, if E is the expected value of R and V is

the variance of R, then the number of "equivalent

nonconformances" is said to be R*E/V, which is usually

not an integer value.  But the mean and variance of

R*E/V are both equal to the "equivalent expectancy" of

E*E/V...and equality of mean and variance is an

intrinsic property of all Poisson distributions.

Example:        Suppose a measurement produces a continuous variable, Z,

that has (approximately) a "normal" distribution with

mean = target value = T and variance = D, say, when

quality is "at standard."  Consider the "Taguchi-like"

measure of regret given by the sum-of-squares of N

deviations of independent Z-values from the target, T,

all divided by D.  Then normal distribution theory

implies that E(R) = N and V(R) = 2N.  It follows that

the number of "equivalent nonconformances" is R/2, and

the "equivalent expectancy" is N/2.  (Note that the

sample index value = nonconformances/expectancy is R/N;

the average squared deviation from target,

scaled by dividing by D.)

"Power" of

Variables

Data:           Variables data can be incredibly "rich" in sample

information about quality compared with attributes data.

The above "normal theory" example shows that a single

continuous observation can yield a nonconformance

expectancy of one-half!  And the "uniform" distribution

would yield an even higher nonconformance expectancy of

.66 per observation with the above "Taguchi-like" R

measure!

Do-It-Right

from the

START:          Quality engineers should consult a professional

statistician if there is any doubt in their minds about

how to implement the above basic concepts.  One needs to

utilize a realistic "Regret Function," which might need

to be one-sided (rather than symmetric about the target)

or linear or saturating (rather than always

quadratically increasing.)  The calculation of the mean

and variance of the regret depends upon the statistical

distribution of test results when the manufacturing

process is within its historical "process capability."

Theoretical calculations can be complicated or require

numerical integration; empirical studies of historical

results are generally more straight-forward and more

satisfactory.

Section A.3    Expectancy and Information

Contents of This Section:

Two fundamental concepts are established by the highly

technical arguments of this section.

...The information content in a set of regret measurements

is directly proportional to the Poissonized

"nonconformance expectancy" of the regrets at standard

process capability.

...The sample regret index (ratio of observed to

expected nonconformances) is the minimum variance,

unbiased estimate of the true index.

Minimum Variance Bound:

If a sample statistic, tau, is an unbiased estimate of a

parameter, theta, of the statistical distribution of the

observed data vector, x, then the so-called "Cramer-Rao-

Aitken-Silverstone Inequality" states that:

��������������������������������������������������Ŀ

�  the product of the variance of tau times the    �

�                     ---------------              �

�  Fisherian Information in the data must be >= 1, �

�  --------- -----------                           �

����������������������������������������������������

where this "information" is defined to be the expected

value of the square of the derivative of the log-

likelihood function of the data with respect to the

parameter theta.  Furthermore, equality holds if and

only if the derivative of the log-likelihood with

respect to theta is of the form:

A(theta) * (tau-theta),

in which case, tau is the minimum variance estimate of

theta, and A(theta) is the Fisherian Information.

Poisson Example:

If the observed numbers of nonconformances, { x(1),...,

x(n) }, are stochastically independent and each follows a

Poisson distribution with intensity parameter lambda(i),

then the total number of observed nonconformances is

also Poisson distributed with intensity parameter equal

to the sum of the individual lambdas.  This total

intensity can be re-expressed as:

lambda(1)+...+lambda(n) = (nonconformance expectancy) * theta

where theta is the true regret index (total lambda

divided by the "standard" nonconformance intensity) and

expectancy is n times the standard intensity.  As a

result the log-likelihood function is

[ x(1)+...+x(n) ] * log(theta) - expectancy * theta

+ terms that do not depend on theta.

The theta-derivative of the log-likelihood is thus:

���������������������������������������������Ŀ

�  expectancy     x(1)+...+x(n)               �

�  ----------  (  -------------  -  theta )   �

�    theta         expectancy                 �

�����������������������������������������������

Thus two basic facts are established by the C-R-A-S

inequality...

...the sample index (observed / expected nonconformance

ratio) is the minimum variance estimate of the true

regret index, theta, and

...the (Fisherian) information content of the data

is A(theta) = expectancy / theta, which is directly

proportional to the nonconformance expectancy at

standard process capability.

Section A.4    Scale Invariance

Definition:     Equivalent nonconformances and expectancy are "scale

invariant" quantities in the following sense:

multiplying the regret by any strictly positive, constant

value does not change the implied equivalent

nonconformances and expectancy.  One implication of this

is that it doesn't make any difference whether regret is

expressed in terms of dollars or pennies or pounds

sterling.

Proof:          Suppose that a regret measure, R, is multiplied by any

constant value, K.  Then the expected value of K times R

would be K times E, the original expected value of R.

And the variance of K times R would be K-squared times

V, the original variance of R.  As a result,

multiplicative factors of K-squared cancel from both the

numerators and the denominators of the ratios, R*E/V and

E*E/V, that define equivalent nonconformances and

expectancy.

Specific Implications for

the Nine Regret Families:

ATTRIBUT.....Max Impact is unitless; call it ONE or

BOUNDMAX     use any convenient value.

LINEREGRET...The slope of the regret line is unimportant.

RANGREGRET

QUADRATC.....Regrets that are squared deviations from

ENDPOINT     target values can be measured in any

ONESIDED     convenient units.

BILINEAR.....Either the Inventory Shortage or the Excess

Inventory Regret Slope parameter can be set

equal to 1 if the second Slope is expressed

as a MULTIPLE of the first.

Section A.5    Summary

��������������������������������������������������������������������Ŀ

�                                                                    �

�  Knowledge of the statistical distribution of Regret when Quality  �

�  is at a STANDARD LEVEL (that is within the Present Capability     �

�  of the Manufacturing Process) is needed for Poissonization.       �

�                                                                    �

�    The two statistical parameters which must be specified are:     �

�            EXPREGRET = Expected Regret at Standard Quality,        �

�            VARREGRET = Regret Variance at Standard Quality.        �

�                                                                    �

�    Poissonization is accomplished by multiplying Regret by         �

�    EXPREGRET and then dividing by VARREGRET...                     �

�                                                                    �

�            EQUIVALENT DEFECTS = REGRET * EXPREGRET / VARREGRET.    �

�                                                                    �

�    Equivalent nonconformances have a statistical distribution that �

�    is like the Poisson in the sense that the mean and the variance �

�    of this distribution are EQUAL.  This common value is...        �

�                                                                    �

�            EQUIVALENT EXPECTANCY = EXPREGRET^2 / VARREGRET.        �

�                                                                    �

����������������������������������������������������������������������

#