CKDRIVE.COM

Version 2.0

Memory-Resident Driver for Add-On Computer Clocks

Document revised 12/15/90

IMPROVEMENTS IN THIS VERSION:

Version 2.0 of CKDRIVE eliminates limitations of

version 1.0 that required manual resetting for a

new year or a leap year.  The program now (1)

advances the year automatically after midnight

December 31 and (2) advances the date to February

29 instead of March 1 in leap years.  Other

changes improve synchronization between the add-on

clock and the system clock; they now agree within

about .01 sec.

AUTHOR:        Pete Petrakis

COMPANY:       Life Sciences Software

812 Vista Drive

Camano Island, Washington 98292

DISTRIBUTION:  Distributed as shareware, which means

"try before you buy," not "free."

Payment of $5 is required if you find

after 21 days that CKDRIVE meets your

needs.  Please send your payment (check

or money order) to the above address.

1

DESCRIPTION:

CKDRIVE is a memory-resident program that allows

battery-powered add-on clocks to be set with the

TIME and DATE commands in DOS.  It also

automatically updates the DOS clock/calendar an

instant before the time and date are displayed

whenever you use DOS's TIME or DATE command or are

running a program that reads the time and date.

CKDRIVE eliminates the need to run special

clocksetting programs such as SETCLOCK (to set the

add-on clock) and GETCLOCK (to refresh the DOS

clock/calendar from the add-on clock).  Whenever

you want to change the time or date in your add-

on clock, just change the time or date in DOS, and

CKDRIVE will automatically pass the change along

to the add-on clock.  Whenever you want to know

the time or date in DOS, just ask for it in DOS,

and CKDRIVE will refresh the DOS clock/calendar

from the add-on clock an instant before the time

or date is displayed on your screen.

It's important to understand what is meant here by

an add-on clock. Although the so-called slotless

clocks, which plug into a ROM chip socket in your

computer, are also add-on clocks, they are not the

kind that CKDRIVE can work with.  Nor will it work

with the CMOS clock, which is a battery clock but

not an add-on, and is an integral part of the

original equipment in AT and better computers.

The add-on clock that CKDRIVE is designed to work

with is the type that comes on a circuit board

that plugs into an expansion slot in your

computer.  Although some of these clocks, such as

the one on the AST and Paradise multifunction

cards, come from well known manufacturers, the

clock-only type are typically quite inexpensive,

with mail order prices often ranging from $7.50

to $15, and very often neither the card nor the box

it came in even has a manufacturer's name on it.

For example, the box mine came in says only, "Add-

On Card for '88 or '286 Personal Computer" and

"Made in China."  The word "clock" doesn't appear

2

anywhere on the box and there is not a shred of

documentation.  The label on the diskette has only

the handwritten letters, "CK," which presumably is

an abbreviation for "CLOCK."  That's the type of

clock CKDRIVE is designed for, and what is meant

here by "add-on clock."  (Unfortunately, this does

not include Tandy's Smart Clock, which is also a

slot-type add-on clock but has altogether different

requirements.)

Despite its generic nature this cheap real-time

clock, which is typically used in XT-class computers,

is quite accurate (as computer clocks go) and in a

properly designed program can track time to the

millisecond (DOS can only display or set time to

1/100 second).

Interestingly, the cheap no-name generic imports

and some brand-name clock boards use the same type

of clock chip and usually the same set of I/O

ports.  The base port is typically 240 hex, 2C0

hex, or 340 hex (occasionally 3C0 hex) and most of

the boards allow you to select from a pair of

these port addresses (usually 240 hex and 340 hex)

by moving a jumper.  When CKDRIVE is loaded, it

checks to see which port addresses your add-on

clock is using and configures itself accordingly

for memory-resident operation.  If it finds no

add-on clock at any of the four likely base ports,

it reports that and quits.

Some of the better known brands of add-on clocks,

such as the one on the AST Six-Pack, come with

their own memory-resident driver software.  AST's

driver is called ASTCLOCK, which goes memory-

resident when you run it with /R on the command

line.  If your add-on clock came with that kind of

software, you won't need CKDRIVE, although you

could probably substitute CKDRIVE for the original

driver software.  If yours didn't come with that

kind of software, then CKDRIVE is for you.

3

RUNNING THE PROGRAM:

Type CKDRIVE at the DOS prompt, preferably

immediately after boot-up.  The first thing

CKDRIVE does when it's loaded is reset the DOS

clock/calendar from the add-on clock.  Thereafter

your DOS clock/calendar and your add-on clock will

behave as a single clock, passing time and date to

each other as needed.  It's advisable to put

CKDRIVE in your AUTOEXEC.BAT file so it can reset

the DOS clock/calendar from the battery clock

whenever you boot the system.  Obviously it should

go ahead of any program in the AUTOEXEC sequence

that needs to check the DOS time and date.

When loaded memory-resident, CKDRIVE takes less than

7k of RAM.

AUTOMATIC YEAR ADVANCE AND LEAP YEAR ADJUSTMENT:

Version 2.0 of CKDRIVE will automatically advance

the year in both the add-on clock and the DOS

clock/calendar the first time you request the date

in January -- provided you requested the date at

least once with CKDRIVE installed during the

previous month.  After midnight February 28 in a

leap year it will automatically set the date to

February 29 in both clocks -- provided you

requested the date at least once with CKDRIVE

installed between January 1 and February 28.

(Requesting the date during those preliminary

periods is practically unavoidable; you

automatically request the date every time you save

a new file to disk or date-stamp a new entry in a

data file, for example.)

SYNCHRONIZATION ACCURACY:

When you set or read the time and date from DOS

with CKDRIVE 2.0 loaded, the two clocks should

agree within about .01 second, according to tests

on my 8086 computer.  However, agreement between

the two clocks thereafter will depend on two

factors:  (1) the relative drift rates of the two

4

clocks and (2) the frequency of setting or reading

the time.  Remember that all computer clocks

drift, including DOS clock/calendars, battery-

operated add-ons, and CMOS clocks.  So, if the

add-on clock drifts a few seconds a day, its

deviation from the true time will be passed to the

DOS clock/calendar whenever you ask for the time;

the two clocks will then agree, but both will

deviate from true time to the extent that the add-

on clock does.

KNOWN INCOMPATIBILITIES:

If CKDRIVE is loaded, you should NEVER use the

original software that came with your add-on clock

for reading and setting it.  If you do, the hours,

minutes, seconds, fractions of seconds, month, and

day of the month will probably be OK, but the year

will probably be read or set wrong, from CKDRIVE's

perspective.  The reason is that the three ports

available for year information in these clocks are

for passive storage and are not controlled by the clock

chip, allowing each software developer to devise his

own method of managing that information.  Every

program, including CKDRIVE, stores and reads

information about years differently, so you should

always read these clocks with the same software or

software brand you used to set them.  Besides, you

won't need any of that software if you use

CKDRIVE; its purpose is to make those separate

programs unnecessary.

Use only one clock driver at a time.  If you

already have a driver that does what CKDRIVE does,

there is no need to load CKDRIVE too.  There will

be trouble if you do; competition between the two

drivers can slow things down dramatically and could

even cause a crash.

The Microsoft BASIC Professional Development

System (version 7.0 and probably 7.1 of BASIC)

is incompatible with CKDRIVE.  This is disappointing

but not surprising since Microsoft recommends that

no memory-resident programs at all be present when

running that version of BASIC.  No system crash

5

results when BASIC 7 is loaded after CKDRIVE, and

the two clocks will continue to run, but each will

be oblivious to the other thereafter.  BASIC 7 messes

things up so the two clocks can no longer communicate

with each other through CKDRIVE, and you'll have to

reboot and reload CKDRIVE to get that interaction again.

Finally, conflicts between memory-resident programs

are a fact of life, and they may require you to modify

the sequence in which such programs are loaded.

PROGRAM HISTORY:

CKDRIVE.COM was inspired by CLKDRV10.SYS, a driver

for slot-type add-on clocks that is loaded as a

device in the CONFIG.SYS file and was written by

my friend Tom Strickland of Alexandria, Virginia.

Tom's driver is part of my TIMESET 6.0 package,

which can be found on Compuserve and BBSs around

the country (depending on the sysop's preference,

the archive file will be named TSET60 or TIMSET60

(.ZIP, .ARC, etc.).

Tom programs in assembly and C.  I program in

BASIC and wanted to see if I too could write a

driver for these clocks using Microsoft's BASIC

Professional Development System (BASIC 7.0) and

linkable assembly routines from MicroHelp, Inc.

and Crescent Software, as well as some special

purpose assembly routines that I wrote.  Crescent

Software's P.D.Q. replacement library for

QuickBASIC is mainly responsible for the program's

small size (for a BASIC program) and its being a

TSR.

How do the two drivers compare?  Surprisingly

well.  With my 8086 computer, the DOS clock and

the add-on clock usually agree within 0.01 sec,

making version 2.0 of CKDRIVE.COM precise enough

to be used with TIMESET.

6

DISTRIBUTION AND DISCLAIMER:

You can distribute CKDRIVE.COM to anyone you want

provided you do not charge money for it (that's my

department) and do not separate it from this document.

Use of the program is at your own risk.  Pete Petrakis

does not claim that the program will work for everybody

and he accepts no responsibility or liability for

anything that might go wrong.

(Please don't forget to register.  It's only five bucks.)

Pete Petrakis

Camano Island, Washington

December 15, 1990

Your comments and suggestions are most welcome.  I can be reached on

Compuserve (76555,1175).

7