Filter Designer

1) System Requirements

Filter Designer will run on any IBM-AT or PS/2 with 512 kilobytes of

memory, a fixed disk, an 80287 or 80387 floating-point coprocessor,

either an EGA or VGA monitor, and the MS-DOS 3.0 (or later) operating

system.  The Filter Designer will also run on IBM-compatible systems

as long as they meet the above requirements.

2) Installation Instructions

Filter Designer consists of one program file, EFILTER.EXE, which can be

placed in any directory on your hard disk.  If you plan on running this

program from other directories, besides the one in which the file resides,

be sure to include the program directory in your path description.

Included along with the program file is a PSpice library file, FILTER.LIB,

which contains generic subcircuits that Filter Designer uses for filter

realization. Filter Designer's PSpice netlist files reference FILTER.LIB

with a .LIB statement.

3) Operating Instructions

To start the Filter Designer program, type:

EFILTER<cr>

4) Filter realization example

The following text represents an example of a PSpice circuit file which

uses a subcircuit netlist created by Filter Designer. The PSpice circuit

file connects the filter subcircuit to an input and output, and then

performs an AC sweep, printing the filter's magnitude and phase frequency

response. The circuit uses an include statement to reference the netlist

generated by Filter Designer.

EXAMPLE FILTER FREQUENCY RESPONSE

*

* THIS FILTER IMPLEMENTS A 3RD ORDER LP CHEBYSHEV FILTER WITH

* A PASSBAND OF 1KHZ, STOPBAND AT 2KHZ, RIPPLE OF 1DB, AND

* ATTENUATION OF 20DB. THE FILTER IS FOR CONTINUOUS INPUT AND

* THE REALIZATION IS WITH AKERBERG-MOSSBERG.

*

* SETTING UP AC INPUT VOLTAGE FOR FILTER AT NODE 1

*

VIN 1 0 AC 1

*

* CONNECTING FILTER TO NODES 1, 2, AND GROUND

*

XFILT 1 2 0 FILTER

*

* INCLUDING SUBCKT FILE FROM FILTER DESIGNER

*

.INC LP_EXAMP.CIR

*

* PERFORMING AC SWEEP WITH MAGNITUDE/PHASE OUTPUT AND SETTING UP FOR PROBE

*

.AC DEC 100 100HZ 10KHZ

.PRINT AC VM(2),VP(2)

.PROBE

.END

The following is an example netlist generated by Filter Designer. It is

referenced in the preceeding circuit file by the name LP_EXAMP.CIR with

an include statement.

* **************  NET LIST  **************

*  ORIGINAL BY FILTER DESIGNER

*  FILTER FILENAME: LP_EXAMP

.LIB FILTER.LIB

.SUBCKT FILTER IN OUT AGND

RIN IN 1 1

X1 1 2 AGND LINSTG

+  PARAMS:  C1VAL=   1.000e-20  C2VAL=   1.000e-08  R1VAL=   3.221e+04

+           R2VAL=   3.221e+04

X2 2 3 AGND AMSTG

+  PARAMS:  C1VAL=   1.000e-08  C2VAL=   1.000e-08  C3VAL=   1.000e-20

+           R1VAL=   1.596e+04  R2VAL=   1.596e+04  R3VAL=   1.596e+04

+           R4VAL=   1.000e+20  R5VAL=   1.000e+20  R6VAL=   3.221e+04

+           R7VAL=   1.596e+04  R8VAL=   1.596e+04

ROUT 3 OUT 1

ROUTGND OUT AGND 1.0e12

.ENDS

For further details about the Filter Designer package and for user tutorials,

you may purchase the Filter Designer User's Guide for $35.00.  This

documentation will get you started and answer many of your questions.  The

User's Guide may be purchased from MicroSim by calling (800) 245-3022.