GrayRule:

This program lets you control the powerful Rule, Pattern and Grayscale commands

that are resident in the HP-LaserJet II and compatible laser printers.

Rule:  Use to draw vertical or horizontal lines or boxes.

Boxes can optionally be shaded in.  See Grayscale below.

Left and Top Edge values specify the location of the

boxes or lines on the page.

Width value is the width of the box or line.  Use a Width value

of 0 for a vertical line.

Height value is the height of the box or line.  Use a height

value of 0 for a horizontal lines.

Thickness is the thickness of the lines.  A value of 45 is

equal to 1/16 inch.

Style is the style of the line.  Specify black, one of the

resident patterns or percent grayscale.

Grayscale:  Use to shade in a portion or all of the page.  Specify one

of the resident patterns or percent grayscale.

After all values have been entered you will return to the menu.

Create more lines AND boxes on the same page or save your work.

Print:      Use to print the page.  Use the Save option first.

Save:       Use to save entries to disk for later printing.  Optionally, the

file can be saved as an Overlay.  When loaded in the printer

an Overlay will cause the image to appear on every page until

the printer is reset.  You can then use your word processor to

merge text onto the page.  When saving files as an Overlay you

will be prompted for a ID number.  This can be any number from

0 to 32767. If your word processor resets the printer before the

document prints, the following escape sequence will activate the

Overlay [ ESC&f4X ].  To load the file into the printer; use the

MS-DOS command  [ COPY /B filename PRN ].

Logo:       Use to merge a graphic file on to the page.

Left and Top Edge values specify the location of the

Graphic on the page.  If no file name is given the cursor

will be moved to this location.  Full Path names can be used

in the file name.

Text:       Use to merge a Text file on to the page.

This can be any ASCII file or SoftModule.  A SoftModule is

a file of one or more SoftFonts with their ID#'s and settings.

A SoftModule can be loaded into the printer without using a

special download utility.