Copyright (C) 1989, 1990, 1991 Aladdin Enterprises.  All rights reserved.

Distributed by Free Software Foundation, Inc.

This file is part of Ghostscript.

Ghostscript is distributed in the hope that it will be useful, but

WITHOUT ANY WARRANTY.  No author or distributor accepts responsibility

to anyone for the consequences of using it or for whether it serves any

particular purpose or works at all, unless he says so in writing.  Refer

to the Ghostscript General Public License for full details.

Everyone is granted permission to copy, modify and redistribute

Ghostscript, but only under the conditions described in the Ghostscript

General Public License.  A copy of this license is supposed to have been

given to you along with Ghostscript so you can know your rights and

responsibilities.  It should be in a file named COPYING.  Among other

things, the copyright notice and this notice must be preserved on all

copies.

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

This file, drivers.doc, describes the interface between Ghostscript and

device drivers.

For an overview of Ghostscript and a list of the documentation files, see

README.

********

******** Adding a driver ********

********

To add a driver to Ghostscript, all you need to do is edit gdevs.mak in

two places.  The first is the list of devices, in the section headed

# -------------------------------- Catalog ------------------------------- #

Pick a name for your device, say smurf, and add smurf to the list.

(Device names must be 1 to 8 characters, consisting of only letters,

digits, and underscores, of which the first character must be a letter.

Case is significant: all current device names are lower case.)

The second is the section headed

# ---------------------------- Device drivers ---------------------------- #

Suppose the files containing the smurf driver are called joe and fred.

Then you should add the following lines:

# ------ The SMURF device ------ #

smurf_=joe.$(OBJ) fred.$(OBJ)

smurf.dev: $(smurf_)

gssetdev smurf.dev $(smurf_)

joe.$(OBJ): joe.c ...and whatever it depends on

fred.$(OBJ): fred.c ...and whatever it depends on

If joe uses platform-specific, non-ANSI-compatible features (like inline

assembly code), then the line should read

joe.$(OBJ): joe.c ...and whatever it depends on

$(CCNA) $*

********

******** Driver structure ********

********

A device is represented by a structure divided into three parts:

- procedures that are shared by all instances of each device;

- parameters that are present in all devices but may be different

for each device or instance; and

- device-specific parameters that may be different for each instance.

Normally, the procedure structure is defined and initialized at compile

time.  A prototype of the parameter structure (including both generic and

device-specific parameters) is defined and initialized at compile time,

but is copied and filled in when an instance of the device is created.

The gx_device_common macro defines the common structure elements, with the

intent that devices define and export a structure along the following

lines:

typedef struct smurf_device_s {

gx_device_common;

... device-specific parameters ...

} smurf_device;

smurf_device gs_smurf_device = {

sizeof(smurf_device),		* params_size

{ ... procedures ... },		* procs

... generic parameter values ...

... device-specific parameter values ...

};

The device structure instance *must* have the name gs_smurf_device, where

smurf is the device name used in gdevs.mak.

All the device procedures are called with the device as the first

argument.  Since each device type is actually a different structure type,

the device procedures must be declared as taking a gx_device * as their

first argument, and must cast it to smurf_device * internally.  For

example, in the code for the "memory" device, the first argument to all

routines is called dev, but the routines actually use md to reference

elements of the full structure, by virtue of the definition

#define md ((gx_device_memory *)dev)

(This is a cheap version of "object-oriented" programming: in C++, for

example, the cast would be unnecessary, and in fact the procedure table

would be constructed by the compiler.)

Structure definition

--------------------

This essentially duplicates the structure definition in gxdevice.h.

typedef struct gx_device_s {

int params_size;		/* size of this structure */

gx_device_procs *procs;		/* pointer to procedure structure */

char *name;			/* the device name */

int width;			/* width in pixels */

int height;			/* height in pixels */

float x_pixels_per_inch;	/* x density */

float y_pixels_per_inch;	/* y density */

gs_rect margin_inches;		/* margins around imageable area, */\

/* in inches */\

int has_color;			/* true if device supports color */

unsigned short max_rgb_value;	/* max r, g, b value */

int bits_per_color_pixel;	/* for copy_color */

int is_open;			/* true if device has been opened */

} gx_device;

The name in the structure should be the same as the name in gdevs.mak.

gx_device_common is a macro consisting of just the element definitions.

If for any reason you need to change the definition of the basic device

structure, or add procedures, you must change the following places:

- This document and history.doc (if you want to keep the

documentation up to date).

- The definition of gx_device_common and/or the procedures

in gxdevice.h.

- The null device in gsdevice.c.

- The tracing "device" in gstdev.c.

- The "memory" devices in gdevmem.c.

- The command list "device" in gxclist.c.

- All the real devices in the standard Ghostscript distribution,

as listed in gdevs.mak.

- Any other drivers you have that aren't part of the standard

Ghostscript distribution.

You may also have to change the code for zmakedevice (in zdevice.c) and

gs_makedevice (in gsdevice.c).

********

******** Types and coordinates ********

********

Coordinate system

-----------------

Since each driver specifies the initial transformation from user to device

coordinates, the driver can use any coordinate system it wants, as long as

a device coordinate will fit in an int.  (This is only an issue on MS-DOS

systems, where ints are only 16 bits.  User coordinates are represented as

floats.)  Typically the coordinate system will have (0,0) in the upper

left corner, with X increasing to the right and Y increasing toward the

bottom.  This happens to be the coordinate system that all the currently

supported devices use.  However, there is supposed to be nothing in the

rest of Ghostscript that assumes this.

Drivers must check the coordinate parameters given to them: they should

not assume the coordinates will be in bounds.

Types

-----

Here is a brief explanation of the various types that appear as parameters

or results of the drivers.

gx_device (defined in gxdevice.h)

This is the device structure, as explained above.

gs_matrix (defined in gsmatrix.h)

This is a 2-D homogenous coordinate transformation matrix, used by

many Ghostscript operators.

gs_rect (defined in gs.h)

This defines a rectangle by giving the coordinates of its minimal

and maximal corners.  It is only used (perhaps misused) to define the

margins of the imageable area.

gx_color_index (defined in gxdevice.h)

This is meant to be whatever the driver uses to represent a device

color.  For example, it might be an index in a color map.  Ghostscript

doesn't ever do any computations with these values: it gets them from

map_rgb_color and hands them back as arguments to several other

procedures.  The special value gx_no_color_index (defined as

(gx_color_index)(-1)) means "transparent" for some of the procedures.  The

type definition is simply:

typedef unsigned long gx_color_index;

gx_bitmap (defined in gxbitmap.h)

This structure type represents a bitmap to be used as a tile for

filling a region (rectangle or trapezoid).  Here is a copy of the relevant

part of the file:

/*

* Structure for describing stored bitmaps.

* Bitmaps are stored bit-big-endian (i.e., the 2^7 bit of the first

* byte corresponds to x=0), as a sequence of bytes (i.e., you can't

* do word-oriented operations on them if you're on a little-endian

* platform like the Intel 80x86 or VAX).  Each scan line is normally

* padded only to a byte boundary, although this should be of no

* concern since the raster and width are specified separately.

* The first scan line corresponds to y=0 in whatever coordinate

* system is relevant.

*/

struct gx_bitmap_s {

byte *data;

int raster;		/* bytes per scan line */

int width, height;

};

********

******** Driver procedures ********

********

All the procedures that return int results return 0 on success, or -1 on

invalid arguments.

Most procedures are optional.  If a device doesn't supply an optional

procedure proc, the entry in the procedure structure should be

gx_default_proc, e.g. gx_default_tile_rectangle.  The device procedure can

also call this procedure if it doesn't implement the function for

particular values of the arguments; alternatively, it can return -1, since

all callers of optional procedures will call the default procedure in this

case.

Open/close/sync

---------------

int (*open_device)(P1(struct gx_device_s *)) [OPTIONAL]

Open the device: do any initialization associated with making the

device instance valid.  This must be done before any output to the device.

void (*get_matrix_and_clip)(P3(struct gx_device_s *, gs_matrix *)) [OPTIONAL]

Construct the initial transformation matrix mapping user

coordinates (nominally 1/72" per unit) to device coordinates.  The default

procedure computes this from width, height, and x/y_pixels_per_inch on the

assumption that the origin is in the upper left corner, i.e.

xx = x_pixels_per_inch/72, xy = 0,

yx = 0, yy = -y_pixels_per_inch/72,

tx = 0, ty = height.

int (*sync_output)(P1(struct gx_device_s *)) [OPTIONAL]

Synchronize the device.  If any output to the device has been

buffered, send / write it now.  Note that this may be called several times

in the process of constructing a page, so printer drivers should NOT

implement this by printing the page.

int (*output_page)(P1(struct gx_device_s *)) [OPTIONAL]

Output a fully composed page to the device.  The default

definition just calls sync_output.  Printer drivers should implement this

by printing and ejecting the page.

int (*close_device)(P1(struct gx_device_s *)) [OPTIONAL]

Close the device: release any associated resources.  After this,

output to the device is no longer allowed.

Color mapping

-------------

Ghostscript represents colors internally as RGB values.  In communicating

with devices, however, it assumes that each device has a fixed palette of

colors identified by integers (to be precise, elements of type

gx_color_index), with the following properties:

- The palette contains at least white and black.

- If the device has gray-scale but not color capabilities, and

max_rgb_value is less than 255, the palette consists of an arbitrary

number of gray shades, uniformly spaced on the diagonal of the RGB color

cube.

- If the device has color capabilities, and max_rgb_value is less

than 255, the palette consists of a color cube uniformly spaced in RGB

space (obviously including at least the three primary colors (red, green,

blue)).

These are very strong conditions: they are required by the algorithm that

Ghostscript uses to create spatial halftones to approximate unavailable

colors or gray levels.  In the future, we envision giving drivers much

more control over color selection, and these conditions will disappear.

gx_color_index (*map_rgb_color)(P4(struct gx_device_s *, unsigned short red,

unsigned short green, unsigned short blue)) [OPTIONAL]

Map a RGB color to a device color.  The default algorithm simply

returns the brightness, i.e. max(red, green, blue).  The range of legal

values of the RGB arguments is given by the max_rgb_value element of the

device parameter structure.  Ghostscript assumes that for devices that

have color capability (i.e., has_color is true), map_rgb_color returns a

color index for a gray level (as opposed to a non-gray color) iff red =

green = blue.

int (*map_color_rgb)(P3(struct gx_device_s *, gx_color_index color,

unsigned short rgb[3])) [OPTIONAL]

Map a device color code to RGB values.  The default algorithm

simply sets all three elements of the result to the color.

Drawing

-------

All drawing operations use device coordinates and device color values.

int (*fill_rectangle)(P6(struct gx_device_s *, int x, int y,

int width, int height, gx_color_index color))

Fill a rectangle with a color.  The set of pixels filled is

{(px,py) | x <= px < x + width and y <= py < y + height}.  In other words,

the point (x,y) is included in the rectangle, as are (x+w-1,y), (x,y+h-1),

and (x+w-1,y+h-1), but *not* (x+w,y), (x,y+h), or (x+w,y+h).  If width <=

0 or height <= 0, fill_rectangle should return 0 without drawing anything.

int (*draw_line)(P6(struct gx_device_s *, int x0, int y0, int x1, int y1,

gx_color_index color)) [OPTIONAL]

Draw a minimum-thickness line from (x0,y0) to (x1,y1).  The

precise set of points to be filled is defined as follows.  First, if y1 <

y0, swap (x0,y0) and (x1,y1).  Then the line includes the point (x0,y0)

but not the point (x1,y1).  If x0=y0 and x1=y1, draw_line should return 0

without drawing anything.

int (*fill_trapezoid)(P8(struct gx_device_s *, int x0, int y0, int width0,

int x1, int y1, int width1, gx_color_index color)) [OPTIONAL]

Fill a trapezoid whose corners are (x0,y0), (x0+width0,y0),

(x1,y1), and (x1+width1,y1), where y0 < y1.  If width0 < 0, width1 < 0,

width0 = width1 = 0, or y0 >= y1, fill_trapezoid should return 0 without

drawing anything; however, width0 = 0 or width1 = 0 is valid, and should

result in a triangle being filled.  As for fill_rectangle, the coordinates

are "half-open": the points (x0+width0,y0) and (*,y1) are not included, so

the extremal points that *are* included are

(x0, y0)	(x0 + width0 - 1, y0)

(x1, y1 - 1)	(x1 + width1 - 1, y1 - 1)

Bitmap imaging

--------------

Bitmap (or pixmap) images are stored in memory in a nearly standard way.

The first byte corresponds to (0,0) in the image coordinate system: bits

(or polybit color values) are packed into it left-to-right.  There may be

padding at the end of each scan line: the distance from one scan line to

the next is always passed as an explicit argument.

int (*copy_mono)(P10(struct gx_device_s *, unsigned char *data, int data_x,

int raster, int x, int y, int width, int height,

gx_color_index color0, gx_color_index color1))

Copy a monochrome image (similar to the PostScript image

operator).  Each scan line is raster bytes wide.  Copying begins at

(data_x,0) and transfers a rectangle of the given width at height to the

device at device coordinate (x,y).  (If the transfer should start at some

non-zero y value in the data, the caller can adjust the data address by

the appropriate multiple of the raster.)  The copying operation writes

device color color0 at each 0-bit, and color1 at each 1-bit: if color0 or

color1 is gx_no_color_index, the device pixel is unaffected if the image

bit is 0 or 1 respectively.

This operation is the workhorse for text display in Ghostscript,

so implementing it efficiently is very important.

int (*tile_rectangle)(P10(struct gx_device_s *, gx_bitmap *tile,

int x, int y, int width, int height,

gx_color_index color0, gx_color_index color1,

int phase_x, int phase_y)) [OPTIONAL]

Tile a rectangle.  Tiling consists of doing multiple copy_mono

operations to fill the rectangle with copies of the tile.  The tiles are

aligned with the device coordinate system, to avoid "seams".

Specifically, the (phase_x, phase_y) point of the tile is aligned with the

origin of the device coordinate system.  (Note that this is backwards from

the PostScript definition of halftone phase.)  phase_x and phase_y are

guaranteed to be in the range [0..tile->width) and [0..tile->height)

respectively.

If color0 and color1 are both gx_no_color_index, then the tile is

a color pixmap, not a bitmap: see the next section.

int (*tile_trapezoid)(P12(struct gx_device_s *, gx_bitmap *tile,

int x0, int y0, int width0, int x1, int y1, int width1,

gx_color_index color0, gx_color_index color1,

int phase_x, int phase_y)) [OPTIONAL]

Tile a trapezoid similarly.

Pixmap imaging

--------------

Pixmaps are just like bitmaps, except that each pixel occupies more than

one bit.  All the bits for each pixel are grouped together (this is

sometimes called "chunky" or "Z" format).  The number of bits per pixel is

given by the bits_per_color_pixel parameter in the device structure: the

legal values are 1, 2, 4, 8, 16, 24, or 32.  The pixel values are device

color codes (i.e., whatever it is that map_rgb_color returns).

int (*copy_color)(P8(struct gx_device_s *, unsigned char *data, int data_x,

int raster, int x, int y, int width, int height))

Copy a color image with multiple bits per pixel.  The raster is in

bytes, but x and width are in pixels, not bits.

tile_rectangle and tile_trapezoid can also take colored tiles.  This is

indicated by the color0 and color1 arguments both being gx_no_color_index.

In this case, as for copy_color, the raster and height in the "bitmap" are

interpreted as for real bitmaps, but the x and width are in pixels, not

bits.