NAME

ASYNC FUNCTIONS:

readchar -- read a character from serial port

ready_rcv -- check for character availability

ready_xmt -- check if port can accept a char to send

all_sent -- check if port has completed pending transmission

setport -- set port configuration

writechar -- send a char to the serial port

SYNOPSIS

r = readchar(port);

x = ready_rcv(port);

x = ready_xmt(port);

x = all_sent(port, wait);

(void) setport(port, r);

(void) writechar(port, r);

unsigned char r;   character to send or receive

int port;          port number

int x;             TRUE if condition exists, else FALSE (0)

int wait;          TRUE to wait until task completed,

FALSE to return condition immediately

DESCRIPTION

These are very low level (direct port access) subroutines

to enable I/O through COM1 through COM4.  Only setport() uses

BIOS int 14H.  These functions are not intended to be the

ultimate in async functions, but a starting point for an

integrated set of async functions, and a means of getting at

the serial port directly for speed and to work around the

problems that some "clone" BIOS routines have in returning

proper status values.  File "smdefs.h" contains #defines

to handle these ports mnemonically.

The value to use for the setport() function is an 8-bit

value, bitwise organized as follows:

bits 1,0:     10 = 7-bit data, 11 = 8-bit data

bit  2:        0 = 1 stop bit, 1 = 2 stop bits

bits 4,3:     x0 = No Parity, 01 = odd, 11 = even

bits 7,6,5:   baud rate:

000 = 110 baud (use 2 stop bits)

001 = 150 baud

010 = 300 baud

011 = 600 baud

100 = 1200 baud

101 = 2400 baud

110 = 4800 baud

111 = 9600 baud

ready_rcv() returns a TRUE condition if a character is

waiting to be read, but no error conditions are returned.

ready_xmt() returns a TRUE condition if the transmit

holding register is empty; i.e., the transmitter can accept

a character to send.  In both these functions, the status

returned is a result of reading the port status and masking

all but the relevant bit.  These routines will NOT return

errors such as overrun or parity.

async functions,      page 2

all_sent(port, TRUE) will wait until the transmitter shift

register is empty before returning.  It will always return

a TRUE condition.  all_sent(port, FALSE) will return at once

with a TRUE condition if the transmitter is empty, and a

FALSE condition if a character is still being sent or held

in the shift register.

It is important to note the difference between ready_xmt() and

all_sent().  The former indicates that the transmitter can or

cannot accept another character.  The latter indicates whether

all pending characters in the transmitter have been sent.

The latter is used most frequently to implement hardware

handshaking schemes.

readchar() will return the character waiting in the receiver

register of the port.  It is returned as an unsigned char

to facilitate 8-bit binary transfer protocols.

writechar() will place the specified unsigned char into the

transmitter buffer register where it will be sent.

If the program "loop" is short and fast enough, these functions

will be ample to allow communications at up to 9600 baud.

A program has already been implemented to communicate with a

prom programmer at this rate, so it can be done.  However,

when the program does not poll the receiver fast enough it

is likely that characters may be lost and the maximum usable

baud rate will be lower.  It is highly unlikely that baud rates

less than 2400 baud will be necessary if the program is properly

structured.

EXAMPLE

/* this routine receives characters from COM1 at one

** speed and resends them to COM2 at another speed */

unsigned char i;

setport(SER1, 0xe3);  /* set COM1 to 9600 baud, No parity,

** 8-bits, 1 stop bit */

setport(SER2, 0xc3);  /* set COM2 the same, except 4800 baud */

while(TRUE) {

if(!ready_rcv(SER1)) continue;  /* wait for a char */

i = readchar(SER1);     /* fetch it */

while(!ready_xmt(SER2));/* wait for transmitter to be ready */

writechar(SER2, i);  /* then send it */

}

async functions,      page 3

CAVEAT:

3-10-88:

The use of setport() is HIGHLY discouraged.  A new function,

config_port() should be used instead.  The problem exists

because the ROM-BIOS idea of what constitutes COMx could be

different that the direct access method used by these functions.

This could result in setport()-ing to a different address

than the other async functions will be using.  In general,

async functions which use ROM-BIOS calls should not be

mixed with functions which use direct port access.