NAME

config_port -- configure serial port

SYNOPSIS

r = config_port(baud, parity, bits, stop, port);

int r;          FALSE on error, TRUE otherwise

unsigned baud;  actual numerical baud rate

int parity;     NONE, ODD, or EVEN (see smdefs.h)

int bits;       5, 6, 7, or 8

int stop;       FALSE = 2 stop bits, TRUE = 1

int port;       port address 0 - 3 (see async.doc)

DESCRIPTION

This function replaces the earlier function setport(), by

directly configuring the UART IC instead of using ROM-BIOS.

This is needed since the port accessed by the ROM-BIOS routines

may NOT be the same as accessed by the other async functions

in this library.  This library uses direct port addressing,

while the ROM-BIOS routines use the indirect COMx notation,

which can be re-assigned by DOS.  The setport() function is

still provided, but it is recommended that it NOT be used if

the other async functions of this library are used.

The config_port() function returns FALSE if a parameter error

is detected.  No checks are made to verify that the addressed

port exists or is responding.

EXAMPLE

/* initialize serial port #2 (COM2 usually) to 9600 baud,

no parity, 8 bits, 1 stop bit */

if(config_port(9600, NONE, 8, TRUE, SER2))

puts("Port at address 02F8H initialized to 9600, N-8-1");

/* attempt to initialize port #1 to 350 baud, Even parity,

7 bits, 1 stop bit.  350 baud is non-standard, and not

supported.  Function will return FALSE. */

if(!config_port(350, EVEN, 7, TRUE, SER1))

puts("Port at address 03F8H cannot accept these parameters.");