NAME

gameport functions -- manipulate game port

SYNOPSIS

(void)init_game(mode);    initialize and set mode

(void)clear_game();       clear pending input

r = get_press();          get waiting keypress

(void)debounce();         input debouncer

int mode;                 single/multi stroke mode

unsigned char r;          key value

DESCRIPTION

This series of functions is used to read and debounce the

four switch inputs on the joystick input port.  The

analog joystick inputs are not supported.

init_game() is called to setup the processor.  If its

argument is TRUE, then multiple key inputs are allowed.

If FALSE, any multiple key closure will be locked out.

debounce() must be called periodically in order to

debounce and process the switch inputs.  It is suggested

that debounce() be called about 75 to 100 times per second.

The structure "gamesw" contains information used by

these routines.  The number of times debounce must be invoked

to return a valid input is contained in gamesw.seed, and may

be altered as desired.

get_press() is called to return any fully debounced input.

A value of 0 indicates no input available.

Calling clear_game() clears any pending input and forces

the debouncer to assume a new input sequence.

The game port was originally intended for playing games via

an arcade-style joystick.  However, for the serious engineer,

this port is an excellent "quick and dirty" 4 bit TTL level

input port.  It may be used for reading external switches,

sensors, etc.

Switch values are returned as an 8 bit value, with only the

lowest 4 bits in use.  A set bit indicates a closed switch.

If multiple keystrokes are not allowed, then get_press() will

return 0, 1, 2, 4, or 8.  If multiples are allowed, then

all values 0-15 (0-0x0f) may be returned.

bit position            pin number on connector

0                            2

1                            7

2                            14

3                            10

gameport functions    page 2

EXAMPLE

main() {

unsigned char i;

init_game(FALSE);    /* allow only single switch inputs */

for ever() {

debounce();       /* call this regularly */

i = get_press();

if(i is 0) continue;

printf("switch value is %x\n", i);

}

}