TTY READER version 1.3� - The RTTY/ASCII/CW Viewer

PUBLIC DOMAIN SOFTWARE - Copyright (c) 17 Feb 91, FineWare, Inc.

This is a preliminary 'manual' to the beta release of TTY Reader

v1.3.  The system requirements for operation of this program are an

MS-DOS Computer with at least 128k of memory, Color Graphics Adapter

(CGA), and at least one available serial port.  Since this program is

currently under development, this text serves as a tutorial on what

the program is intended to do, how to set it up, and finally how to

run it.  The distribution file TTYRDR13.ZIP should contain the follow-

ing:

TTY.EXE        - The TTY Reader v1.3� Program

TTY.DOC        - This Text

TTY.SCN        - TTY Reader Graphic Screens.

It is suggested that the user print out this text (TTY.DOC) for future

reference.

Note:  This (and all preceding versions of TTY Reader) were designed

for the IBM CGA display to appear as bright white characters on a

black background.  The resulting color scheme on EGA and VGA systems

will appear as green on white.  An attempt to correct this may be in-

cluded in a later version. (Much Thanx to Steve Sawicki for pointing

this out to me via Pinelands BBS)

PURPOSE.  The TTY Reader is intended to display radio teletype

and continuous wave (CW) Morse code from standard Terminal Unit (TU)

interfaces such as the Advanced Electronic Applications' CP-1 Computer

Patch Interface and NOT from All Mode Controller type devices (also

known as TNCs).  Such TU devices are designed to only convert the RTTY

transmission tones to RS-232 voltage levels, and do not contain the

necessary hardware/firmware to automatically adjust to transmission

rate, mode, and the like.  Limited by that, the TTY Reader was de-

signed to handle a wide selection of data rates (40.5 to 300 Baud) in

Baudot (RTTY) or ASCII modes as well as most CW keying speeds.  Given

that not all digital transmissions are alike, included in the Reader

was the ability to graphically transliterate Morse code as well as the

various 5 unit teleprinter code alphabets as dictated by Chapter 35 of

the ITT Reference Data for Radio Engineers and Chapter 19 of the ARRL

Handbook.  It was also desirable to add sundry other features in order

to enhance the Reader's ability to perform properly.

SETUP.  First, it is suggested that most terminate but stay resi-

dent programs (TSRs) are removed from the environment and that the

following commands are included in the user's CONFIG.SYS file:

DEVICE=ANSI.SYS

FILES=20

BUFFERS=15

It is necessary (but not essential) that the computer's real time

clock be accurately set.  If no battery back up clock is used in the

computer, the DOS TIME and DATE commands should be used prior to run-

ning the Reader.  The importance of this will be detailed later.  It

TTY Reader Version 1.3�                                          Page 1 of 9

is essential, though, that the user know to which COMM port the TU is

connected to.  This is only required the first time the program is

run.  If the previous version (v1.2�) of TTY Reader was used, it is

additionally necessary to delete the old TTY.DAT file.

RUNNING.  Upon first starting the TTY Reader, the user is

prompted to input the COMM port number to which the TU is connected.

The user should respond with either a '1' for COM1: or a '2' for

COM2:.  The second piece of information required by the Reader is the

number of hours required to convert to Greenwich Mean Time (GMT).  The

main terminal screen of the TTY Reader contains a real-time GMT clock

both for the user's information, and for use in some of the program's

other functions.  The user should respond with the number of hours re-

quired to be added to his local time to achieve GMT.  For example, the

user should respond to this prompt with a '4' for summer time on the

East Coast (The clock is restored to the user's local time upon exit-

ing the program).  These two pieces of information will then be stored

in a file created by the Reader, in the default directory, named

TTY.DAT.  Upon future runs of the program, this file will be automati-

cally read at start-up.

TTY Reader then begins with the opening screen, which simulates a TTY

test tape.  If it is desired, this screen may be skipped with an '/S'

switch as follows:

TTY /S

The program then continues to it's terminal screen.  The bottom of the

screen contains the time of day in GMT, a reminder of how to get to

the help screens, and the Reader's default settings.  A summary and

description of these settings are as follows:

BAUD RATE AND DATA TYPE:  Upon adjustment these settings are dis-

played in the lower right hand corner of the screen.  The standard

rates are supported by the function keys, non-standard rates may also

be set by the cursor keys:

Alt-F2 - Change to CW

F1 - Change to 45.45 baud Baudot code

F2 - Change to 50 baud Baudot code

Alt-F1 - Change to 56.92 baud Baudot code

F3 - Change to 75 baud Baudot code

F4 - Change to 100 baud Baudot code

Alt-F3 - Change to 110 baud ASCII code

Alt-F4 - Change to 300 baud ASCII code

CrUp - Increase baud rate

CrDn - Decrease baud rate

CrLf - Step through Baudot/ASCII/CW modes

CrRt - Step through CW/ASCII/Baudot modes

The CrUp and CrDn keys will step through the following baud rates:

40.5, 43, 45.45, 48, 50, 56.92, 60, 64, 72, 74.2, 75, 96, 100, 110,

134.5, 150, 200, and 300 only when running in Baudot or ASCII modes.

TTY Reader Version 1.3�                                          Page 2 of 9

Special Note:  TTY Reader already configures the NumLock and CapsLock

states of the computer's keyboard, regardless of their initial state

when starting the program.  They need not be changed during program

operation by the user.  Their original states are restored upon exit-

ing the program.

CW MORSE CODE:  The major addition to version 1.3� is the new CW

capability.  Although this mode has been made available in the soft-

ware a minor hardware modification must be made to your TU hardware.

This modification should NOT be made by the meek.  The CW algorithm

employed in the software utilizes the RS-232's Data Carrier Detect

(DCD) line to capture the synchronous nature of the CW signal.  In or-

der for the computer's COMM port to receive this signal properly the

normal output of the TU must be co-routed to the DCD output line.

Most simply, this may be a jumper between Received Data (RXD) pin and

Data Carrier Detect (DCD) pin at your TU's output port, provided the

RS-232 driver chip being used can handle the load (please refer to the

TU's operator manual, or the data sheet for the RS-232 driver chip

used).  On the other hand, the author strongly recommends using a sep-

arate buffer, sourced by the same unconditioned data signal, to drive

the DCD line.  The author has successfully breadboarded a 1489 RS-232

quad line driver in the AEA CP-1 in place of the 75150 dual driver

chip included in the RS-232 Option.  This deviation leaves at least

one extra line driver on the chip free for this purpose.

Once properly modified, the CW mode is activated by pressing Alt-F2 or

by using the left/right cursor keys to cycle through the modes (make

sure your TU is properly set for CW).  The algorithm used in the soft-

ware to display CW should adapt to changing or slightly irregular key-

ing speeds.  However, pressing Alt-F2 while already in CW mode will

attempt to re-sync after a drastic keying change (i.e.: new

sender/station).

BAUDOT TRANSLATION:  Upon adjustment these settings are displayed

to the right of the data type.  The difference between the transliter-

ation in these cases is usually in the figures (FIGS) case of the RTTY

character set:

F5 - US Military Standard (displayed as ' ').  This is the

most common of the translations.  It is normally used by both Amateurs

and US news and information services (such as the VOA).

F6 - US Telex Standard (displayed as 't').  This is used by

most of the international news services (such as TASS) and aero sta-

tions.  It basically corrects for the apostrophe ('), the bell and WRU

(Who aRe yoU?) symbols, and the dollar sign ($) characters.

F7 - International Telegraph Alphabet #2 (ITA2, displayed as

'i').  This is similar to the Telex Standard and is used by some of

the Spanish services such as TELAM of Argentina.  The difference lies

in the replacement of the dollar sign with the pound sign (#) and the

additions of the plus sign (+), n-tilde (�), the semi-colon (;) and

the equals sign (=).

TTY Reader Version 1.3�                                          Page 3 of 9

F8 - US Weather Standard (displayed as 'w').  This is hardly

ever used, yet is included as a possible baudot transliteration mode.

It includes graphical representations of wind directions and baromet-

ric symbols which may be seen on the second help screen.

Alt-F5 - Russian Cyrillic Character Set (displayed as 'c').

This may be used when reading the Soviet Ship traffic.  It was in-

cluded for those users that are able to read and understand the Rus-

sian language in its proper form.  The Cyrillic alphabet is accurately

converted with the following minor exceptions:

a)  The symbol '4' will be displayed instead of the Cyrillic

character which looks something like a 'y', but in larger case.

b)  The symbol 'b' will be displayed for the Cyrillic 'hard'

and 'soft' signs.

c)  The symbol 'E' will be displayed for the Cyrillic 'E' as

well as the upper case '�' symbols ('e' with two dots on top).

Special Note:  These exceptions are a result of the way the characters

are transmitted at the sending station and not a bug in the software.

OPERATIONAL MODES:  RTTY is sent under varying conditions and

modes.  It was the intent of this author to design into the program

the manipulation of these factors in order to facilitate these condi-

tions and to accommodate the personal tastes of the user.  It should

be noted that the condition of all of these functions (as well as the

baud rate and modes described above) with the exception of the ASCII

debug mode are saved in the file 'TTY.DAT' upon exiting from the pro-

gram so they may be automatically reinstated upon re-entry.  A summary

and description of these modes are as follows:

Alt-C - Toggle Carriage Returns (CRLF).  Most RTTY is sent

with the line ending with the CR,CR,LF character sequence.  The TTY

Reader automatically ignores the carriage return characters and pro-

cesses only the linefeeds.  However, in certain instances, such for-

matting is undesirable.  The line endings take more time to process

during a full screen, or some information may scroll off the screen

too quickly.  Normally, the program will display a 'CRLF' on the bot-

tom of the screen to denote that linefeeds will be processed.  Press-

ing Alt-C again will inhibit this and will toggle the 'CRLF' display

off so that the received text will 'wrap around' the full screen.

Alt-U - Toggle Unshift on Space (UOS).  RTTY convention re-

quires a FIGS (shift) character to be sent before a string of figures

are to be sent, with a LTRS (unshift) character following it.  Under

poor reception conditions, sometimes this LTRS character is missed and

the text will appear garbled between numbers and other symbols.  In

these instances, it is desirable to have a secondary safeguard to copy

the transmission correctly.  Pressing Alt-U will make sure that the

LTRS case is automatically reinstated upon the reception of a space

character or at the beginning of a new line.  When this 'unshift on

space' is enabled, a 'UOS' will appear on the bottom of the screen.

TTY Reader Version 1.3�                                          Page 4 of 9

Special Note:  The TTY Reader does not process third and fourth Cyril-

lic or Romanized shifts in the normal manner.  The reason for this is

that it is rarely, if ever, actually used.  A lot has been written

about it's use with Soviet Ship transmissions, when in actuality, it

is never used.  These special shifts are supposedly recognized by a

null character preceding the FIGS or LTRS characters.  Soviet ships

actually send a single null instead of LTRS and a FIGS without a null

for FIGS cases.  This program has accounted for this, by automatically

unshifting upon the reception of a null character.  This feature has

no bearing on the operation of the user selected 'unshift on space'

and actually allows the user to correctly copy Soviet Ships (under

good reception conditions) without the unshift on space enabled.

Alt-B - Toggle Bell Sounds (BELL).  Operation of RTTY in the

evening is noisy enough if a speaker is used to aid in tuning your re-

ceiver properly.  It is for this reason, that the bell sound is inter-

preted graphically (as a bell symbol) by default.  Pressing the Alt-B

combination will toggle the bell sound on and off.  'BELL' will be

displayed on the bottom of the screen if the bell sounds are enabled.

Alt-I - Invert Character Bit Polarity, and

Alt-R - Reverse Character Bit Order.  Some RTTY transmis-

sions may use simple forms of encryption in which one or more of the

character bits are inverted in polarity or mispositioned.  These func-

tions are an attempt to reconstruct two instances of this encryption.

Pressing Alt-I will invert all of the bits (XOR $FF) and Alt-R will

reverse the order of the bits in the received character.  Pressing ei-

ther of these key combinations will display an 'INV' and/or 'REV' on

the bottom of the screen to indicate that these respective processes

will be performed.  These functions may be used separately or in con-

junction with another to attempt to decipher some of those hidden

transmissions.

Alt-D - Toggle Debug Mode.  Some RTTY and ASCII transmis-

sions are sent specially formatted with control characters that do not

translate well to the screen.  An instance of this are packet trans-

missions which contain packets of text with message headers and trail-

ers.  For this reason, it may be desirable to display graphical inter-

pretations of these control characters, rather than attempt to display

them.  A unique feature of the TTY Reader incorporates this by dis-

playing the graphical representations as depicted in Chapter 19 of the

ARRL Handbook as well in the second help screen.  When enabled, the

'RTTY' or 'ASCII' indicator is retyped in lower case letters ('rtty',

'ascii') in the lower right hand corner of the screen.

Alt-A - Toggle Art Mode.  Amateur Radio enthusiasts often

send RTTY character artwork over the air.  Sending artwork will often

include apostrophe characters, as well as the ability to 'write over'

a line sent, in order to create the desired visual effect.  Since the

Ham operator has no idea whether the receiver is using either of the

US Military or TELEX transliteration modes (described above), the

apostrophe character is usually paired with a 'bell', distorting the

picture somewhat.  Also, TTY Reader normally ignores carriage return

characters in favor of the linefeeds, which makes 'write over's appear

TTY Reader Version 1.3�                                          Page 5 of 9

as two lines sent instead of one.  Pressing the Alt-A key combination

will automatically strip all bell characters from being displayed, as

well as properly translate the carriage return and linefeed characters

to accommodate artwork (regardless of the state of CRLF).  When the

Art Mode is enabled, the normal transliteration indicator (located to

the right of the data type indicator) will be replaced by an 'a' in

order to signify that this special mode is being processed.

OTHER UTILITIES:  Working RTTY transmissions is a hobby that re-

lates well to both the Amateur and Shortwave Listening hobbies.  Since

a computer is used as the medium, it is almost second nature to allow

the abilities of the computer to perform some of the mundane tasks as-

sociated with both.  It is for this reason that the following generic

utilities have been incorporated into the TTY Reader:

Alt-L - Write Log Information.  One of the mundane tasks of

such radio hobbies is the logging of activity.  The TTY Reader already

has converted the computer's clock to GMT and can automatically time

stamp the entries.  Upon pressing Alt-L, the TTY Reader stops incoming

traffic and prompts the user for a log entry on the top line of the

terminal screen.  Any information (less than 75 characters long) may

be typed here, such as frequency, service, call, rate and mode of

transmission.  The entry is concluded with the ENTER key, and the

traffic copy is resumed.  All entries are posted in the file

'TTY.LOG'.

Alt-P - Toggle Writing to Printer, or

Alt-F - Toggle Writing to File.  An enhancement to being

able to log the activity is the ability to simultaneously write the

traffic to either the printer or a file in the program's resident di-

rectory.  By pressing Alt-P, all information is simultaneously sent to

the printer as well as the screen.  In this case, characters which

have been created as graphic renditions (bell, WRU, and ASCII control

characters, as well as the special Cyrillic characters) will not

translate accurately to the printer (or a file).  Some of these char-

acters will be denoted as foreign characters or line symbols.  It

should also be noted, that since hard copy may be derived from either,

both functions may not be performed at the same time.  For instance,

if the traffic is currently being sent to the printer, pressing Alt-F

will disable the printer, and continue sending the rest of the trans-

mission to a file.  The same is true for going from first copying to a

file, and then to the printer.  The bottom line of the screen will de-

note 'FIL' or 'PRN' to denote that alternate copying is being per-

formed.  Pressing Alt-F will also display the name of the file being

copied to where the 'Help key' reminder normally is displayed.  Also,

while copying to a file, the beginning and end of the copy session

will be date and time stamped, since many sessions may use the same

file.

Alt-N - Change Output Filename.  As the default name for

file copying is 'TTY.TXT', this may be changed by the user.  By press-

ing Alt-N, the current filename is displayed on the top line of the

terminal screen, and the user is prompted for the new name.  The user

may then opt to type in a new filename, concluding with the 'ENTER'

TTY Reader Version 1.3�                                          Page 6 of 9

key, or simply press 'ENTER' to keep the old filename.  Only legal

filenames allowed by DOS are likewise allowed by the TTY Reader.  As

well, it is not recommended to include path names, as the file will

default to the resident directory of the Reader.

Alt-S - Shell to DOS.  As is human to do, errors are in-

evitable.  Pressing Alt-S will close all open files, discontinue the

traffic copy, and temporarily put the user in hands of DOS.  This al-

lows the user to edit the copied file or log entry with a small text

editor or his/her choosing.  It is recommended that the user remember

that this is only a SHELL of DOS and the TTY Reader is still in mem-

ory.  The user must type 'EXIT' to return to the Reader.  A small re-

minder of this is displayed after pressing Alt-S, has been inserted

into the DOS prompt while in the DOS Shell.

F9 - Clear Screen.  Sometimes it is desirable to erase traf-

fic from the screen and start with a fresh terminal.  Pressing the F9

function key will accomplish this, with no effect to the rest of the

program (alternate file copies, etc.).

F10 - Exit Program.  Pressing the F10 function key will

close the COM port to traffic, as well as any open output files, save

the current settings to 'TTY.DAT', restore the computer's clock to lo-

cal time, and return the user to DOS.  Since this program alters cer-

tain keyboard and clock functions, it is important that the user ONLY

exit the program this way.

CONCLUSION:  This and all of the other files contained in the

file TTYRDR13.ZIP are provided as public domain software to be used

and disseminated free of charge to the public.  The author requests

that no fee be charged for the enjoyment or use of this software, or

subsequent versions will contain numerous annoying pleas for cash.

Neither the author nor his associates are responsible for any results

of using this software or hardware modification recommendations in-

cluded in this document on any computer system, whether used or per-

formed either properly or improperly.  The user hereby agrees that he

is using this software/modifying his hardware at his own risk.

This and all of the other files contained in the file TTYRDR13.ZIP are

provided as is, with no warranties, express or implied, including but

not limited to the implied warranties of fitness for a particular pur-

pose, and all such warranties are expressly and specifically dis-

claimed.  In no event should the author or any of his associates be

responsible for any indirect, special, incidental or consequential

damages or lost profits to you or any other person or entity regard-

less of the legal theory, even if the author or any of his associates

have been advised of the possibility of such damage.

No person or entity is authorized to expand or alter either these war-

ranties or this agreement; any such representation will not bind the

author or his associates.

However, the author would appreciate any comments regarding the im-

provement of this software or any bug reports (nobody's perfect).  The

TTY Reader Version 1.3�                                          Page 7 of 9

author may be contacted on the following bulletin board systems that

he regularly calls:

The Cat's Meow BBS, Midland, Virginia             1-703-439-3060

Pinelands BBS, Vincentown, New Jersey             1-609-859-1910

(Name used on BBSs: 'MARK FINE')

He may also be contacted via any bulletin board system currently par-

ticipating in the following national BBS conference echos:

FidoNet SWL echo

RIME HAM and SCANNERS echoes (route using '->MEOW')

The author may also be contacted by mail at his home address (phone

calls are discouraged):

MARK J. FINE

c/o FineWare, Inc.

11252 Cardinal Drive

Remington, Virginia 22734

Comments should be constructive and should contain detailed descrip-

tions of what the user would like to see included in this software.

If a bug report is in order, please include information such as com-

puter make and model, version and make of DOS used, a hard copy of the

user's AUTOEXEC.BAT and CONFIG.SYS files, as well as detailed descrip-

tions of the nature of each problem.  All constructive comments and

bug reports will result in a prompt, courteous response.

APPENDIX A -

VERSION HISTORY:

Version 1.3� - Released 17 February 1991:

a)  Added CW Morse Code mode (Alt-F2).

b)  Added cursor in top line editing functions (filename, log,

etc.).

c)  Added toggle of screen border (Alt-O).  Toggling the screen

border off may reduce 'computer video noise' interference to the re-

ceiver.

d)  Added support for alternate command processors (i.e.: 4DOS).

e)  Added reminder to 'EXIT' in SHELL prompts.

f)  Fixed bug in GMT date/time calculation.  Upon exiting, the

software incorrectly reset the local date when it was the 31st of the

month.

g)  Fixed bug in SHELL (Alt-S) routine.  Software now returns to

the proper default drive and directory.

h)  Fixed bug in CR/LF handing in normal ASCII mode.  The soft-

ware would properly handle the line feed, but would ignore the car-

riage return.

Version 1.2� - Released 05 March 1990:

a)  Made documentation easier to understand.

b)  Converted software to quicker, more compact PASCAL code.

c)  Decreased time delay of opening screen display.

TTY Reader Version 1.3�                                          Page 8 of 9

d)  Utilized PASCAL windowing techniques for terminal format.

e)  Added border to aid visualization of terminal area limits.

Terminal area is now 21 lines tall by 75 characters wide.

f)  Moved Help key from Alt-F1 to Alt-H.

g)  Moved 56.92 baud Baudot Quick key from Alt-F2 to Alt-F1.

h)  Added Russian Cyrillic character set mode (Alt-F5).

i)  Added RTTY Debug Mode (Alt-D).

j)  Added RTTY Art Mode (Alt-A).

k)  User prompts for Log entries (Alt-L) and filenames (Alt-N)

were moved to the top line of the display.  Log entries are now lim-

ited to 75 characters, but now allow commas as part of the text.

l)  I/O Errors (i.e.: Disk, Printer) will be displayed on the top

line.  Will now prompt user to Retry, Abort, or Fail, instead of auto-

matically aborting, or disturbing the display.

m)  Shelling to DOS (Alt-S) now produces a one-time reminder to

type 'EXIT' to return to the program.

n)  Clearing the screen (F9) now only clears just the terminal

window, speeding up the process.

Version 1.1� - Released 05 January 1990:

First release of TTY Reader software.

APPENDIX B -

GRAPHIC SCREENS:  A file containing screen dumps of the opening

screen as well as the two help screens are also contained within this

archive.  They have been included as a quick reference to the various

TTY Reader commands.  If you have an Epson or IBM Graphics Printer,

these may be printed using the following DOS command:

COPY/B TTY.SCN PRN

Be sure to include the '/B' switch since TTY.SCN is a binary file.

TTY Reader Version 1.3�                                          Page 9 of 9