UNLOAD86.COM is a program that converts or "unloads" a file into it's INTeL

hex representation.  Alternately, it will also output a BNPF, BHLF, OR B10F

type file for those users that require these formats.  This program was

written in response to the need to have INTeL hex files for downloading to

an EPROM programmer.  As MS-DOS programs don't support INTeL hex, the need

to convert binary files to hex arose.  Most development systems, INTeL, CP/M,

& others do support INTeL hex, and most EPROM programmers expect it.

Use of the program is simple:

UNLOAD86  <RETURN>

prints a brief help screen.

The main form of the unload86 invocation is:

UNLOAD86 FILENAME.TYP [/OPTIONS]

B  output a BNPF file instead of INTeL hex.  This option overrides any other

selections.  If the B option is chosen, no other options will work, and the

program will output a straight 8-bit BNPF file, uless it has been modified

for BHLF, or B10F.

O Override extended addressing.  Any output file representative of 64K or

greater normally uses extended address records.  This is any file created

with /PC, /PD, /PE, or /PF, as well as any time the input file is longer

than 64K.  This inhibits the extended address records for the rare cases

where this is needed.  Interacts with /E.  Whichever is last is the effective

switch, normaly only one would be used at a time.

E forces the use of extended address records for file smaller than 64K.  Files

larger than 64K already automaticly select extended addressing.  Interacts

with /O.  Whichever is last is the effective switch, normaly only one would

be used at a time.

P allows selection of an EPROM size so that all unused locations are filled

with the user specified data (defaults to FF)  This option is followed

immediately by a parameter number 0-F.  This parameter is:

0  32 bytes.  Creates a file to fill a 32 byte ROM.  All unused locations

are filled with the user defined data.  As with all these options, if the

file is already longer than 32 bytes, an error message will be displayed

on the system console, and the operation will be aborted.  The file left in

this case is a useful INTeL hex file, but isn't padded up to size as it is

already too big for the device specified.  32 byte devices are generally

little bipolar ROMs often used as memory decoders

1  64 bytes.  This is also generally a bipolar device.

2  128 bytes.  Bipolar.

3  256 bytes.  1702A EPROM, some bipolars

4  512 bytes.  2704 EPROM.

5  1k bytes.  2708 EPROM.

6  2k bytes.  2716 EPROM.

7  4k bytes.  2732 EPROM.

8  8K bytes.  2764 EPROM.

9  16K bytes.  27128 EPROM.

A  32K bytes.  27256 EPROM.

B  64K bytes.  27512 EPROM.

C  *128K bytes.  27010 EPROM.

D  *256K bytes.  Not yet available.

E  *512K bytes.  Not yet available.

F  *1M bytes.  Not yet available.

* These options force the generation of extended addressing and thus

do not require the use of the 'E' option.  This may be overridden

with the 'O' option.

It should be noted that all INTeL hex output, regardless of options selected,

is padded up to the next 32 byte increment.

ERRORLEVEL support.  UNLOAD86 now supports ERRORLEVEL.  It exits with 0

for no errors, and 255 if there was some kind of error.

EXAMPLES:

UNLOAD86 FOOBAR.COM/B    <-- produces a BNPF file called FOOBAR.BNP

UNLOAD86 FOOBAR.COM/EB   <-- produces a BNPF file called FOOBAR.BNP

UNLOAD86 FOOBAR.COM/E    <-- produces a hex file called FOOBAR.HEX

w/extended addressing

UNLOAD86 FOOBAR.COM      <-- produces a hex file called FOOBAR.HEX

w/extended addressing if input >64K

UNLOAD86 FOOBAR.COM/P0   <-- produces a hex file called FOOBAR.HEX

padded to 32 bytes.

UNLOAD86 FOOBAR.COM/PF   <-- produces a hex file called FOOBAR.HEX

padded to 1M byte, w/ extended addressing

UNLOAD86 FOOBAR.COM/PFO  <-- produces a hex file called FOOBAR.HEX

padded to 1M byte, WITHOUT extended addressing

UNLOAD86 FOOBAR.COM/PFE  <-- produces a hex file called FOOBAR.HEX

padded to 1M byte, w/ extended addressing

UNLOAD86 FOOBAR.COM/EPF  <-- produces a hex file called FOOBAR.HEX

padded to 1M byte, w/ extended addressing

UNLOAD86 FOOBAR.COM/EPFB <-- produces a BNPF file called FOOBAR.BNP

Notes, patches, etc.:

UNLOAD86 cannot realisticly handle binary input larger than a megabyte as

the extended addressing scheme runs out of addresses.  If you unload a file

larger than 1 megabyte, the extended addressing will wrap around if it has

been selected with the /E or the /Px options.  When we get to a need for

INTeL hex files derived from files of this size, hopefully INTeL will have

come up with yet another patch to this specification.  Of course, you could

always use BNPF...

There are several user patchable options that can be patched with DEBUG.

They are as follows:

102:  FF   This is the data that is padded when using the /Px option.

Change to meet your requirements.

103:  'PN' Change to 'HL' for BHLF output instead of BNPF, or '10' for B10F.

Or any other two ASCII characters for other variations of this.

106:  'HEX' Output file type for INTeL hex.  Change to meet your needs

10B:  'BNP' Output file type for BNPF.  Change to 'BHL' for BHLF, or

'B10' for B10F, or to whatever filetype you prefer for this

format.

This program was written by Mark D. Pickerill on the Pro-Log ABL-2 CP/M

microcomputer. (what else?)  It bears no resemblance to UNLOAD 2.2 or

2.3 for CP/M by the same author.

UPDATES:

Ver. 1.2  After receiving multiple complaints, I changed the command line

parser so that spaces are now allowed between the filename and the switch

character.  Added - and [ as alternate switch characters for UNIX and CP/M

types.  Input files larger than 64K now automaticly generate extended

address records unless overridden with the /O switch.  /O also overrides

forced extended addressing on the larger /Px options.  Added ERRORLEVEL

support.

Ver. 1.1  Corrects a bug that dis-allowed zeros in the output filename.

An input file of ROM0.BIN would be unloaded to ROM.HEX instead of

ROM0.HEX.  Spurious bugs possibly could have also appeared if the input

filename had no extension. 10/3/88

Ver. 1.0  Initial release. 8/10/88

#########################################################################################################