USERS MANUAL

X.00 A Low Level Serial I/O Communications Driver

for MS DOS and like Operating Systems.

Copyright (c) 1990 by Raymond L. Gwinn

12469 Cavalier Drive

Woodbridge, Virginia 22192

All Rights Reserved

Oktober 02, 1990

Fidonet address 1:265/1

Data 703-494-8331

i

INTRODUCTION

Previously,  the documentation  for X00  was little  more  than a

collection of notes.  The notes were added  to as the development

process continued and enhancements were made.  I honestly did not

suspect  that X00  would become as  widely used  as it  is.  This

manual is an attempt at some decent documentation for X00.

But who  do I document  for?  The user  that is setting  up a BBS

just wants to get X00 installed  and to move on to more important

things.  The user that is having problems needs information about

serial  I/O communications  in general  and the  PC specifically.

The application developer (programmer) needs detailed information

about each individual function and the information generated.

This manual is intended for the X00 user.  A separate Application

Programmers manual is being written.

This manual and the software distributed with it is provided with

no guarantees.  Use at your own risk.

ii

TABLE OF CONTENTS

WHAT IS X00 . . . . . . . . . . . . . . . . . . . . . . . . .   1

Warranty  . . . . . . . . . . . . . . . . . . . . . . . . . .   1

Licensing Information . . . . . . . . . . . . . . . . . . . .   2

Non-Commercial License . . . . . . . . . . . . . . . . .   2

Commercial License . . . . . . . . . . . . . . . . . . .   2

GETTING STARTED . . . . . . . . . . . . . . . . . . . . . . .   4

Installing X00 . . . . . . . . . . . . . . . . . . . . .   4

Quick Start  . . . . . . . . . . . . . . . . . . . . . .   4

OVERVIEW OF X00 USAGE . . . . . . . . . . . . . . . . . . . .   5

General Information  . . . . . . . . . . . . . . . . . .   5

X00 loaded as a DOS Device Driver . . . . . . . . .   5

X00 loaded as a TSR . . . . . . . . . . . . . . . .   5

COMn and PORTn  . . . . . . . . . . . . . . . . . .   5

COMMAND LINE OPTIONS  . . . . . . . . . . . . . . . . . . . .   6

Specifying number of ports . . . . . . . . . . . . . . .   6

B option, Locking Baud, Data, Parity and Stop bits . . .   6

Valid locked baud rates  . . . . . . . . . . . . . . . .   7

Eliminate  . . . . . . . . . . . . . . . . . . . . . . .   7

Defer  . . . . . . . . . . . . . . . . . . . . . . . . .   7

DV . . . . . . . . . . . . . . . . . . . . . . . . . . .   8

FIFO . . . . . . . . . . . . . . . . . . . . . . . . . .   8

NOFIFO . . . . . . . . . . . . . . . . . . . . . . . . .   9

NOPOST . . . . . . . . . . . . . . . . . . . . . . . . .   9

SLOWCPU  . . . . . . . . . . . . . . . . . . . . . . . .   9

R, Set Receive Buffer Size . . . . . . . . . . . . . .    10

T, Set Transmit Buffer Size  . . . . . . . . . . . . .    10

VP or Master Interrupt Enable  . . . . . . . . . . . .    10

Mapping port assignments . . . . . . . . . . . . . . .    10

Un-installing X00  . . . . . . . . . . . . . . . . . .    12

Multi-Tasking . . . . . . . . . . . . . . . . . . . . . . .    13

THE BIOS INT 14h EMULATOR . . . . . . . . . . . . . . . . .    15

IRQ SHARING . . . . . . . . . . . . . . . . . . . . . . . .    16

Appendix A, Hardware Port and IRQ Assignments . . . . . . .    17

Addresses and IRQs for COM1 and COM2 . . . . . . . . .    17

Defacto standard for COM3 and COM4 . . . . . . . . . .    17

Addresses and IRQ for COM3 through COM8 on the PS/2  .    17

iii

Appendix B, SIO Chips . . . . . . . . . . . . . . . . . . .    18

8250 . . . . . . . . . . . . . . . . . . . . . . . . .    18

8250A  . . . . . . . . . . . . . . . . . . . . . . . .    18

16450  . . . . . . . . . . . . . . . . . . . . . . . .    19

16C451 . . . . . . . . . . . . . . . . . . . . . . . .    19

16550 Non A  . . . . . . . . . . . . . . . . . . . . .    19

16550A . . . . . . . . . . . . . . . . . . . . . . . .    19

16550AF  . . . . . . . . . . . . . . . . . . . . . . .    19

16550AFN . . . . . . . . . . . . . . . . . . . . . . .    19

16550s Made by Western Digital . . . . . . . . . . . .    20

16C551 . . . . . . . . . . . . . . . . . . . . . . . .    20

16C552 . . . . . . . . . . . . . . . . . . . . . . . .    20

82510  . . . . . . . . . . . . . . . . . . . . . . . .    20

Appendix C, Multi-Port Serial I/0 Cards . . . . . . . . . .    21

Connect-Com 4  . . . . . . . . . . . . . . . . . . . .    21

4 and 8 Port Boards from Decision-Computer . . . . . .    21

Stargate Plus 8 board from Software Link . . . . . . .    22

INDEX . . . . . . . . . . . . . . . . . . . . . . . . . . .    23

1

WHAT IS X00

X00 is  a Serial Input/Output  (SIO) communications  driver.   It

provides  an interface  between  an application  program and  the

serial communications hardware.

X00 was  originally designed as  a FOSSIL  driver.  FOSSIL  is an

acronym containing the first  character of several communications

programs.   The FOSSIL specification is basically an extension of

the PC's BIOS services for serial communications.

Except as noted in  the Programmers Manual, X00 meets  the FOSSIL

level 5  specification.   X00 provides additional  functions that

are not contained in the FOSSIL level 5 specification.

X00 only works with 8250 (TYPE) serial I/O devices.  Such devices

include the 8250A,  16450, 16550, 16550A and  the 82510.   If you

have a PC that  is an IBM or  near compatible which has  a serial

communication  port, it is likely  that it contains  one of these

devices.  XU, a  utility distributed with X00, will  identify the

existence of compatible SIO devices.

Like  other  Device Drivers,  X00  will do  very  little standing

alone.  There must be an application(s) program to use X00 before

you will gain benefit.

Since  the introduction  of the  FOSSILs, a  great number  of new

communications  programs have  come into  existence.   I like  to

think  that the FOSSILs are  largely responsible for  this.  Many

programs that contain their own communications drivers have added

FOSSIL support.  One notable example is RBBS.

Many  programs  written  to  use  BIOS's   serial  communications

services will also work  with X00.  This gives those programs the

benefits of buffered interrupt driven I/O.

Warranty

There is none.  Use at your own risk.

2

Licensing Information

Non-Commercial License:

A  non-commercial license  for  use of  X00  is included  in  the

distribution file called LICENSE.TXT.

I  have received  several unsolicited  checks from users  of X00.

All  of the checks  have been returned  or uncashed.   To date, I

have not accepted any payments for X00.  If you feel that  X00 is

useful  enough that some sort  of payment is  justified then read

on.

Our  local (public)  High School  desperately needs  funds.   The

school officials work at keeping the students off the streets and

away from  drugs.   The local School  Board is unable  to provide

needed  funds for  non-academic activities.   I suggest  a $10.00

contribution be sent to:

Woodbridge Senior High School

3001 Old Bridge Road

Woodbridge, Virginia, 22192

You  can make the  check payable to  "WSHS, Ballfield Improvement

Fund".   Please make a  note "X00 contribution"  somewhere on the

check.

Many of you have used X00 for years.  I believe  $10.00 is easily

affordable to most X00 users.   Please take the time to sit down,

make out the check, address the envelope and mail a contribution.

Shareware authors that wish to  distribute X00 with their product

should contact me.  Permission will usually be granted.

Commercial License:

Commercial Licenses will  be liberally  granted in  return for  a

donation (usually tax deductible).   Each commercial license will

be individually granted.  That is, there is no single policy that

can be stated  here.  Some  generally uniform commercial  license

policies follow.

1 - X00 is usually licensed by product.

2 - All  commercial licensees of X00 will receive  a special copy

of X00 that will identify, at boot time, the commercial product.

3 -  I may require a fully licensed, fully featured, copy of your

product for personal use on multiple computers.

3

4 - The required  contribution, which is never to  me personally,

will usually be a one time contribution of $100.00.

5 -  Proof of licensing belongs to you.   That is, I am not going

to be responsible for keeping records.  I suggest that you keep a

copy of the donation  check so that you can  send me a copy  if a

question  arises.    Otherwise  you  may  have  to  make  another

contribution.

6  - You may  only distribute versions  of X00 that  contain your

product name in the X00 banner (at boot time).

7 - If you wish  to have the latest version of  X00 automatically

sent to you with your product name in the X00 banner, then do the

following:  Send me a check  for $10.00, in advance, along with a

pre-addressed mailing label.  If you require a 3.5 inch diskette,

then write "3.5 inch" on the pre-addressed mailing label and send

$15.00 (instead of  $10.00).   For updates, the  check should  be

made  out to Raymond  L. Gwinn.   The checks will  be cashed when

received.  Individuals that wish to receive a mailed  copy of the

latest X00 version may  also use this  procedure.  Once you  have

received an  update, you will need  to resend the  update fee and

pre-addressed mailing  label if  you desire  to receive the  next

update.

8  -   Your  contribution  check  should   state  "X00  Licensing

contribution" in the Memo or comment field.

If  you write  me about  a commercial  license, please  include a

voice phone number.  You may  not receive a response if the voice

phone number is omitted.  Correspondence should be sent to:

Raymond L. Gwinn

12469 Cavalier Dr.

Woodbridge, Va. 22192

Prior to making the special version of X00 available, I  may wait

for  the check to clear the bank.   If the check bounces, I will,

most  likely, not give any further  consideration to a commercial

license.

4

GETTING STARTED

Installing X00

X00  is distributed  as a  ZIP file.   To extract  the individual

files  from the distribution file you need the program PKUNZIP or

an  equivalent.   PKUNZIP  is  a  product of  PKWARE  and  can be

downloaded from  almost any  bulletin board  system  (BBS).   You

probably already have PKUNZIP and know how to use it, otherwise I

doubt that  you would be reading  this.  When PKUNZIP  is used to

extract the various files, -AV should appear after each file name

as it is extracted.  In addition, the message:

Authentic files Verified!

should appear after the extraction process.  If this message does

not appear, or  appears with a name other than  Raymond L. Gwinn,

the file is not an original X00 distribution file.

The  only file  that the  basic user  actually needs  is X00.SYS.

Copy  this file to a convenient directory or subdirectory on your

system.   I suggest that all users  also install XU.EXE also.  XU

can provide important  information about X00 that  may be helpful

when problems occur.

Quick Start

Copy X00.SYS and XU.EXE to the root  directory of your boot drive

or diskette.

If you  are going  to use  standard communications  ports, either

COM1 or COM2 on  a PC or AT, or COM1 through COM8 on a PS/2, then

add the following line to your CONFIG.SYS file:

DEVICE=X00.SYS E

If  you wish  to have  more than  one communications  port active

simultaneously,  then  modify the  command  line  to include  the

number of simultaneously active  ports you wish to support.   For

example,  if you are using DESQview and you  wish to run a BBS in

two  different  windows,  then  add the  following  line  to your

CONFIG.SYS file:

DEVICE=X00.SYS E 2

It  should be  noted that the  above command  tells X00  how many

ports that are to  be used simultaneously, not which  ports.  The

application program may use ANY 2 serial ports in any order.

If your setup is more complex, you must read on.

5

OVERVIEW OF X00 USAGE

General Information

Things  can get  a little confusing  here.   At the  time of this

writing,  X00 is almost (if not completely) unique in its ability

to function as a DOS device driver or as a Terminate (or Torment)

and Stay Resident program (TSR).

X00 loaded as a DOS Device Driver

If X00 is to be used as a DOS device driver, it must be installed

using  the CONFIG.SYS  file.   For a  precise description  of the

CONFIG.SYS file  and  its  function, please  refer  to  your  DOS

manual.  Basically,  you use a text  editor to add a  line to the

CONFIG.SYS  file.    The  line  that  you add  is  DEVICE=X00.SYS

followed by optional parameters.  The optional parameters will be

described later.  If X00.SYS is  not in the current directory you

will need to provide a DOS path  to the directory containing X00.

For  example, if X00.SYS is in  a subdirectory called COMM on the

hard  disk drive  C, the  line  in the  CONFIG.SYS file  would be

DEVICE=C:\COMM\X00.SYS.

X00 loaded as a TSR

If  X00 is  to be  used as  a Terminate  and Stay  Resident (TSR)

program,  then you must rename  or copy X00.SYS  to X00.EXE.  The

EXE  version of  X00 must  be in  the current  directory or  in a

directory  in the  DOS  path when  it is  executed.   X00.EXE  is

executed like  any other DOS program.   That is, you  type X00 at

the  DOS prompt followed by any optional parameters.  In reality,

you will probably be using a batch file to execute X00.

COMn and PORTn

X00 allows communications ports  to be swapped around.   That is,

the serial port that DOS calls COM1 can be mapped (reassigned) to

any X00 port.   Therefore, when a  reference in this  document is

made about  PORTn, it  refers to  the serial  I/O device  that is

mapped to  that X00  port.   When  a reference  is  made to  COM1

through  COM8,  it  refers to  the  serial  I/O  device that  DOS

knows/uses as COMn.   Also worth noting  is that DOS numbers  the

serial  I/0 devices  starting at  1 (COM1  through COM8)  and X00

numbers them starting at 0 (PORT0 through PORT7).

X00's  default is to assign  PORT0 through PORT7  to COM1 through

COM8.

6

COMMAND LINE OPTIONS

The  command line options are  included in the  command line that

invokes X00.  In several  cases only the first 1 or  2 characters

of the option need be entered.  In other cases, the entire option

name  must  be  spelled out.    Unless  otherwise  stated in  the

following descriptions,  the entire  option name must  be spelled

out.  The parameters are given one after another on the same line

and in any order.  The length of the command line is limited only

by DOS.

Specifying number of ports

If a  single  decimal number  is  in  the X00  command  line,  it

specifies  the number  of  ports that  X00  is to  simultaneously

support.   The default is  one port.   The maximum  value of  the

number of ports is 4 for non PS/2 systems and 8 for PS/2 systems.

If you  desire to use more than 4 ports  on a non PS/2, each port

address must  be specified (see mapping  port assignments below).

Input  and output  buffers are  allocated for  each of  the ports

specified.  Examples of usage are:

When X00 is loaded as a Device Driver

DEVICE=X00.SYS 3

DEVICE=X00.SYS 8

When X00 is loaded as a TSR

X00 3

X00 8

B option, Locking Baud, Data, Parity and Stop bits

Normally, the  application program sets  the baud rate  and other

parameters  controlling the  serial  I/O hardware.   Some  modems

provide  better  performance  when  the  communications  from the

computer to modem is constant (Locked).  That is, the computer to

modem speed is not the same as the modem to modem speed.  If this

option  is   used,  the  modem  must   support  RTS/CTS  hardware

handshaking and the feature must be enabled on the modem.

The B command  line option  requires 2 parameters  and allows  an

optional  third parameter.  The  option is specified  in the form

B,PORTn,BAUD  RATE,PARITY.  You  will most  likely never  want to

lock the parity, data and stop bits.  Examples of usage are:

When X00 is loaded as a Device Driver

DEVICE=X00.SYS B,0,2400 B,1,19200 or

DEVICE=X00.SYS B,1,19200,8N1

When X00 is loaded as a TSR

X00 B,0,2400 B,0,19200 or

X00 B,1,19200,8N1

7

Valid locked baud rates are: 75, 110, 135.5, 150, 300, 600, 1200,

1800, 2000,  2400, 3600, 4800,  7200, 9600, 19200,  28800, 38400,

57600, and 115200.

I can  think of few conditions  where it is necessary  to use the

optional  third  parameter of  the B  command  line option.   The

optional  third parameter is made up of 3 characters that specify

the  locked setting for data  bits per character,  parity bit and

stop bit(s) in  that order.  The optional  third parameter can be

made up of  any combination  of the following  first, second  and

third characters.

First     Second         Third

5         N = None       1

6         E = Even       2

7         O = Odd

8         M = Mark (always 1)

S = Space (always 0)

The first  character  specifies  the  number  of  data  bits  per

character,  the second  character specifies  the handling  of the

parity bit and the  third character specifies the number  of stop

bits.   When 5 data bits and  2 stop bits (eg 5N2) are specified,

the  number of stop bits will actually  be 1.5.  As stated above,

locking of the parity etc, is not advisable.

Anytime an application program attempts to program the baud rate,

parity etc, through X00, the locked values will be substituted.

Eliminate

Normally,  when X00 is loaded a copyright notice is displayed for

5 seconds.  Using the eliminate  option will disable the 5 second

display  of the copyright  notice.  Only the  first letter (E) is

required to invoke this option.  Examples of usage are:

When X00 is loaded as a Device Driver:

DEVICE=X00.SYS ELIMINATE or

DEVICE=X00.SYS E

When X00 is loaded as a TSR

X00 ELIMINATE or

X00 E

Defer

The Defer option  creates problems  on most systems.   The  Defer

option should be avoided by most users.  X00 must  use several of

the  80x86 interrupts.  Normally,  X00 will hook  itself into the

80x86 interrupts when it is executed.  The Defer option tells X00

8

to  postpone hooking  into some  interrupts until  an XU  CAPTURE

command  is  executed.    If  the defer  option  is  used  and  a

subsequent  XU  CAPTURE  is  not  issued,  X00  may not  function

correctly.  Additionally,  the XU CAPTURE must be  executed prior

to  executing  a multitasker  like DESQview.    Only the  first 2

letters (DE) need  to used to specify  this option.   Examples of

usage are:

When X00 is loaded as a Device Driver

DEVICE=X00.SYS DEFER or

DEVICE=X00.SYS DE

When X00 is loaded as a TSR

X00 DEFER or

X00 DE

The  default for  Defer is  off.   That is,  X00 will  by default

immediately hook into all necessary 80x86 interrupt vectors.

DV

The  DV command  line option is  only of interest  to those using

DESQview.   The  default for  this option  is off.   When  on (DV

specified  in the  command line),  X00 will  make DESQview  Pause

calls when it seems that  the application program is idle.   This

may make  time available for other  tasks to use.   Refer to your

DESQview manuals for details about the DESQview pause command.

If you are  using DESQview, try  the DV option.   If it does  not

adversely affect your system, leave it enabled (on).  Examples of

usage are:

When X00 is loaded as a Device Driver

DEVICE=X00.SYS DV

When X00 is loaded as a TSR

X00 DV

FIFO

Only the first  letter of the command  (F) is required to  invoke

this option.  Some of the serial I/O hardware  devices have First

In First Out (FIFO) buffers.  This command line option is used to

specify the  number of characters that X00 can load into the FIFO

during  a single  transmit  interrupt.   In  general this  number

should be set  the size of the  FIFO minus 1.   For example,  the

popular 16550A has a 16  byte FIFO on both transmit and  received

data.  Thus, if this option is used, I suggest the value of 15 be

used for the 16550A.  Examples of usage are:

When X00 is loaded as a Device Driver

DEVICE=X00.SYS FIFO=10

9

DEVICE=X00.SYS F=15

When X00 is loaded as a TSR

X00 FIFO=15

X00 F=8

The size of the receive FIFO is always set to the maximum by X00.

NOFIFO

This command line option is used to completely disable the use of

hardware FIFOs.  Examples of usage are:

When X00 is loaded as a Device Driver

DEVICE=X00.SYS NOFIFO

When X00 is loaded as a TSR

X00 NOFIFO

NOPOST

Normally,  when X00 is executed,  it will alter  the COM1 through

COM4 hardware port addresses  in BIOS RAM to match  the addresses

being used by X00.  By doing so, many DOOR type programs that use

INT  14h  for  their serial  I/O  will  work  correctly with  X00

installed.   This option (NOPOST) instructs X00 to leave the BIOS

RAM unchanged.  Examples of usage are:

When X00 is loaded as a Device Driver

DEVICE=X00.SYS NOPOST

When X00 is loaded as a TSR

X00 NOPOST

SLOWCPU

This command line option  is provided for those using  slower PCs

and those using software  that executes large amounts of  code on

an interrupt  level (such  as the timer  tick).  When  SLOWCPU is

specified,  X00 will make  the serial ports  the highest priority

interrupt.     This  feature  allows   (most)  4.77  mhz  PCs  to

receive/transmit at full  speed.  This option  may cause problems

in  some  environments.    Most  protected  mode  programs,  like

DESQview, will not  allow the  priority of the  interrupts to  be

changed.

When X00 is loaded as a Device Driver

DEVICE=X00.SYS SLOWCPU

When X00 is loaded as a TSR

X00 SLOWCPU

10

R, Set Receive Buffer Size

The R command line option allows the user to set the  size of the

receive buffer.   The default size  of the receive buffer  is 512

bytes.   The size  of the  buffer must  be a power  of 2.   Valid

buffer sizes are:  256, 512,  1024, 2048, 4096,  8192, 16384  and

32768.   If an invalid buffer  size is specified, the  next lower

correct buffer size is used.

Many users needlessly set  the size of the receive  (or transmit)

buffer too large.  Examples of usage are:

When X00 is loaded as a Device Driver

DEVICE=X00.SYS R=1024

DEVICE=X00.SYS R=2048

When X00 is loaded as a TSR

X00 R=1024

X00 R=2048

T, Set Transmit Buffer Size

This option is identical to the R command line option (see above)

except it sets the size of the transmit buffer.

VP or Master Interrupt Enable

VP stands for Vector Port which  is not a very good name for  the

option.  Some multiple port serial I/O cards default  to only two

ports enabled when power is turned on.  A specified value must be

written to an  I/O port  to enable the  additional serial  ports.

Simply stated the VP  command line option says to write the given

value to the given port.  Examples of usage are:

When X00 is loaded as a Device Driver

DEVICE=X00.SYS VP=03FF,80

When X00 is loaded as a TSR

X00 VP=3FF,80

Both  of the above  examples in effect  says, write the  value 80

(hex) the I/O port at 3FF.

Mapping port assignments

This  is the most complicated command line option for many users.

By default,  X00 maps up to  8 serial I/O ports  (PORTS 0 through

7).  X00 will (attempt to) determine the hardware architecture on

11

which  it is  being executed.   If  a PS/2  is detected  then the

default Serial I/O  port addresses will be set to  match the PS/2

(see Appendix .   If X00  detects that the  system is not a  PS/2

type system, then the default port addresses are as follows:  1 -

Standard  addresses for COM1 and  COM2, 2 -  The defacto standard

for COM3  and COM4,  and nothing for  COM5 through COM8  (ports 4

through 7).

Port mapping is necessary if your hardware configuration does not

match the PS/2 or  defacto standard for  non PS/2 systems.   COM1

and COM2  are the  same for the  PC, XT,  AT and PS/2.   However,

hardware  specifications for COM3  through COM8  became available

only when IBM released the PS/2.

Many  multiple  serial port  cards  are  available for  pre  PS/2

systems that use a defacto standard.  The most common assignments

that I have  seen for COM3 and COM4 are: COM3 at port address 3E8

using  IRQ 3.  COM4  at port address 2E8 using  IRQ 4.  Above the

fourth port,  I have not  observed anything resembling  a defacto

standard.

X00 will  support almost any serial  I/O card.   However, in most

cases,  port mapping  is required  for anything  beyond  COM1 and

COM2.

Mapping port assignments uses two basic forms.  They are:

PORTn=COMn,IRQn

PORTn=HEXADDRESS,IRQn

Where:

PORTn is the X00 port number and must be 0 through 7.

COMn is a standard DOS port and must be COM1 through COM8.

IRQn is the IRQ use by the serial port and must be IRQ0

through IRQ15.

HEXADDRESS is a hexadecimal I/O port address that addresses

the serial I/O device.

In both basic forms, the IRQn is optional.  X00 defaults all IRQn

to IRQ3  except COM1 which  defaults to IRQ4.   Examples of usage

are:

When X00 is loaded as a Device Driver

DEVICE=X00.SYS 0=COM4

DEVICE=X00.SYS 0=COM4,IRQ5

DEVICE=X00.SYS 0=2E8,IRQ3

DEVICE=X00.SYS 2=2E8

When X00 is loaded as a TSR

X00 0=COM4

12

X00 0=COM4,IRQ5

X00 0=2E8,IRQ3

X00 2=2E8

Un-installing X00

When X00 is installed as a TSR, it can be  un-installed by simply

executing X00 with no command line parameters.  If you wish to be

sure that you do not accidently un-install X00, always execute it

with a harmless parameter like X00 E.

13

Multi-Tasking

By  far, DESQview seems to be the multi-tasking software that X00

is  most  often used  with.    At the  time  of  this writing,  I

personally use DESQview to run a multi line BBS.

I believe it is best to load X00 before the multi-tasking program

and  to load  it as a  device driver.   This  enables all windows

(tasks)  access to X00.   Loading X00 first  also allows multiple

windows to share the same IRQ.

If  you have loaded X00 prior to  DESQview, you need to configure

the window  for no  communications.   That is,  answer NO  to the

configuration   questions,   Uses  Serial   Ports   and  Optimize

communications.     I  also  suggest  that   you  enable  printer

management within the window.  Enabling printer management allows

a  remote drop  to DOS  to work  on many  systems.   The Optimize

communications  option and  the  Printer  management  option  are

established with  the DESQview  SETUP program.   The  Uses serial

ports  (Y,N,1,2) option is in  the "Change a  Program" portion of

window configuration.

In  several  cases  I  have  gained  a  significant  increase  in

performance by turning X00 on prior  to executing a program.  Two

notable cases are GATEWAY and DOORWAY.  I used XU to turn X00 on.

For example, XU PORT:n:ON just before invoking DOORWAY or GATEWAY

produced significantly better input and output from the programs.

Depending  on your environment, an  XU PORT:n:OFF may  have to be

issued after  the program terminates.   Also note that the  n for

PORTn is 1 less than the n in GATEn and COMn.

The communications windows seem  to work better if the  ticks for

the  foreground and  background  are set  relatively  low.   Most

people that  I have talked to use 2 ticks for both foreground and

background.  Only experimentation will determine the best setting

for your system.

If you do  not wish to share  an IRQ, you can  load X00 as a  TSR

inside  a DESQview window.  When loaded inside a DESQview window,

X00 is  only available to programs in that window.  X00 should be

configured  to  support   only  1  serial  port  and  you  should

specifically name the port that X00 will use in the command line.

For example use X00 0=COM1,  instead of X00 1 or just  plain X00.

Identifying the specific  serial port  to be used  in the  window

eliminates the chance that X00 will interfere with another active

window.

I strongly recommend that you use the 16550A (or later)  SIO chip

if you are using a multi-tasking system of any kind.

14

Most programs written to use BIOS INT 14h will work directly with

X00's enhanced services.  You should try turning  X00 on using an

XU  PORT:n:ON  command  just  prior  to  executing  the  program.

Programs  that  do not  have their  own  interrupt I/O  will most

likely give better performance.

15

THE BIOS INT 14h EMULATOR

X00 is a  BIOS INT 14h enhancement.   So what  is the need for  a

BIOS  emulator also?   Well, when X00  is turned off  for a given

port,  it  does  not process  the  INT  14h  functions using  the

enhanced code.   Prior  to the introduction  of the BIOS  INT 14h

emulator, X00 simply passed calls to inactive serial ports to the

BIOS INT 14h.

Serial   communications  has   changed  considerably   since  the

definition of  the original BIOS for  the PC.   However, the BIOS

services for  serial  communications have  changed  very  little.

Many  program are still written  to use the  original BIOS serial

communications  services.  Many older  programs that use the BIOS

communications services are still widely used.

Problems occur  when advanced  high  speed modems  are used  with

these programs.  Many of the  programs are totally unaware of the

concept of RTS/CTS handshaking that most of the high speed modems

use.  The programs cannot be aware that the user  may have mapped

the port  addresses.   BIOS  for  all systems  (that  I know  of)

support only  4 serial  communications ports except  PS/2 systems

which support 8.

I added  the BIOS emulator to X00 to support many of the programs

that  are not (or cannot  be) aware of  the active communications

environment.

By default,  X00's BIOS INT 14h  emulator is enabled.   It can be

disabled by adding USEBIOS to the command line that invokes X00.

When the emulator is enabled, port mapping, baud rate locking and

hardware handshaking for the serial  port(s) are maintained and 8

serial ports are supported.   If the serial  I/O chip is  FIFOed,

the  emulator allows the FIFOs to be  used.  This means that many

programs  written to use  BIOS can be  used in  the more advanced

communications environments.

The BIOS  emulator can  be activated and  deactivated dynamically

using XU.

16

IRQ SHARING

Sharing the same IRQ for multiple communications ports is not for

the weak of heart (or mind).

Most popular  multitasking software  seems to do  everything they

can to  prevent the sharing of  a single IRQ between  two or more

tasks (windows).   This big brother attitude  of the multitaskers

is cause of most of the  problems in sharing a single IRQ between

several  serial  communications  ports.    Several communications

ports  sharing a single IRQ is not  too difficult to manage.  The

problem is the multitaskers not allowing the sharing of an IRQ by

more than one task (window).

In any  case, version 1.20 of X00  has been tested with DESQview,

RBBS  and  8 comm  ports  sharing  IRQ 3.    I  have also  tested

MultiLink  with  2  comm  ports  sharing  the  same  IRQ.    With

MultiLink, I had to use  the NASTY option.  Do not  use the NASTY

option with DESQview.

The  method  I  have developed  to  force sharing  of  an  IRQ by

multiple  comm   ports  should   work  on  most,   if  not   all,

multitaskers.  It is too big a job, and too costly for me to test

them all.

Some general rules are as follows:

1  - X00 should  be loaded before  the multitasker.   That is, if

your multitasker allows the  loading of device drivers  into each

window, DO NOT DO  IT if you wish to  share an IRQ.  Load  X00 at

boot time and it should then be available to all tasks (windows).

2 - Two different  boards (plugged into different slots)  can not

share the same IRQ.  This is a hardware limitation and can not be

corrected  by software.  This means that  you can not plug in two

internal  modems, set them  for the same  IRQ and  expect them to

work.   This  does not  apply to  PS/2 systems.   The  problem of

multiple  boards sharing the same IRQ was corrected in the PS/2's

buss design.

3 -  When  configuring  the DESQview  windows,  we  specified  NO

communications  in the  window.   Remember, it  is X00  (which is

hopefully  outside   any  window)   that  is  doing   the  actual

communications and not the application program inside the window.

4 - X00 can not  (will not) share an IRQ with other software.  If

you attempt to run a program in a window that uses an IRQ that is

being used by X00, the results are unpredictable.

5 - Do not load X00 into EMS memory.  Loading high is okay.

17

Appendix A, Hardware Port and IRQ Assignments

I have  obtained the following information  from various sources.

Accuracy is not guaranteed.  In fact, nothing in this document is

guaranteed.

Addresses and IRQs for COM1 and COM2

COM1 uses 03F8h and IRQ4

COM2 uses 02F8h and IRQ3

Defacto standard for COM3 and COM4 for PC, XT and AT

COM3 uses 03E8h and IRQ4

COM4 uses 02E8h and IRQ3

Addresses and IRQ for COM3 through COM8 on the PS/2

COM3 uses 3220h and IRQ3

COM4 uses 3228h and IRQ3

COM5 uses 4220h and IRQ3

COM6 uses 4228h and IRQ3

COM7 uses 5220h and IRQ3

COM8 uses 5228h and IRQ3

All  PCs prior to the PS/2 only  used 12 bits to address hardware

I/0 ports.   Systems prior to  the PS/2 and EISA  can not address

the standard  addresses for COM3 through COM8.  If an AT of below

attempts  to  address  COM3   through  COM8  using  the  standard

addresses, only  the low 12 bits  of the address are  used.  That

is,  the high digit of  the hexadecimal address  will be ignored.

This  means that any reference to COM3 through COM8 will actually

address ports 220h through 22Fh.

You  should avoid  expansion boards that  use ports  220h through

22Fh unless the expansion board is a serial I/O board.

X00 will (attempt to) determine the hardware architecture that it

is being executed  on and  use to appropriate  port addresses  as

defaults.

18

Appendix B, SIO Chips

The following is a collection of comments developed from hearsay,

random documents, experience,  and technical specifications.  So,

take it with a grain of salt.  Order information from the various

manufactures if you want accurate information.

A very good reference book that I use is  available from National

Semiconductor Corporation.   It  covers most  of the  SIO devices

(also  called UARTs)  that you will  find in all  PCs and clones.

The title  of the book is  Microcommunications Elements Databook.

The publication number (manual number) is  400066.  I am not sure

that   the   book  can   be   ordered   directly  from   National

semiconductor.  They  may   require  that  you  get   it  from  a

distributor.   In any case, the address on the back of the manual

is:

National Semiconductor Corporation

2900 Semiconductor Drive

P.O. Box 58090

Santa Clara, CA 95052-8090

Tel:(408)721-5000

TWX:(910)339-9240

8250

As best that I know, the 8250 was the first  SIO chip (integrated

circuit) that was  used by the  IBM PC  and many clones.   In  my

opinion,  it was a poor choice  on the part of IBM.   I feel many

superior devices, at comparable  prices, were readily  available.

At that time, I feel the 8251A or the 8530 would have been better

choices.   I feel IBM  used the 8250  to insure the PC  would not

have   synchronous  communications   capability  in   a  standard

configuration.

From  a hardware standpoint the 8250 is a relatively slow device.

It is advisable that programmers not perform successive inputs or

outputs to this device.  It seems that software programs can load

the various registers of the 8250 faster  than it can process the

information.    The  8250  had  a  total  of  7 registers.    The

specifications state that 56kb is the maximum baud rate.

8250A

I believe the 8250A is the  8250 with some bug fixes.  I  have no

idea  what  the bugs  may  have  been.   A  quick  glance at  the

specifications shows the speed  of the 8250 and 8250A  to be much

the  same.   The 8250A  added an  8th register.   This additional

register enables software to detect if an 8250 is installed.  The

specifications state that 56kb is the maximum baud rate.

19

16450

The  16450 seems to be a speeded up  version of the 8250A.  There

is no  direct way (that  I know  of) for software  to detect  the

difference between an 8250A and a 16450.  I believe the 16450 was

developed to  eliminate the  need for  software to  insert delays

between successive  accesses to  the device.   The specifications

indicate the 16450 is a much faster device than its predecessors.

The additional speed is only the speed at which the processor can

access the  device.  The maximum baud rate for the 16450 is still

stated at  56kb.  However, I  have been told by  some people that

they have run the  16450 successfully at much higher speeds. I do

not believe there was ever a 16450A.

16C451

The  16C451 is a CMOS version  of the 16450.  CMOS  is a term for

the  material and  manufacturing process  used to make  the part.

CMOS typically uses less  power than other technologies.   If you

are  not  designing hardware,  you should  view  the 16C451  as a

16450.

16550 Non A

It is  hard to  find a 16550  (Non A).   I  was told by  National

Semiconductor that  they did  everything  they could  to get  all

16550s  back.  X00 (and  XU) will detect a 16550  and tell you if

you have one.   I am told  that the 16550 was  installed in early

PS/2 systems.

The 16550  was the first  shot at  a FIFOed version  of the  8250

family  from  National semiconductor.    However, I  was  told by

National  Semiconductor  that the  FIFOs  of  the 16550  are  not

reliable and  they should not be enabled.  X00 will treat a 16550

like  a 16450.    In  this mode,  they  are  reliable.   National

Semiconductor would not provide  me with a specification for  the

16550.  However, I suspect  its maximum baud rate is the  same as

the 16550A which is 256kb.

16550A, 16550AF and 16550AFN

In the  manuals  that I  have,  National Semiconductor  does  not

explain  the differences between the  16550A and the  16550AF.  I

suspect the AF part may have a few bug fixes.  I believe the N in

AFN  describes packaging,  ceramic  versus  plastic,  DIP  versus

surface mount etc.

In  the opinion  of the  author, there is  no substitute  for the

16550A (and its  successors) in the 8250 type series.  The 16550A

is compatible with software written for the entire family of 8250

20

type  devices.  Programs that  are 16550A aware  can provide much

improved performance over previous devices.

The  maximum  baud rate  for the  16550A  is specified  at 256kb.

However, due to the hardware design of the PC et al, 115kb is the

maximum baud rate that can be programmed.

The 16550A can be  plugged into the same socket that  contains an

8250, 8250A  or 16450.  If  your SIO expansion board  has the SIO

chips  in  sockets,  you can  upgrade  to  the  16550A by  simply

removing the old chips and replacing them with 16550As.

The key to  the performance increase of the 16550A  is its FIFOs.

It has 16 byte FIFOs for both transmit and receive data.

16550s Made by Western Digital

I have been told, but I have not verified for myself, that 16550s

made  by Western  Digital have  a problem  with their  FIFOs when

working at 2400 baud or below.

16C551

The  16C551 is  a CMOS  version of  the 16550AF.   See  the above

description of  the 16C451 for a discussion of CMOS.  Do not feel

you need to upgrade from a 16550A or AF to a 16C551.  There is no

gain from an existing users point of view.

16C552

The 16C552 is two 16C551s  on a single chip.  These  devices will

start showing up on multi-port serial I/O cards.

82510

I  believe Intel is the only company that manufactures the 82510.

The 82510  is feature rich with several  modes of operation.  Its

default  mode is to operate as  a 16450.  The 82510  has a 4 byte

FIFO  for both  transmit and  receive  data.   A 4  byte FIFO  is

sufficient to provide significant performance over a basic 16450.

The  82510 is small in size.  Therefore,  it is found in many lap

tops.

The 82510 is  somewhat of a sleeper.  I believe  it would be much

more widely used if Intel had promoted it more.  However, given a

choice  between  the 82510  and the  16550A,  I would  select the

16550A.

21

Appendix C, Multi-Port Serial I/0 Cards

I  get many requests for information  about multi-port serial I/O

card.   I will describe  the boards  that I know  about (or  have

heard about) here.  I  will add to the list as  I receive precise

information from users.

My  personal experience  is with two  boards that  allow multiple

serial I/O ports to share  a single IRQ.  The first board  that I

experimented with  was the  Connect-Com 4  from Software  Link in

Atlanta Georgia.  The second board is the 4 PORT RS-232 CARD from

Decision-Computer International in Taipei, Taiwan.

Connect-Com 4

I found the manual for the Connect-Com 4 to be very confusing and

their  tech support  even less helpful.   I  was able  to get the

board  configured to  use  the standard  COM1  and COM2  and  the

defacto standard for COM3  and COM4.  If you use  the Connect-Com

4, I suggest  that you use it  in this manner.   This means  that

COM1 and COM3 will use IRQ4 and COM2 and COM4 will use IRQ3.  The

Connect-Com 4  has an  8 position DIP  switch for  configuration.

The setting that I used was positions 2, 3, 7 and  8 ON (up), and

all  other positions OFF (down).  In retrospect the Connect-Com 4

seems  a bit expensive.   However, once  configured properly, the

Connect-Com  4  worked without  problem.   Software  Link  can be

reached in Atlanta Georgia at 404-448-5465.

4 and 8 Port Boards from Decision-Computer

The second  board that I have personally used, and the one that I

am  currently using  is from  Decision-Computer.   The  manual is

written  well enough that I  did not need  any technical support.

There are two boards of interest from Decision-Computer, one is a

4 port  serial I/O card  and the  other is an  8 port  serial I/O

card.   I read  somewhere in the  documentation (which  I can not

find  at this  time)  that  the  boards are  a  clone  of  boards

available from AST.

The boards from  Decision-Computer are configured by jumpers.   I

use 4  ports of  the 8  port card.   I  selected the  (so called)

Compatible Mode.  This means that the first two  serial I/O ports

operate   as  standard  COM1   and  COM2  using   IRQ4  and  IRQ3

respectively.  The jumper setting for Compatible Mode is  clearly

shown  in  the manual  (page  8  of my  manual).    The I/O  port

addressed used in that mode are  also clearly defined on the same

page.

A second jumper that must be set (JP2) defines the number of wait

states.   I set  the  jumper in  position 3  which  means 2  wait

states.  This was  for a 20 mhz  386 system.  This setting  works

fine for me.  More wait  states may be required or less  may work

22

on your system.   The amount of time lost due to  wait states for

serial I/O is very minimal.

The  last jumper(s) for the Decision-Computer 8 port RS-232 board

is the selection of the IRQ(s) for ports other than  the first 2.

I selected  IRQ3 for the additional  ports that I am  using (I am

only using 4 of the 8).  This means that the first port (COM1) is

using IRQ4, all of the other (3) ports are using  IRQ3.  Position

2 of JP3 (left side) is the jumper that I installed.

Now  for the  seemingly tough  part.   I  ordered the  board from

Decision-Computer in  Taipei, Taiwan.   Why?  Simple,  the price.

The 4 port version of the card is $36.00 US dollars.  The  8 port

version of the  card is $100.00  US dollars.   I have not  really

looked very hard, but I have not seen anything near  this pricing

in the US.

With a few exceptions, I found ordering from Taiwan about as easy

as ordering  from a US company.  The exceptions are, they like to

use a FAX for ordering  etc. and you will have to pay in advance.

I  have ordered several items from Taiwan, paying in advance, and

I  have not  lost  a nickel  yet.   Decision-Computer  will  send

brochures  if  you request  them.   They  can be  reached  at the

following address (FAX is best):

Decision-Computer International Co., LTD.

4/F No. 31-3 Alley 4, Lane 906

Ming-Shen East Road Taipei, Taiwan

R. O. C.

Tel: (02) 766-5753 or 765-9782 or 796-5786

FAX: 886-2-7665702

TELEX: 16059 DECISION

To send a FAX from this area to Decision, I dial:

011-886-2-7665702

The  best  way to  order  is to  have  your bank  wire  the money

directly to Taiwan.   Be sure that you specify Federal Express as

the shipper  from Taiwan to  the US.   Federal Express  will take

care  of any  Customs hassle.   In  most cases  there will  be no

customs fees.   In the worst case that I  have experiences, I had

to pay Federal 2 or 3 dollars in customs fees.

Stargate Plus 8 board from Software Link

Dan  Fox originally helped me with the testing of multiple serial

I/O ports sharing  a single IRQ.  He informed  me that the serial

I/O board that he uses is a  Stargate Plus 8 Board.  He purchased

the board  from Software  Link in Atlanta  Georgia, 404-448-5465.

Dan has all 8 ports using IRQ3.

23

INDEX

16450 . . . . . . . . . . . . . . . . . . . . . . . . . . . .  19

16550 Non A . . . . . . . . . . . . . . . . . . . . . . . . .  19

16550A  . . . . . . . . . . . . . . . . . . . . . . . . . . .  19

16550AF . . . . . . . . . . . . . . . . . . . . . . . . . . .  19

16550AFN  . . . . . . . . . . . . . . . . . . . . . . . . . .  19

16550s Made by Western Digital  . . . . . . . . . . . . . . .  20

16C451  . . . . . . . . . . . . . . . . . . . . . . . . . . .  19

16C551  . . . . . . . . . . . . . . . . . . . . . . . . . . .  20

16C552  . . . . . . . . . . . . . . . . . . . . . . . . . . .  20

8250  . . . . . . . . . . . . . . . . . . . . . . . . . . . .  18

8250A . . . . . . . . . . . . . . . . . . . . . . . . . . . .  18

82510 . . . . . . . . . . . . . . . . . . . . . . . . . . . .  20

Addresses and IRQ for COM3 through COM8 on the PS/2 . . . . .  17

Addresses and IRQs for COM1 and COM2  . . . . . . . . . . . .  17

Appendix A, Hardware Notes  . . . . . . . . . . . . . . . . .  17

Appendix B, SIO Chips . . . . . . . . . . . . . . . . . . . .  18

Appendix C, Multi-Port Serial I/0 Cards . . . . . . . . . . .  21

B option, Locking Baud, Data, Parity and Stop bits  . . . . . . 6

Baud rates  . . . . . . . . . . . . . . . . . . . . . . . . . . 7

BIOS RAM  . . . . . . . . . . . . . . . . . . . . . . . . . . . 9

CAPTURE . . . . . . . . . . . . . . . . . . . . . . . . . . . . 8

COMn and PORTn  . . . . . . . . . . . . . . . . . . . . . . . . 5

CONFIG.SYS  . . . . . . . . . . . . . . . . . . . . . . . .  4, 5

Defacto standard for COM3 and COM4  . . . . . . . . .  11, 17, 21

Defer . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 7

DESQview  . . . . . . . . . . . . . . . . . . . . .  4, 8, 13, 16

DV  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 8

Eliminate . . . . . . . . . . . . . . . . . . . . . . . . . . . 7

FIFO  . . . . . . . . . . . . . . . . . . . . .  8, 9, 15, 19, 20

FOSSIL  . . . . . . . . . . . . . . . . . . . . . . . . . . . . 1

GETTING STARTED . . . . . . . . . . . . . . . . . . . . . . . . 4

IRQ SHARING . . . . . . . . . . . . . . . . . . . . . . . . .  16

Licensing Information . . . . . . . . . . . . . . . . . . . . . 2

Commercial License . . . . . . . . . . . . . . . . . . . . 2

Non-Commercial License . . . . . . . . . . . . . . . . . . 2

Locked  . . . . . . . . . . . . . . . . . . . . . . . . . . . . 6

Locking . . . . . . . . . . . . . . . . . . . . . . . . . . . . 6

Baud . . . . . . . . . . . . . . . . . . . . . . . . . . . 6

Data . . . . . . . . . . . . . . . . . . . . . . . . . . . 6

Parity and Stop bits . . . . . . . . . . . . . . . . . . . 6

Valid locked baud rates  . . . . . . . . . . . . . . . . . 7

Mapping port assignments  . . . . . . . . . . . . . . . . . .  10

Multi-Tasking . . . . . . . . . . . . . . . . . . . . . . . .  13

NOFIFO  . . . . . . . . . . . . . . . . . . . . . . . . . . . . 9

NOPOST  . . . . . . . . . . . . . . . . . . . . . . . . . . . . 9

Number of data bits . . . . . . . . . . . . . . . . . . . . . . 7

Number of stop bits . . . . . . . . . . . . . . . . . . . . . . 7

Parity bit  . . . . . . . . . . . . . . . . . . . . . . . . . . 7

PKUNZIP . . . . . . . . . . . . . . . . . . . . . . . . . . . . 4

Authentic files  . . . . . . . . . . . . . . . . . . . . . 4

24

Port assignments  . . . . . . . . . . . . . . . . . . . . . .  10

Quick Start . . . . . . . . . . . . . . . . . . . . . . . . . . 4

R, Set Receive Buffer Size  . . . . . . . . . . . . . . . . 9, 10

RBBS  . . . . . . . . . . . . . . . . . . . . . . . . . . . .  16

SIO . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 1

SIO Chips . . . . . . . . . . . . . . . . . . . . . . . . . .  18

SLOWCPU . . . . . . . . . . . . . . . . . . . . . . . . . . . . 9

Specifying number of ports  . . . . . . . . . . . . . . . . . . 6

T, Set Transmit Buffer Size . . . . . . . . . . . . . . . . .  10

Terminate and Stay Resident . . . . . . . . . . . . . . . . . . 5

THE BIOS INT 14h EMULATOR . . . . . . . . . . . . . . . . . .  15

TSR . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 5

Uninstalling X00  . . . . . . . . . . . . . . . . . . . . . .  12

VP or Master Interrupt Enable . . . . . . . . . . . . . . . .  10

Warranty  . . . . . . . . . . . . . . . . . . . . . . . . . . . 1

WHAT IS X00 . . . . . . . . . . . . . . . . . . . . . . . . . . 1