XA Command Interpreter

Version 1.0

March  1991

Copyright 1991 by Bruce Christensen

All rights reserved.

Table of Contents

1  Introduction....................................................  3

2  Installation....................................................  4

3  XA - the X10 Command Interpreter................................  4

4  XA Tokens.......................................................  6

4.1  XA Token Reference............................................  7

4.2  Tokens in detail..............................................  8

4.2.1  Action Tokens (ON, OFF, DIM)................................  8

4.2.2  Module Address Tokens (HOUSE, UNIT, A1...P16)...............  9

4.2.3  Coordinate Tokens (LATITUDE, LONGITUDE, TIMEZONE)........... 10

4.2.4  Clock Tokens (SYNCHRONIZE X10, PC).......................... 11

4.2.5  Day Tokens (SUNDAY...SATURDAY, WEEKDAYS, WEEKENDS, etc)..... 12

4.2.6  Time Tokens (TIME, SUNRISE/SET, OFFSET, SECURITY)........... 13

4.2.7  Misc Tokens (BASECODE, PORT, DEFINE, EVENT, CLEAR, DIAGNOSE. 15

5  Command Files................................................... 18

6  POWERUP Utility................................................. 19

7  License Agreement and Registration.............................. 20

XA Command Interpreter                                                PAGE 3

1) Introduction

This floppy contains a set of extremely useful DOS utilties for the

X10 Computer Interface (CP290). These utilties are being marketed under

the try-before-you-buy concept to allow you a complete test of the

product before you are required to register. See Section 7.0 - License

Agreement and Registration for more details.

Utilities provided in this release include:

A) XA - X10 Command Interpreter.

This utility reads and downloads commands to the CP290

in two modes. The first mode is from command line arguments,

ie, from the DOS prompt. The second mode can read an

entire file of commands. These commands can be either

timed events or direct commands.

In addition to these functions, the interpreter has several

features not found with the original X10 software. Some of

these features are:

1) Sunrise/Sunset times based on the latitude,

longitude, and timezone of your city.

2) The ability to change the base housecode without

losing all the data already stored in the interface.

3) String substitution in the command file allows you

to equate a device (FAMILY_ROOM_LAMP) with its module

address (HOUSE A UNIT 3).

4) Comments are allowed in the file to properly document

its content.

2) POWERUP - AUTOEXEC Time Check Utility

This utility is provided for those of you that control your

computer with an X10 Appliance module. When placed in your

AUTOEXEC.BAT file, this utility compares the current PC time

with a pre-set time you define. During your computers powerup,

if the PC's time falls within a certain "window", it will

return an errorlevel of 1. This errorlevel can then be checked

and used to perform any number of functions you desire.

For instance, X10 turns my computer on at 3:00 AM every

Sunday morning. POWERUP checks the computers time and then

invokes a complete backup of my hard drives as well as

downloads updated times to the X10 Computer Interface.

There will be more on this subject in Section 6.

XA Command Interpreter                                                PAGE 4

2) Installation

You will probably want to copy the contents of this floppy to the same

drive and directory in which your X10 software resides. It's also a

good idea to include this drive and directory within your PATH statement

of the AUTOEXEC.BAT file.

Ex: SET PATH=C:\DOS;D:\X10;...

You should set the XA environment variable to the drive and directory

of your command files. This can also be placed in your AUTOEXEC.BAT file.

Ex: SET XA=D:\X10

(See the AUTOEXEC.BAT supplied with this package for an example).

3) XA - The X10 Command Interpreter

As stated earlier, XA can operate on commands from the DOS command

line, or from a file. In either mode, XA uses several options to

alter its operation. The XA command has the following form:

XA [f=filename] [c=comm port] ["commands"] [-e] [-s]

The options may appear in any order, but the presence of some of

these options negates other options.

[f=filename]   - this is the name of the file containing

the commands to be interpreted by XA. To use XA

in its file mode, you would type:

XA f=filename

The default file name is XA.CMD. If you want to use

the default file, you do not have to specify the

filename - just type:

XA

[c=comm port]  - this allows you to select the serial port

for communications with the interface. You may

select ports 1...4. For example, if your CP290

is connected to COM3:

XA c=3

The default port is 1. To use the default, you do

not have to specify the port number - just type:

XA

See Section 4.2.7 - Miscellaneous tokens for a

different way to set the comm port within a

command file.

["commands"]   - "commands" consist of several tokens (huh? - see

Section 4 - Tokens) that describe an event of

direct command. For example:

XA "A1 ON"

XA Command Interpreter                                                PAGE 5

In the above example, A1 and ON are tokens.

Together they form a single command. Multiple

commands may appear in the command line, but they

must be enclosed by quotation marks (" "). The

next example shows the use of multiple commands

to create a flashing effect:

XA "A1 ON" "A1 OFF" "A1 ON" "A1 OFF"

One other note, the presence of any commands on

the command line will nullify the use of a file.

Therefore in the next example, the XA utility will

ignore the file request and process just the

commands:

XA f=null.cmd "A1 A2 A3 A4 ON"

[-e]           - this option is used to prevent any communications

to the interface. Use this option only when

you want to validate commands in a file.

[-s]           - this option is used to prevent sorting the commands

based on their EVENT numbers (see EVENTS described

in Section 4.2.7 - Miscellaneous tokens).

[+h]           - displays a brief summary of the options described

above.

XA Command Interpreter                                                PAGE 6

4) XA Tokens

What are tokens?

A token is to a command as a word is to a sentence. Individual words are

constructed to form a complete sentence - tokens are constructed to form

complete commands. This section will describe each token that is

recognized by the interpreter. In most cases, tokens may appear anywhere

in the command. For example, the command

XA "A1 ON"    may also be constructed as

XA "ON A1"

In other instances, tokens are must sometimes be arranged in groups,

and their order is important. For example, the command to set your PC's

internal clock to the time that is maintained by the CP290 is:

XA "SYNCHRONIZE PC"      <---This is legal

If the tokens SYNCHRONIZE and PC were reordered as in the following

command:

XA "PC SYNCHRONIZE"      <---This is illegal

The interpreter would respond with the following message:

WARNING: Unrecognized token "PC" in line 1.

Therefore, throughout this documentation any tokens that must be

arranged in a certain order will be contained in curly braces {..}.

XA Command Interpreter                                                PAGE 7

4.1) Token Reference

The following is list of available tokens. They are discussed in more

detail in the next section:

4.1.1 Action tokens -

ON             - turns ON the specified modules.

OFF            - turns OFF the specified modules.

{DIM %%}       - DIM to a % (percentage) level of brightness.

4.1.2 Module address tokens

{HOUSE x}      - module HOUSEcode A...P

{UNIT z}       - module UNITcode 1...16, ALL

A1...P16       - HOUSE UNIT combined

4.1.3 Coordinate tokens:

{LATITUDE d m} - your LATITUDE in Degrees and Minutes

{LONGITUDE d m}- your LONGITUDE in Degrees and Minutes

{TIMEZONE x}   - your TIMEZONE (Standard/Daylight Savings time)

4.1.4 Clock tokens:

{SYNCHRONIZE PC}  - set PC time based on X10 time

{SYNCHRONIZE X10} - set X10 time based on PC time

4.1.5 Day tokens:

SUNDAY,    SUN - Specific day for event to occur

MONDAY,    MON -     "

TUESDAY,   TUE -     "

WEDNESDAY, WED -     "

THURSDAY,  THU -     "

FRIDAY,    FRI -     "

SATURDAY,  SAT -     "

WEEKDAYS       - Event occurs only on weekdays

WEEKENDS       - Event occurs only on weekends

EVERYDAY       - Event occurs everyday

TODAY          - Event occurs once today only

TOMORROW       - Event occurs once tomorrow only

4.1.6 Time tokens:

{TIME hh:mm AM} - Schedule event time

{TIME hh:mm PM}

SUNRISE         - Calculate time of sunrise

SUNSET          - Calculate time of sunset

{OFFSET +/-mm}  - Event + or - 60 mins of programmed time (dawn)

NORMAL          - Event occurs at specified time

SECURITY        - Event occurs within the hour of programmed time

4.1.7 Miscellaneous tokens:

{BASECODE x}   - change house BASECODE of interface to A...P

{PORT COMx}    - send commands via PORT COM1..4

{DEFINE x z}   - substitute string 'x' for string 'z'

{EVENT xxx}    - Place event xxx in a known location of CP290 memory

{CLEAR xxx}    - Delete event xxx from CP290 memory

DIAGNOSE       - Invoke CP290 self-test routine

XA Command Interpreter                                                PAGE 8

4.2) Tokens in Detail

4.2.1 Action Tokens

ON

This token informs the interface to turn ON the selected

modules.

Example: XA "A1 ON"

OFF

This token informs the interface to turn OFF the selected

modules.

Example: XA "A1 OFF"

{DIM %%}

These two tokens specify the intensity level of the selected

lamp module or wall switch module. The modules will turn on,

brighten to full intensity, then dim to the percentage

specified in the second token. There are 16 different

brightness levels available, the percentage you choose is

mapped to one of these levels.

Example: XA "A1 DIM 50"

Notes:

1) Only one token in this category should appear in a command. If

two or more of these tokens appear, only the last one will be

acted on. For example:

XA "A1 ON OFF" - will be interpreted as:

XA "A1 OFF"

If you were trying to flash the lights, the proper way to perform

this action is:

XA "A1 ON" "A1 OFF"

2) You may activate as many modules as you want in a single command

providing these modules share the same HOUSE code:

XA "A1 A2 A3 A4 A5 A6 ON"

To activate modules with different HOUSE codes, use separate

commands:

XA "A1 A2 A3 ON" "B1 B2 ON" "C1 OFF"

3) If you download a timed event that contains the DIM token, the

X10 interface program (X10.EXE) will report the action taken on

the module as DIM 90%.

XA Command Interpreter                                                PAGE 9

4.2.2 Module Address tokens

{HOUSE x}

These two tokens specify the HOUSE code of the module you

want to select. There are 16 HOUSE codes available, lettered

A through P. This method of selecting address requires the

use of the UNIT token. Only one HOUSE token should appear in

a command.

{UNIT y}

{UNIT ALL}

These two tokens specify the UNIT code of the module you want

to select. There are 16 UNIT codes available, numbered 1

through 16. Specifying UNIT ALL operates on all 16 modules

addressed by the HOUSE token.This method of module addressing

requires the use of the HOUSE token.

Examples: XA "HOUSE A UNIT 1 ON"   or,

XA "UNIT 1 HOUSE A ON"   or,

Multiple UNITs may appear in a single command.

Example:  XA "HOUSE A UNIT 1 UNIT 2 UNIT 3 ON"

Turn off all units on HOUSE P.

Example: XA "HOUSE P UNIT ALL OFF"

A1...P16

This is a shortcut method of specifying both HOUSE and UNIT

tokens. The house code must appear first, followed by the

unit code. Specifying ALL is not permitted.

Example:  XA "A1 ON" "P16 OFF"

Notes:

1) You may activate as many modules as you want in a single command

providing these modules share the same HOUSE code:

XA "A1 A2 A3 A4 A5 A6 ON"

To activate modules with different HOUSE codes, use separate

commands:

XA "A1 A2 A3 ON" "B1 B2 ON" "C1 OFF"

XA Command Interpreter                                                PAGE 10

4.2.3 Coordinate tokens

This group of tokens enable the precise calculation of sunrise and

sunsets times for your location.  If your events don't require

these functions, then don't worry about them. This information can

be found in almanacs, or extrapolated from maps. Your local

library or city hall may also have the precise coordinates.

{LATITUDE d m}

Your LATITUDE in degrees (0...+/-90) and minutes (0...59).

Example: LATITUDE 41d35m

{LONGITUDE d m }

Your longitude in degrees (0...+-180) and minutes (0...59).

Longitudes West of the prime meridian are positive, those East

of the prime meridian are negative. In the US, all longitudes

are positive.

Example: LONGITUDE 81d20m

{TIMEZONE x}

Exact calculations depend on the local time. Use the

following table to determine your time zone.

Zone                     x

====================     =========

Eastern Standard Time:        5

Eastern Daylight Time:        4

Central Standard Time:        6

Central Daylight Time:        5

Mountain Standard Time:       7

Mountain Daylight Time:       6

Pacific Standard Time:        8

Pacific Daylight Time:        7

Example: TIMEZONE 5

Notes:

1) Since all three tokens in this group are required, it is easier

to include these tokens in a file and invoke the XA utility in

the file mode:

Example: XA f=rise_set.cmd

where the file "rise_set.cmd" contains the following

commands:

:

LATITUDE 41d35m

LONGITUDE 81d20m

TIMEZONE 5

:

XA Command Interpreter                                                PAGE 11

4.2.4 Clock tokens

{SYNCHRONIZE PC}

These tokens instruct the XA utility to set your computers

clock based on the current time retrieved from the CP290. It

will not affect the date.

Example: XA "SYNCRONIZE PC"

{SYNCHRONIZE X10}

These tokens instruct the XA utility to set the time and day

of the CP290 internal clock based on the current time

retrieved from your computer.

Example: XA "SYNCHRONIZE X10"

Notes:

1) You might want to include the command:

XA "SYNCHRONIZE PC"

in your AUTOEXEC.BAT file to set your PC's internal clock if

your system lacks a on-board battery backed-up clock. Note that

this command will not set up the date since X10 does not

contain a real-time calendar.

2) Use:

XA "SYNCHRONIZE PC"   or,

XA "SYNCHRONIZE X10"

in your AUTOEXEC.BAT file if either your PC or CP290 do not

keep accurate time. My CP290 tends to keep better track of time

than my PC does.

XA Command Interpreter                                                PAGE 12

4.2.5 Day tokens:

Use of these tokens indicate a timed event, ie, the complete command

will be downloaded to the interface and stored there for later

activation.

SUNDAY,    SUN

MONDAY,    MON

TUESDAY,   TUE

WEDNESDAY, WED

THURSDAY,  THU

FRIDAY,    FRI

SATURDAY,  SAT

These tokens specify the specific day of the event. Multiple

tokens may appear within a command.

Example: XA "MONDAY FRIDAY A1 ON TIME 7:30 AM NORMAL"

WEEKENDS

Event occurs on SATURDAY and SUNDAY

WEEKDAYS

Event occurs MONDAY TUESDAY WEDNESDAY THURSDAY FRIDAY

EVERYDAY

Event occurs MON TUE WED THU FRI SAT SUN

TODAY

Event will occur today only at the specified time. It is then

deleted from the interface after midnight.

TOMORROW

Event will occur tomorrow only at the specified time. It is

then deleted from the interface after midnight.

Notes:

XA Command Interpreter                                                PAGE 13

4.2.6 Time tokens:

Use of these tokens indicate a timed event, ie, the complete command

will be downloaded to the interface and stored there for later

activation.

{TIME hh:mm AM}

{TIME hh:mm PM}

Schedule event at a certain time.

Example: XA "SUNDAY ON TIME 12:30 AM D1 NORMAL"

SUNRISE

SUNSET

These tokens will calculate the sunrise or sunset for the day

token in the command. The use of these tokens also requires the

LATITUDE, LONGITUDE, and TIMEZONE tokens to be included.

Therefore, these tokens should be used in the file mode.

Example: MONDAY ON SUNSET D1 NORMAL

If multiple day tokens appear in the command line, the

sunrise or sunset will be calculated for the last day token

that appears in the command line.

Example: MONDAY TUESDAY WEDNESDAY ON SUNSET D1 NORMAL

If more accuracy is needed, then break each daily event into

its own command.

Example: MONDAY ON SUNSET D1 NORMAL EVENT 0

TUESDAY ON SUNSET D1 NORMAL EVENT 1

WEDNESDAY ON SUNSET D1 NORMAL EVENT 2

{OFFSET +-mm}

Push or postpone the scheduled event by up to 60 minutes. This

token is useful if you want to activate lights at dusk rather

than sunset, or to deactivate lights at dawn rather than

sunrise. The '+' sign is the default condition, you must

specify the '-' if needed.

Example: Turn on lights 30 minutes after sunset

MONDAY ON SUNSET D1 NORMAL EVENT 0 OFFSET 30

Turn lights off 45 minutes before sunrise

MONDAY OFF SUNRISE D1 NORMAL EVENT 1 OFFSET -45

XA Command Interpreter                                                PAGE 14

NORMAL

This token activates the event at the specfied time.

It is the default mode, therefore it does not need to be

specified.

SECURITY

This token allows the interface to randomly select the

activation time of the event. This activation occurs within the

hour of the programmed time. Therefore, if the downloaded

time was for 7:18 PM, actual activation time could range from

7:00 PM through 7:59 PM. This mode provides a security

feature giving your lighting a more lived-in look.

Example: XA "A1 ON TIME 7:30 PM SECURITY EVERYDAY"

XA Command Interpreter                                                PAGE 15

4.2.7 Miscellaneous tokens:

{BASECODE x}

These tokens change the base housecode of the interface

WITHOUT losing the data stored in the interface. Prior to

changing the housecode, a complete upload of all events

stored in the interface is performed. The housecode is

changed, then all events are downloaded back into the

interface. Valid housecodes are A...P.

Example: XA "BASECODE D"

During upload, the interface sends all 128 possible events that

are stored in its memory. If an event actually exists, then an

up arrow is displayed, otherwise a dot (�) is displayed. During

download, a down arrow is displayed. Downloads will take

significantly longer due to the overhead involved with sending

synch bytes for each event.

{PORT COMx}

These tokens allow you to specify which comm port the

interface is attached to. Legal ports are:

COM1 = I/O address 0x3F8   Interrupt 4

COM2 =      "      0x2F8       "     3

COM3 =      "      0x3E8       "     4

COM4 =      "      0x2E8       "     3

These tokens were intended to be placed in command files.

They will take precedence over the 'c=x' command line

argument (see section 3).

Example line in a command file:

PORT COM3

XA Command Interpreter                                                PAGE 16

{DEFINE x z}

These tokens were designed to be used in command files. They

allow you to substitute several tokens into a single token you

create. They help to make command files more readable. Up to 50

substitutions are allowed. For example, you can assign a string

CHRISTMAS_ON to an equivalent group of tokens HOUSE A UNIT 4 ON

as follows:

DEFINE CHRISTMAS_ON HOUSE A UNIT 4 ON TIME 8:00 PM EVERYDAY

============ =======================================

x                          z

The 'x' and 'y' strings can be up to 60 characters each.

Notice that the 'x' portion uses underscore characters (_) to

separate the words but to keep the string contiguous. This is

important because the XA interpreter looks for a SPACE

character to separate the 'x' and 'y' components.

Later in the command file, you can use the following command

to activate your Christmas lights:

CHRISTMAS_ON

Internally, XA will convert this string to its equivalent:

HOUSE A UNIT 4 ON TIME 8:00 PM EVERYDAY

More Examples:

:

DEFINE PLANT_GROW_LIGHTS A8

DEFINE OUTSIDE_LIGHTS A1

PLANT_GROW_LIGHTS ON TIME 6:00 AM EVERYDAY

PLANT_GROW_LIGHTS OFF TIME 12:00 AM EVERYDAY

OUTSIDE_LIGHTS OFF SUNRISE EVERYDAY

**** Important Note ****

DEFINEs must be declared before being used. The following

would cause an error message because the DEFINE was placed

out of order:

OUTSIDE_LIGHTS ON SUNSET EVERYDAY

DEFINE OUTSIDE_LIGHTS A1

XA Command Interpreter                                                PAGE 17

{EVENT xxx}

The CP290 can store up to 128 events in its memory. Each event

occupies fixed locations in memory. When you download events

to the interface, an event number will be assigned to the

command. The first event will have an event number of 0, the

second will be 1, etc. Therefore, any event that was

previously stored at those locations will be overwritten. The

token {EVENT xxx} allows you to override the number assigned to

this command.

If you normally use one command file then the need to use

this token is probably unnecessary. If multiple command

files are used, then this token is helpful.

Example: MON ON SUNSET D1 NORMAL EVENT 100

TUE ON SUNSET D1 NORMAL EVENT 101

WED ON SUNSET D1 NORMAL EVENT 102

{CLEAR EVENT xxx}

These tokens delete the specified event from the computer

interface. See description for EVENT xxx above. Only event

may be cleared per command.

Example: XA "CLEAR EVENT 100" "CLEAR EVENT 101"

DIAGNOSE

This token triggers the X10 diagnostic routine. You will be

informed of the results after a few seconds.

XA Command Interpreter                                                PAGE 18

5)  Command files

Since the CP290 is only capable of storing a single weeks worth of

events, command files were created allow easy update of the events that

may need to revised due to ever-changing conditions, such as sunrise

and sunset times.

Command files should only be created with an editor or word processor

that saves files in pure ascii format. Any extraneous characters or

control codes will cause the XA interpreter to ignore your commands.

You may place comments in your command file. A comment is delineated by

the pound sign character '#'. This character must appear in the first

column of the line.

# This is an example comment.

Comments are ignored by the interpreter. Therefore, comments can be

placed in front of commands you may not wish to download to the

interface at this time. For example, you can hide your Christmas

lighting commands from the interpreter after the holidays:

#DEFINE XMAS HOUSE A UNIT 2

#XMAS ON EVERYDAY SUNSET OFFSET 20

#XMAS OFF WEEKDAYS TIME 10:00 PM

#XMAS OFF WEEKENDS TIME 12:00 AM

Blank lines are permitted in the command file.  These allow you to

group similar events together.

See the sample command file "XA.CMD" (included in this package) I use for

automating events in my home.

XA Command Interpreter                                                PAGE 19

6)  POWERUP Utility

The POWERUP utility is provided in this package to help you perform

unattended, and automatic event downloads to the interface. POWERUP

should be placed in your AUTOEXEC.BAT file so it will always be executed

after your computer boots. This way, it can decide whether or not it's

time to perform any updates to your interface, and prevents you from

needlessly sending commands when you don't need to.

The POWERUP command has the following form:

POWERUP d=day s=hh:mm e=hh:mm

The command line parameters may appear in order, and they must all be

present:

d=day     - This is the text string for day you want to check

for. Legal strings are: MONDAY, TUESDAY,

WEDNESDAY, THURSDAY, FRIDAY, SATURDAY and SUNDAY.

s=hh:mm   - This is the "window" start time. Hours must be

entered in 24 hour format only.

e=hh:mm   - This is the "window" end time. Again, 24 hour

format is required.

Example: POWERUP d=SUNDAY s=2:55 e=3:05

If the "day" specified matches the day reported by DOS, and the time

reported by DOS falls within the specified "window", then POWERUP will

return an ERRORLEVEL of 1, otherwise a 0 is returned. Therefore, by

placing the proper batch file commands in AUTOEXEC.BAT, you can now

execute a variety of functions at certain times.

If your computer is controlled by an Appliance module, you can now

automatically download revised event times. In addition, you can perform

other actvities such as backups, communications, etc. without even being

at your computer. I perform my backups and event revisons on Sunday

mornings at 3:00 AM. This way, I never forget about backing up and now I

don't have to sit around and wait for them to finish. See the sample

AUTOEXEC.BAT file included in this package that I use to accomplish this

task.

XA Command Interpreter                                                PAGE 20

7)  License Agreement and Registration

You may make copies of this program, manual, and other files and give it

to your friends, upload it to bulletin boards, or include it in the

library of a non-profit computer club.

I expressly forbid any for-profit venture from selling this software and

documentation, either separately or as part of a "library" diskette.

I disclaim any liability for its use, misuse, or abuse, including any

direct or indirect actions. The user accepts full responsibility for his

or her use of this program.

SUPPORT SHAREWARE!

If you find this software has any value for you, please send a

contribution. Contributions of $20 or more will entitle you to (2) updates

of this program as new enhnacements are added. Also, a completely new

human interface is being designed. Your contribution will enable me to

notify you when it's complete.

Whether or not you contribute, please share this software with others.

Your suggestions are always welcome. Send any comments to the address

given below, or you may contact me on Prodigy.

Bruce Christensen

6594 Hudson Ave.

Mentor, OH. 44060-4545

Prodigy ID:  MHNC39A

X-10 is a registered trademark of X-10 (USA) Inc.

Prodigy is a registered service mark and trademark of Prodigy Services

Company.