The KayPro II (and the other models) emulate the Lear-Seigler ADM-3A

terminal. In order to install some screen oriented software it is necessary

to know the terminal attributes. The following may help.

CURSOR CONTROL

cursor left  -    8 dec    08H

cursor right -    12 dec   0CH

cursor down  -    10 dec   0AH

cursor up    -    11 dec   0BH

home cursor  -    30 dec   1EH

clear scr & home  26 dec   1AH

carriage return   13 dec   0DH

CURSOR POSITIONING

rows              = 0 - 23 dec

columns           = 0 - 79 dec

escape sequence  -  27, 61 dec     1BH, 3DH

constant offset   = 20 dec    14H

So in Mbasic to position the cursor at row 12, col 40

print chr$(27)chr$(61)chr$(20+12)chr$(20+40)

ESC      =        row        col

LINE INSERT / DELETE

insert line    - 27, 69 dec      1BH, 45H { scroll down }

delete line    - 27, 82 dec      1BH, 52H { scroll up   }

CLEAR TO END OF LINE / SCREEN

clear to EOL - 24 dec   18H

clear to EOS - 23 dec   17H

CHANGE CHARACTER SETS

set ASCII - 27,65 dec   1BH,41H

set greek - 27,71 dec   1BH,47H

In order to use the RS-232 port for communications it must be initialized

to the proper data word length, parity, and number of stop bits as well as

the correct baud rate. Below is an I/O port map and an RS-232 initialization

procedure in Sbasic.

I/O PORTS

RS-232 status -   6 dec

RS-232 data   -   4 dec

keyboard status - 7 dec

keyboard data -   5 dec

parallel status - 9 dec

parallel data -   8 dec

system status -   29 dec   1DH

system data -     28 dec   1CH

RS-232 INITIALIZATION

procedure initmod(word1,word2,word3 = integer)

var sio,resetchan,nointerrupt,reg1,reg3,reg4,reg5 = integer

sio = 6           rem { RS-232 status port }

resetchan = 24    rem { sio channel reset  }

nointerrupt = 0   rem { turn off interupts }

reg1 = 1          rem { next byte goes to register #1 }

reg3 = 3          rem { next byte goes to register #3 }

reg4 = 4          rem { next byte goes to register #4 }

reg5 = 5          rem { next byte goes to register #5 }

begin

rem { change 2 byte integer to 1 byte integer with chr function }

out(sio),chr(resetchan)

out(sio),chr(reg4) : out(sio),chr(word1)

out(sio),chr(reg1) : out(sio),chr(nointerrupt)

out(sio),chr(reg3) : out(sio),chr(word2)

out(sio),chr(reg5) : out(sio),chr(word3)

end

The values for 8 data bits, 1 stop bit, No Parity are:

word1 = 68 dec

word2 = 193 dec

word3 = 106 dec

These are values to correctly tell the Z-80 sio chip what to do, each bit is

significant. For other values see a Zilog data book.

To set the baud rate you just send an integer between 0 and 15 dec to port 0

For 300 baud send 5, for 1200 baud send 7

out(0),5  ' 300 baud

OR

out(0),7  ' 1200 baud

The status port for the RS-232 has 3 registers, only 2 are used for

asynchronus communication, 0 and 1. Nothing special needs to be done to

read register #0,

i.e N=inp(6)

works just fine, but in order to get to read register #1 you must write

to the status port first,

i.e out(6),1

to get register #1 then:

N=inp(6)

to get the value in register #1.

The bits in read register #0 are:

0 = Rx char ready

1 = Int pending

2 = Tx buffer empty

3 = DCD (data carrier detect)

4 = Sync/Hunt (synchronous only)

5 = CTS (Clear to send)

6 = Tx underrun/Eom (?)

7 = Break/Abort

The bits in read register #1 are:

0 = All sent

1 =

2 = Field bits in previous & 2nd previous bytes (?)

3 =

4 = Parity error

5 = Rx overrun error

6 = CRC/framing error

7 = End of frame

KAYPRO RS-232 PINOUT

Only pins 1 - 8 and 20 seem to be used and they go like this:

1  frame ground

2  Tx transmit data

3  Rx recieve data

4  RTS request to send

5  CTS clear to send

6  DSR data set ready

7  signal ground

8  DCD data carrier detect

20 DTR data terminal ready

Well there you have it the hardeware-software interface for two of the most

used communications peripherals on the KayPro the screen and the RS-232 port.

############################################################################################################